3GPP TSG-RAN WG2 Meeting #130 	R2-2504135

St. Julians, Malta, 19th – 23rd of May 2025

Agenda item:	8.6.3

Source:	Nokia

Title:	Final Details on L1 Measurement Reporting Enhancements for Rel-19 LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The phase 4 mobility enhancements work item description captures the measurement enhancements in the [1] as follows:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Following agreements have been made in the previous RAN2 meetings [2-4]:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

Agreements made at the most recent RAN2 meeting (RAN2#128 [5]):

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

And the following decisions have been made at RAN2#129 (February 2025) [6]:

Agreements on L1 event-triggered MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

And the following decisions have been made at RAN2#129bis [9]:

1. Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource. 2. The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id. 3. In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to). 4. In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam. 5. RAN2 will consider only the following options: - Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} - Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting. Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE. Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE. Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before). Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info) 6. Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} 7. Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR. 8. For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR. • For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. • When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE. • When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed. • The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE. • FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates. • Select two values of 20ms and 60ms for the additional values (report periodicity)

In this paper we present our views on the remaining open issues for event-triggered L1 measurement reporting framework and for activation/deactivation of semi-persistent candidate cells’ CSI-RSs for LTM.

# Event-triggered reporting for LTM

## On the indication of beam types in MR MAC CE

At RAN2#129bis it was debated whether Option 1 or Option 3 should be adopted for representing different types of beams to be reported in MAC CE. During the come back session the following working assumption has been made:

Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

According to option 1, type1 and type3 represent the beam that meets the reporting entry criteria. The only difference is that type 1 denotes a beam that fulfils the condition for the first time whereas type 3 is used to label a beam which has been already reported but remains in the “beamsTriggeredList”. In a nutshell, this beam was reported as type 1 beforehand.

Observation 1: The only difference between type1 and type3 beams is that type1 denotes a beam that fulfils the condition for the first time whereas type3 is used to label a beam which has been already reported but remains in the “beamsTriggeredList” (such beam was reported as type 1 beforehand).

However, there is no genuine value of having such differentiation as both beams could be equally good (i.e. if type1 and type3 beam fulfil the entry criteria for reporting then any of these could be a suitable candidate for becoming the target beam).

Observation 2: If type1 and type3 beams fulfil the entry criteria for reporting then any of these could be a suitable candidate for becoming the target beam (i.e. the differentiation does not make sense).

Furthermore, Option 1 brings additional complexity to the User Equipment (UE), as the UE needs to keep track of the beams previously reported. In Option 3, where type1 and type3 are not differentiated, the UE would just check if certain beam has fulfilled the entry criteria when evaluating beams for the current reporting procedure (i.e. it does not need to keep track and check the past status of the beams).

Observation 3: Option 1 brings additional complexity to the UE, as the UE needs to keep track of the beams previously reported and maintain their status.

Considering all aspects listed above, we propose the following:

Proposal 1: RAN2 is asked to adopt Option 3 and therefore reject the working assumption taken at RAN2#129bis.

## 2.2. On the need for L2 filtering

For event triggered reporting, as agreed in RAN2, there would be additional TTT based time-domain filtering of measurements at Layer 2 before reporting, where the UE would be configured to observe whether the evaluated criteria are fulfilled for the duration of the TTT (e.g., LTM3). Additionally, some hysteresis and/or smoothing filter would be needed to avoid dropping the event triggering based on one slightly lower quality sample

To analyze the impact of additional filtering, which is applied to each L1 measurement before it’s used as input for TTT/hysteresis-based time-domain filtering, we conducted system-level simulations using the LTM3 event. We evaluated the impact of the filter coefficient in the IIR filter for different TTT values on the following metrics:

- Number of LTM handover triggers

- Number of reports sent to the network

- Number of ping-pongs

- Number of handover failures and radio link failures

Figure 1. Number of LTM Handovers and number of measurement reports sent.

Figure 2. Number of ping-pongs and number of HOFs and RLFs.

We observed that by introducing filtering of raw L1 measurements, handover frequency decreases, which effectively reduces the number of ping-pongs. However, we also noted a counterintuitive increase in the number of reports: as handover execution is delayed, more beam reports are transmitted during that period. Interestingly, the failure rate KPI suggests that filtering may negatively impact mobility robustness, as the number of failures (both handover and radio link failures) increases with additional filtering which may be result of delaying handovers.

Based on the simulation results, finding the optimal parameters for filter coefficients and TTT may not be straightforward. Nevertheless, it is beneficial to leave the decision regarding additional filtering for event-triggered LTM to the network, as it has been defined for L3 mobility. The network could configure this filtering just in specific scenarios if desired or opt not to configure it at all

Observation 4: L2 filtering for LTM could be optionally configured by the NW and applicable just to certain scenarios.

Furthermore it should be noted, that the TTT and higher layer filtering (i.e. L2 filtering of L1 measurements) together are important parameters for tuning mobility KPIs. The reason why L3 mobility works is that both of these coexist. It is not either or type of approach.

Observation 5: TTT and L2 filtering together are important parameters for tuning mobility KPIs.

Thus, if RAN2 intention is to have a working and practical functionality, we suggest the following:

Proposal 2: RAN2 reconsiders previous decision and specifies L2 filtering of L1 measurements for event triggered reporting.  The L2 filtering at MAC is applied for candidate RS in similar manner as L3 Beam filtering (see TS 38.300, section 9.2.4).

## 2.3. Truncation of MR MAC CE

At RAN2#129 the following has been agreed:

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

It was also discussed via offline thread when the UE can use the truncated MR MAC CE. The following was concluded [6]:

13. Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader

In our opinion this should be clarified further – the UE may use truncated format only when the normal MAC CE is not able to accommodate the RSRIs that fulfil the criteria (i.e. type1 and type3 if Option 1 in section 2.1 is adopted) for the measurement report event (i.e. not just when the total number of N beams cannot be accommodated in the normal MAC CE).

Proposal 3: The UE uses truncated MR MAC CE when it is not able to report all the beams that triggered the report or beams that have earlier triggered the report (i.e. type1 or type3 if Option 1 is adopted), up to the configured number of beams.

Proposal 4: The truncated MR MAC CE can only include RSRIs of beams that fulfil or have fulfilled the triggering conditions earlier.

Proposal 5: If the triggered beam (the beam or beams that triggered the report) is the type1 the UE includes first the beam or beams that triggered the report:

After that the UE includes the beams that are reported for leaving condition (type2), if these exist

After that the UE includes the beams that are reported as type3 (i.e. still meet the triggering condition but were reported already beforehand).

In case Option 3 (section 2.1) is adopted (i.e. working assumption is rejected) then type1 and type3 are not differentiated and are reported first in such MR.

Proposal 6: If Option 3 is adopted then then type1 and type3 are not differentiated and are reported first in MR MAC CE.

## 2.4 Prohibit Timer for triggering the MAC CE Report

To avoid too frequent measurement reporting by the UE, a prohibit timer could be applied for the event triggered reporting. The prohibit timer could be applied per reporting configuration i.e. the UE would not be allowed to trigger another MR MAC CE for the same reporting configuration while the associated prohibit timer is running.

Observation 6: A prohibit timer could be applied for the LTM event triggered reporting to avoid too frequent measurement reporting.

Proposal 7: RAN2 to support a prohibit timer for event triggered reporting. The prohibit timer is applied per reporting configuration.

## 2.5 Number of reported beams in MR MAC CE

The following has been agreed so far regarding the number of beams and what kind of beams could be reported:

7. For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

The definition of Rel-18 LTM report content in TS 38.331 is as follows:

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

The maximum number of reported beams in R18 LTM was specified to be 16. We see no strong reason to deviate from this and therefore the number of reported beams can be up to N=16. The value is network configurable. The same has been also confirmed by RAN1 in their response LS [10]. In [10] it is indicated the maximum of 16 beams can be reported and that is independent of whether the serving beam is included or not. In addition, RAN1 may introduce the capability to indicate the maximum number of beams that the UE supports.

Observation 7: There is no reason to deviate from the R18 reporting parameters, where reporting of up to 16 beams can configured.

Observation 8: RAN1 confirms the maximum of 16 beams can be reported and that amount is independent of whether the serving beam is included.

Observation 9: RAN1 may introduce the capability to indicate the maximum number of beams that the UE supports.

Furthermore, we don’t see strong need to have limitation of reported beams per cell in the MR MAC CE.

Proposal 8: For reporting beams in MR MAC CE, up to 16 beams can be included. Potential candidate values for the number of beams are: {1,2,3,4,8,12,16}.

In some case, the number of reported beams not fulfilling the criteria should be limited to prevent the UE from including information that may not be useful for the network. For example, when the maximum number of beams which can be reported is relatively large, but only a few beams fulfil the reporting criteria, it is not beneficial to always include up to N beams in the MR MAC CE. This could unnecessarily consume UL resources that could otherwise be available for reporting other information or sending UL user data.

Also, when truncating the MR MAC CE, any beam not fulfilling the reporting criteria should be omitted, like we have suggested already above.

Observation 10: The number of reported candidate beams that do not fulfil the reporting criteria should be limited.

Proposal 9: A maximum limit should be set for the number of candidate beams in the MR MAC CE that do not fulfil the reporting criteria.

## 2.6 Applying Beam Specific offset

RAN2#129bis agreement states the following:

3.	In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

Based on the above agreement the offset value can be only configured when the candidate ID is provided in the reporting configuration. However, the candidate ID inclusion also means that the UE would limit the measurements to the candidate IDs which are included in the reporting configuration. The two features of: offset value configuration and limiting the UE measurements per candidate cell should not be mixed and should remain independent.

Observation 11: The offset value configuration and limiting UE measurements per candidate cell are different features and should not be mixed.

Any configuration to limit measurements for certain candidate cells or RS of certain candidate cells, at least in R19 should be done via CSI resource configuration. If the NW does not want the UE to perform measurements on one specific set of RSs on certain cell, or exclude certain cell from the measurement set, it can use the RRC signaling to update the CSI resource configuration.

Observation 12: Any configuration to limit measurements for certain candidate cells or RS of certain candidate cells, at least in R19 should be done via CSI resource configuration.

In case the network wants to limit the measurements it could  be done in more dynamic manner i.e. via MAC CE but this can be studied as potential enhancement in later releases.

Observation 13: Offset value can be configured for a candidate cell as part of the reporting configuration. The candidate ID.

If the offset value is not configured for the certain candidate cell it should mean that offset value is not applied for the beams of the candidate cell when evaluating the reporting criteria.

Observation 14: If the offset value is not configured for the certain candidate cell or the candidate ID is not present it should mean that offset value is not applied for the beams of the candidate cell when evaluating the reporting criteria.

Based on above, we propose the following:

Proposal 10: RAN2 to slightly revise the existing agreement to: offset value can be configured in the reporting configuration per candidate cell. If offset value is configured, the offset value is used for event evaluation for the RS included in the CSI resource configuration for that candidate cell. If the offset value is not configured for a candidate cell, the UE does not apply offset value for the candidate cell.

## 2.7 Support for mTRP Operation in the Serving Cell

Based on the RAN1 conclusion that the support of coexistence of LTM event-triggered reporting and mTRP operation in the serving cell is up to RAN2, the following agreement was made in RAN2 #129bis:

RAN2# 129bis

For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until/in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

Note that for Rel-18/19 LTM and Rel-19 CLTM, cell switch to a target PCell with mTRP deployment (i.e., two TA) has already been supported by RAN2. Therefore, it would be illogical to restrict the scenario where the UE is in mTRP operation in the serving cell from supporting Rel-19 event-triggered reporting.

Observation 15: For Rel-18/19 LTM and Rel-19 CLTM, RAN2 has supported the cell switch to a target PCell with mTRP deployment (i.e., 2 TA).

Proposal 11: Coexistence of LTM event-triggered reporting and mTRP operation at the serving cell is supported in Rel-19.

Supporting mTRP operation at serving cell implies that the UE may have two indicated TCI states. This means the UE can derive two RSs for current serving beam measurements in the serving cell. This would result in the following two issues:

- Event evaluation reference: which of the two indicated TCI states should be used for event evaluation?

- Reporting format: which of the two indicated TCI states is used to include the serving cell information in the MR MAC CE.

Regarding 1), we propose that UE implementation should decide which TCI state to use, or alternatively, simply use the beam with better measurement quality (i.e., the one with higher RSRP). The latter option is to allow the cell switch operation only when there is a candidate beam better than the best serving beam.

Proposal 12: If the UE is configured with mTRP operation, one of the following options should be selected for event evaluation:

- Option 1: UE may select which of the two TCI states it uses (i.e. left to UE implementation).

- Option 2: UE selects the TCI state (beam) with higher RSRP for event evaluation

Regardless of which TCI state was used for event evaluation, it would be beneficial for the UE to report RSRP measurements of both beams as serving beam information in the MR MAC CE. However, this may require a modified MAC CE format to include two octets for RSRP instead of one (used when mTRP is not configured).

As a simpler alternative, only one serving RSRP can be reported, along with an indication of whether it corresponds to the first or second indicated TCI state. This bit can be included in the same octet as the absolute RSRP value (which consumes 7 bits).

Proposal 13: If the network configures the UE to report serving beam information when the UE is operating with  mTRP in the serving cell, the MR MAC CE should include an indication of whether the reported serving RSRP is associated with the first or second TCI state.

# 3 Activation/Deactivation of Semi-Persistent CSI-RS Resources

## 3.1 SP CSI-RSs for L1-RSRP Measurements

Based on the RAN1 LS [R1-2501500] received before the RAN2#129bis meeting, the following agreements were made for the MAC CE design to activate/deactivate a candidate semi-persistent CSI-RS resource for L1-RSRP measurement and reporting [9]:

RAN2 #129bis

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

Based on the above, the current running CR proposes a MAC CE design, shown below, where the SP CSI-RS resource set ID refers to the index of an NZP-CSI-RS-ResourceSet configured under the LTM-Candidate IE of the candidate indicated by the Candidate Cell Index.

Figure 3. MAC CE design for SP CSI-RS resources.

However, there may be an issue with this approach.

The NZP-CSI-RS-ResourceSet under the LTM-Candidate IE is simply a list of NZP-CSI-RS resource indices. This set is not directly used to indicate which RSs the UE should measure and report for this candidate cell. For L1-RSRP measurements associated with an LTM-CSI-ReportConfig, the relevant CSI-RS resources for a candidate cell are defined by the ltm-NZP-CSI-RS-ResourceSet, which is provided under the LTM-CSI-ResourceConfig linked with the LTM-CSI-ReportConfig.

The ltm-NZP-CSI-RS-ResourceSet includes CSI-RS indices from one or more candidate cells and may consist of either periodic or SP CSI-RSs. While it includes indices of CSI-RSs from the LTM-Candidate IE, there is no explicit linkage between:

- The CSI-RSs indicated by an ltm-NZP-CSI-RS-ResourceSet, and

- The CSI-RSs indicated by an NZP-CSI-RS-ResourceSet of a candidate cell.

In fact, an ltm-NZP-CSI-RS-ResourceSet may contain only a subset of the CSI-RSs from a candidate’s NZP-CSI-RS-ResourceSet.

Therefore, if the source cell wants to activate measurements for certain SP CSI-RSs and wants to receive the measurement reports for those activated CSI-RSs, it should do so based on the ltm-NZP-CSI-RS-ResourceSet, which is linked to the reporting configuration. This ensures that the UE can perform the measurement and reporting (via SP or aperiodic reporting) using the correct set of activated SP CSI-RSs.

Observation 16: An ltm-NZP-CSI-RS-ResourceSet contains CSI-RS indices from the LTM-Candidate IE, but it may not directly map to any NZP-CSI-RS-ResourceSet of a candidate cell.

Observation 17: Activating a candidate’s NZP-CSI-RS-ResourceSet may not guarantee activation of all SP CSI-RSs that are actually part of the measurement/reporting configuration.

Proposal 14: Use ltm-NZP-CSI-RS-ResourceSet instead of NZP-CSI-RS-ResourceSet in the MAC CE to indicate the set of SP CSI-RSs to be activated for a candidate cell.

## 3.2 SP CSI-RSs for Early CSI Acquisition Measurements

In addition to the previous LS, another LS [11] was sent by RAN1 from the April meeting, in which RAN1 agreed to support SP CSI-RSs for early CSI acquisition for the target cell. RAN1 also requested RAN2 to provide a MAC CE design for activation/deactivation of SP CSI-RSs in this scenario.

Based on the RAN1 agreements:

- For early CSI acquisition, the CSI measurements (e.g., CQI, PMI, RI) are directly reported to the target cell after the cell switch and are used for fast link adaptation at the target.

- While baseline UEs begin such measurements only after receiving the cell switch command, UEs capable of early measurement can begin measurement before the switch

- For a given candidate cell, up to one CSI report configuration can be provided, and it can be linked to one or more CSI-RSs.

The configuration structure for early CSI (CSI report and CSI-RS resource configurations) may differ from the structure used for L1-RSRP measurements. This is still an open issue in RAN1 and is expected to be further discussed in the upcoming RAN1 meeting (May 2025). For example, in the case of early CSI, the CSI report configuration may be included directly in the LTM-Candidate IE (instead of in the current serving cell configuration as done for L1-RSRP measurement reporting) and linked to one of the NZP-CSI-RS-ResourceSets defined under the LTM-Candidate IE.

If such a structure is adopted, the MAC CE design proposed in the current running CR for L1-RSRP measurements may be suitable for early CSI acquisition instead.

However, in our view RAN2 can still wait for RAN1 to finalize the configuration structure for early CSI before initiating the design of the MAC CE for activation/deactivation of SP CSI-RSs in this context.

Observation 18: RAN1 has agreed to support SP CSI-RSs for early CSI acquisition for the target cell and has requested RAN2 to provide the MAC CE design for activation/deactivation of SP CSI-RSs.

Observation 19: The configuration structure for early CSI may differ from that of L1-RSRP measurements, and RAN1 is expected to discuss this further in the next meeting.

Proposal 15: RAN2 waits for RAN1 to finalize the configuration structure for early CSI before starting the MAC CE design for activation/deactivation of SP CSI-RSs.

# 4	Conclusion

This paper discussed final RAN2 aspects of LTM L1 measurement framework which shall be addressed in Release 19. The following observations have been made:

Observation 1: The only difference between type1 and type3 beams is that type1 denotes a beam that fulfils the condition for the first time whereas type3 is used to label a beam which has been already reported but remains in the “beamsTriggeredList” (such beam was reported as type 1 beforehand).

Observation 2: If type1 and type3 beams fulfil the entry criteria for reporting then any of these could be a suitable candidate for becoming the target beam (i.e. the differentiation does not make sense).

Observation 3: Option 1 brings additional complexity to the UE, as the UE needs to keep track of the beams previously reported and maintain their status.

Observation 4: L2 filtering for LTM could be optionally configured by the NW and applicable just to certain scenarios.

Observation 5: TTT and L2 filtering together are important parameters for tuning mobility KPIs.

Observation 6: A prohibit timer could be applied for the LTM event triggered reporting to avoid too frequent measurement reporting.

Observation 7: There is no reason to deviate from the R18 reporting parameters, where reporting of up to 16 beams can configured.

Observation 8: RAN1 confirms the maximum of 16 beams can be reported and that amount is independent of whether the serving beam is included.

Observation 9: RAN1 may introduce the capability to indicate the maximum number of beams that the UE supports.

Observation 10: The number of reported candidate beams that do not fulfil the reporting criteria should be limited.

Observation 11: The offset value configuration and limiting UE measurements per candidate cell are different features and should not be mixed.

Observation 12: Any configuration to limit measurements for certain candidate cells or RS of certain candidate cells, at least in R19 should be done via CSI resource configuration.

Observation 13: Offset value can be configured for a candidate cell as part of the reporting configuration. The candidate ID.

Observation 14: If the offset value is not configured for the certain candidate cell or the candidate ID is not present it should mean that offset value is not applied for the beams of the candidate cell when evaluating the reporting criteria.

Observation 15: For Rel-18/19 LTM and Rel-19 CLTM, RAN2 has supported the cell switch to a target PCell with mTRP deployment (i.e., 2 TA).

Observation 16: An ltm-NZP-CSI-RS-ResourceSet contains CSI-RS indices from the LTM-Candidate IE, but it may not directly map to any NZP-CSI-RS-ResourceSet of a candidate cell.

Observation 17: Activating a candidate’s NZP-CSI-RS-ResourceSet may not guarantee activation of all SP CSI-RSs that are actually part of the measurement/reporting configuration.

Observation 18: RAN1 has agreed to support SP CSI-RSs for early CSI acquisition for the target cell and has requested RAN2 to provide the MAC CE design for activation/deactivation of SP CSI-RSs.

Observation 19: The configuration structure for early CSI may differ from that of L1-RSRP measurements, and RAN1 is expected to discuss this further in the next meeting.

And the following has been proposed:

Proposal 1: RAN2 is asked to adopt Option 3 and therefore reject the working assumption taken at RAN2#129bis.

Proposal 2: RAN2 reconsiders previous decision and specifies L2 filtering of L1 measurements for event triggered reporting.  The L2 filtering at MAC is applied for candidate RS in similar manner as L3 Beam filtering (see TS 38.300, section 9.2.4).

Proposal 3: The UE uses truncated MR MAC CE when it is not able to report all the beams that triggered the report or beams that have earlier triggered the report (i.e. type1 or type3 if Option 1 is adopted), up to the configured number of beams.

Proposal 4: The truncated MR MAC CE can only include RSRIs of beams that fulfil or have fulfilled the triggering conditions earlier.

Proposal 5: If the triggered beam (the beam or beams that triggered the report) is the type1 the UE includes first the beam or beams that triggered the report:

After that the UE includes the beams that are reported for leaving condition (type2), if these exist

After that the UE includes the beams that are reported as type3 (i.e. still meet the triggering condition but were reported already beforehand).

Proposal 6: If Option 3 is adopted then then type1 and type3 are not differentiated and are reported first in MR MAC CE.

Proposal 7: RAN2 to support a prohibit timer for event triggered reporting. The prohibit timer is applied per reporting configuration.

Proposal 8: For reporting beams in MR MAC CE, up to 16 beams can be included. Potential candidate values for the number of beams are: {1,2,3,4,8,12,16}.

Proposal 9: A maximum limit should be set for the number of candidate beams in the MR MAC CE that do not fulfil the reporting criteria.

Proposal 10: RAN2 to slightly revise the existing agreement to: offset value can be configured in the reporting configuration per candidate cell. If offset value is configured, the offset value is used for event evaluation for the RS included in the CSI resource configuration for that candidate cell. If the offset value is not configured for a candidate cell, the UE does not apply offset value for the candidate cell.

Proposal 11: Coexistence of LTM event-triggered reporting and mTRP operation at the serving cell is supported in Rel-19.

Proposal 12: If the UE is configured with mTRP operation, one of the following options should be selected for event evaluation:

- Option 1: UE may select which of the two TCI states it uses (i.e. left to UE implementation).

- Option 2: UE selects the TCI state (beam) with higher RSRP for event evaluation

Proposal 13: If the network configures the UE to report serving beam information when the UE is operating with  mTRP in the serving cell, the MR MAC CE should include an indication of whether the reported serving RSRP is associated with the first or second TCI state.

Proposal 14: Use ltm-NZP-CSI-RS-ResourceSet instead of NZP-CSI-RS-ResourceSet in the MAC CE to indicate the set of SP CSI-RSs to be activated for a candidate cell.

Proposal 15: RAN2 waits for RAN1 to finalize the configuration structure for early CSI before starting the MAC CE design for activation/deactivation of SP CSI-RSs.

# References

[1]	RP-242356 Revised Work Item: NR mobility enhancements Phase 4 3GPP TSG RAN Meeting #105, Sep 9-12, 2024

[2] R2-2403731 	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #125bis Changsha, China, April 15 – 19, 2024

[3] R2-2405701	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #126	 Fukuoka, Japan, May 20 – 24, 2024

[4] R2-2409211 Report from session on V2X/SL, R18/19 MOB, and R19 NES 3GPP TSG-RAN WG2 Meeting #127bis Hefei, China, October 14th – 18th, 2024

[5] R2-2410911 Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #128, Orlando, USA, November 18th – 22nd, 2024

[6] R2-2501331 Report from session on R17/18 V2X/SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, February 17 – 21, 2025

[7] R1-2501500 LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells 3GPP TSG-RAN WG1 Meeting #120, Athens, Greece, February 17 – 21, 2025

[8] R2-2501485 LS on number of beam measurements in the measurement report MAC CE TSG-RAN WG2 Meeting #129, Athens, Greece, February 17 – 21, 2025

[9] R2-2502981 Report from session on R18 SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #129bis Wuhan, China, April 7th – 11th, 2025

[10] R1-2503077 Reply LS on number of beam measurements in the measurement report MAC CE

[11] R1-2503079 LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells