3GPP RAN WG2 Meeting #130	R2-2504140

St Julians, Malta May 19th – 23rd, 2025

Agenda Item:	8.9.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining open issues for Idle mode procedure for IoT-NTN

Document for:	Discussion, Decision

# Introduction

Following open issues are identified during the post e-mail discussion of RRC Running CR for TS36.304. The corresponding issues are marked as FFS in the running CR.  In this document we provide rapporteur analysis on these issues and proposals for further discussion and conclusion at RAN2-130.

# Open Issues

## Support for acceptable cell camping for IoT-NTN for PWS

In TS36.304 the idle mode functions for NB-IoT excludes acceptable cell camping. The reason for this exclusion is based on the assumptions that NB-IoT devices are expected to operate only with registered PLMN alternatively only with valid subscription. There were no use-case for the UE to camp to acceptable cell which is outside registered PLMN or operation of NB-IoT UE without valid USIM subscription.

Moreover the primary need for acceptable cell functionality is to enable te UE to make emergency call outside the registered network that is not supported for NB-IoT. In addition to emergency calls the UE are allowed to receive ETWS/PWS notifications from acceptable cell as part of limited services operation.

In Rel-19 support for ETWS/PWS/CMAS for NB-IOT is introduced. However the work item scope is not clear on whether the support should be extended for reception of these messages outside registered network.

In our view, current NB-IoT devices have the functionality defined for suitable cell operation ( idle mode tasks in the registered network).  Support for acceptable cell camping requires considerable UE behaviour changes.

In addition to the support for NB-IoT UE for acceptable cell camping the UE power consumption related to acceptable cell idle mode operation without emergency calls just for PWS/ETWS reception also needs to be studied.

Observation 1: Support of acceptable cell camping requires additional UE functionality over the simple UE capability for PWS/ETWS reception from suitable cells.

Observation 2: The additional power consumption for idle mode operation without emergency call (only for ETWS reception) support needs to be justified.

Based on the above observations we prefer the ETWS/PWS/CMAS operation for NB-IoT is supported only for suitable cell.  The acceptable cell camping can be considered when NB-IoT UE supports voice services and emergency calls in future releases. However we propose that RAN2 to discuss and conclude on additional support needed for acceptable cell.

Proposal 1:  RAN2 to discuss and conclude whether ETWS/PWS/CMAS reception for NB-IoT can be limited only to suitable cells.

## Emergency call support when SF operation parameter is transmitted with barred.

As per 36.304, if the UE supports emergency calls,. Section 5.3.3 specifies the following related to restriction for emergency call support.

A restriction on emergency calls, if needed, is indicated by the field ac-BarringForEmergency TS 36.331 [3]. If access class 10 is indicated as barred in a cell, UEs with access class 0 to 9 or without an IMSI are not allowed to initiate emergency calls in this cell. For UEs with access classes 11 to 15, emergency calls are not allowed if both access class 10 and the relevant access class (11 to 15) are barred. Otherwise, emergency calls are allowed for those UEs.

Full details of operation under "Access class barred list" are described in TS 22.011 [4].

For E-UTRA connected to 5GC, the restriction on emergency calls is indicated by access control information of access category 2 under unified access control TS 36.331 [3].

However when SF operation mode is included the emergency calls may not be possible due to non availability of feeder link connectivity. Hence further clarification needed on handling emergency calls in SF mode. This also depends on SA2 assumption on support for IMS calls and emergency calls in SF mode.

Following options are possible

- O1:Specification text can indicate that emergency calls are not supported of SF-Operation mode is transmitted.

- O2:It is upto SA2 to clarify the support for IMS services and emergency services for SF operation. EN can be removed and updated later based on SA2 specification status.

Proposal 2: RAN2 to down-select any of the above option related to emergency call support in SF mode operation.

# Conclusions

Based on the analysis on the open issues for idle mode operation for IoT-NTN we make following observations and proposals.

Issue 1:  Acceptable camping for NB-IoT for ETWS/PWS reception

Observation 1: Support of acceptable cell camping requires additional UE functionality over the capability for PWS/ETWS reception from suitable cells.

Observation 2: The additional power consumption for idle mode operation without emergency call (only for ETWS reception) support needs to be justified.

Proposal 1:  RAN2 to discuss and conclude whether ETWS/PWS/CMAS reception for NB-IoT can be limited only to suitable cells.

Issue 2:  Emergency call support in SF operation

Proposal 2: RAN2 to down-select any of the following option related to emergency call support in SF mode operation.

O1:Specification text can indicate that emergency calls are not supported when cell is operating in store and forward mode

O2:It is upto SA2 to clarify the support for IMS services and emergency services for SF operation. EN can be removed and updated if needed based on SA2 specification status.

# References

[1] R2-2504149 Running CR for TS3.304

[2] RP-240776 WID for IoT-NTN Enhancements