3GPP RAN WG2 Meeting #130 	R2-2504180

# St.Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	8.9.3

Source:	InterDigital, Inc.

Title:	CB-EDT.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#126, the following was agreed:

Agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements

In RAN2#127, the following was agreed:

Agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In RAN2#127bis, the following was agreed:

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

. In RAN2#128 the following was agreed.

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In RAN2#129 the following was agreed.

Agreements:

1.	It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used

2.	RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC

3.	RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects)

-	Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

-	Frequency domain resources for (N)PUSCH occasions

-	repetition number

-	(N)PDCCH resource

-	MCS

4.	For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels.

5.	The UE shall at most have one ongoing CB EDT procedure at any time.

6.	The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT.

7.	The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels.

8.	CB EDT Config has three RSRP thresholds for eMTC for the four CE levels.

7.	As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config.

8.	RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature

Working assumption:

- One CB-MSG4 can target multiple UEs simultaneously (FFS on the details)

- For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today

The UE first selects the next DSA transmission window and then randomly select K replicasinside the window.

RAN2 assumes that a pointer solution is not needed in Rel-19

In RAN2#129bis the following was agreed.

Agreements;

1.	Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

2.	We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

3.	For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same).

4.	We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

5.	We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

6.	If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

7.	The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

8.	The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

9.	RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

(CB-Msg3-EDT configuration for eMTC)

10.	For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

11.	For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

12.	We will not support TDD related parameters.

13.	For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

14.	For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

15.	For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

(CB-Msg3 configuration for NB-IoT)

16.	For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

17.	For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

-	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

-	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

-	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

-	MCS configuration for NPUSCH (as in npusch-MCS-r16).

-	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

-	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

NOTE: confirm with RAN1 is needed

18.	For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

Agreements (part 2):

1.	For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

2.	The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

3.	The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

4.	FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

Agreements (part3):

1.	The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

2.	The Msg4 monitoring window configuration (e.g. length) is CE level specific

3.	RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

4.	RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT

-	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

-	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

5.	RAN2 also intends to support CB-msg3-EDT for MT cases

6.	The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

7.	Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

8.	The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

9.	Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

10.	Backoff information could be included in CB-Msg4.

11.	L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported.

12.	HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

# Msg3 transmission without msg1/RAR

In RAN2#129 it was agreed to use 2 thresholds for CE level selection (3 for eMTC).  In addition, a “minimum” threshold was agreed to enable/disable CB-EDT. If the UE measured RSRP is below the minimum threshold then PRACH shall be used.

It was not agreed whether the thresholds are the same or separate from the PRACH thresholds.

For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels.

While re-using precisely the same thresholds would be simpler from a specification and implementation point of view, technically there is a case for introduction of new thresholds, since the coverage level selection thresholds may need to be different for the different physical channels (i.e. PRACH thresholds may not be suitable for PUSCH in all cases). Hence, we propose to introduce new thresholds.

Proposal 1: Introduce new thresholds for CE level selection for PUSCH, separate to the existing PRACH thresholds, and a new “minimum” threshold for selecting CB-EDT/PRACH.

An implication of this is that the UE, if detecting that the RSRP is below the minimum threshold for CB-EDT, needs to then check the thresholds for PRACH. In addition, in case of CB-EDT failure, the UE should perform PRACH. One option is to attempt CB-EDT, and if a failure occurs to perform PRACH CE level selection as per legacy. Another option would be to always select the maximum repetitions for PRACH after CB-EDT selection. For the sake of simplicity we think the legacy PRACH procedure can be used for both of these cases, to avoid modifying or adding special conditions for the legacy PRACH procedure.

Proposal 2: If the UE determines that the RSRP is below the minimum threshold for CB-EDT, then perform CE level selection for PRACH as per legacy.

Proposal 3: Upon detecting CB-EDT failure then perform CE level selection for PRACH as per legacy.

In the previous meeting, a per-CE level parameter for maximum re-attempts was agreed.

9.	Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

We need to decide what happens in case of failure of a specific CB-EDT CE level

For example, if the UE selects coverage level 0 but the CB-Msg3 transmission is not successful, then what happens next? In our opinion there is already a well-defined fallback mechanism for PRACH and this should be re-used also for CB-EDT.

In other words, if transmission fails using the selected coverage level then the UE selects the next coverage level and re-tries. Once all of the CB-Msg3 coverage levels are tried, then UE considers CB-EDT to have failed, and performs PRACH CE level selection as per legacy. The other option would be to immediately fallback to using RACH without trying other CE levels for CB-EDT.

Proposal 4: Select from the following options for determining CB-EDT failure (what happens when UE reaches the configured maximum attempts for the current CE level).

Option 1) Re-use the per-CE level PRACH failure handling for CB-Msg3 failures – if transmission fails using the selected CE level configuration, then select the next CE level configuration and retry. Once all CE levels have been tried then CB-EDT is considered to have failed.

Option 2) If CB-EDT fails for the first selected CE level then CB-EDT is considered to have failed, without attempting other CE levels.

# Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

If PUR is used, it is possible to terminate EDT without using RRC. As described in 36.300 [2] :

7a	If the (ng-)eNB is aware that there is no pending downlink data or signalling, the (ng-)eNB can send a Layer 1 ACK optionally containing a Time Advance Adjustment to the UE to update the TA and terminate the procedure.

7b	If the (ng-)eNB is aware that there is no further data or signalling, the (ng-)eNB can send a Time Advance Command to update the TA and terminate the procedure.

Observation 1: PUR feature can terminate the EDT procedure without using RRC, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

The main question as to whether early termination of EDT (or PUR) works in practise is “how does eNB know that there is no pending downlink data from the application layer?“

Proposal 5: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

The UE can transmit an indication (L1Ack) in PURConfigurationRequest, which informs the eNB a preference for using RRCEarlyDataComplete. Note that, although the figure states “L1 ACK” this is named “rrc-ACK” in the RRC specification and it indicates RRC response message is preferred by the UE for acknowledging the reception of a transmission using PUR. This is an indirect way to specify that the UE can send some information about whether it expects downlink data in response to the uplink. It is then up to the eNB implementation what (if anything) to do with that knowledge. The main issue with terminating EDT early is when there is some downlink application layer data to send in response. If the procedure is terminated early then the network must page the UE in order to transmit this, which is even more inefficient than waiting and terminating using RRC.

Observation 2: With the PUR feature it is possible for the UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRC.

Since for CB-Msg3 we will not be using PURConfigurationRequest, there needs to be another way to indicate from the UE whether RRC response message is preferred by the UE for acknowledging the reception of a transmission. The most convenient place would be in the RRCEarlyDataRequest.

Proposal 6: Introduce “rrc-ACK”, to RRCEarlyDataRequest to indicate whether an RRC Response Message is preferred by the UE for CB-Msg3, similar to PURConfigurationRequest

It was agreed in the RAN2#128:

Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

In previous contributions, we listed various options for early termination of EDT. With the above agreement, it seems that the most suitable approach for terminating early is use of contention resolution MAC CE.

However, UE needs to know whether to terminate only the uplink transmissions and wait for an RRC response, or to terminate the procedure entirely and return to RRC_IDLE. For this purpose, an additional flag would be needed in the contention resolution MAC CE.

Proposal 7: Contention Resolution MAC CE includes a new flag indicating whether to terminate only the uplink transmissions, or to terminate the entire procedure and return to RRC_IDLE.

# Conclusion

In this contribution we discuss uplink capacity enhancements and make the following proposals for discussion:

Msg3 transmission without msg1/RAR

Proposal 1: Introduce new thresholds for CE level selection for PUSCH, separate to the existing PRACH thresholds, and a new “minimum” threshold for selecting CB-EDT/PRACH.

Proposal 2: If the UE determines that the RSRP is below the minimum threshold for CB-EDT, then perform CE level selection for PRACH as per legacy.

Proposal 3: Upon detecting CB-EDT failure then perform CE level selection for PRACH as per legacy.

Proposal 4: Select from the following options for determining CB-EDT failure (what happens when UE reaches the configured maximum attempts for the current CE level).

Option 1) Re-use the per-CE level PRACH failure handling for CB-Msg3 failures – if transmission fails using the selected CE level configuration, then select the next CE level configuration and retry. Once all CE levels have been tried then CB-EDT is considered to have failed.

Option 2) If CB-EDT fails for the first selected CE level then CB-EDT is considered to have failed, without attempting other CE levels.

Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

Observation 1: PUR feature can terminate the EDT procedure without using RRC, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

Proposal 5: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

Observation 2: With the PUR feature it is possible for the UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRC.

Proposal 6: Introduce “rrc-ACK”, to RRCEarlyDataRequest to indicate whether an RRC Response Message is preferred by the UE for CB-Msg3, similar to PURConfigurationRequest

Proposal 7: Contention Resolution MAC CE includes a new flag indicating whether to terminate only the uplink transmissions, or to terminate the entire procedure and return to RRC_IDLE.

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”