3GPP TSG-RAN WG2 Meeting #130											        R2-2504201

Malta, 19-23 May 2025

Agenda item:			8.8.4

Title:						Remaining issues for MBS service continuity over NTN

Source:					Continental Automotive

Document for:		Discussion and Decision

# Introduction

The Non-Terrestrial Networks (NTN) for NR Phase 3 work item aims at specifying support for broadcast services via NR NTN in Rel-19 [1]:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

During RAN2#125bis [2] the following agreements were made:

Agreements:

- For MBS broadcast service we don’t restrict the work to any satellite constellation type

- We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

Further, during RAN#126 [3], RAN2#127bis [4], RAN2#128 [5], and RAN2#129bis [6] the following agreements were made:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

Agreements:

1.	For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

Agreements:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB..

Agreements:

1.	RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

2.	If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

3.	If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

4.	The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

In this paper, we share further remaining issues for MBS service continuity over NTN.

# 2. Discussion

The concept of “intended MBS service areas” was introduced due to regional regulations or other circumstances that may require to constrain the reception of MBS over NTN. Already in early discussions [2], RAN2 agreed to not restrict the work to any satellite constellation type. Further, RAN2 agreed [3] that for MBS broadcast service, both EFC and EMC are supported.

Different scenarios were discussed in which MBS content reception is restricted to a portion of an NTN cell or where intended MBS service areas overlap multiple NTN cells. The intended MBS service areas, also referred to “ISA” in some contributions, represent geographically confined areas on ground. However, they may be served by fast moving quasi-earth-fixed or earth-moving cells. Hence, the UE may experience frequent changes in MBS broadcast service and the associated ISAs.

Fig. 1 depicts an exemplary scenario, where fast moving quasi-earth-fixed or earth-moving NTN cells serve several ISAs on ground. At T=t0, the UE (located in ISA #0) is served by cell #0, and at T=t1, the UE (still located in ISA #0) is served by cell #1.

Fig. 1 – Different fast moving quasi-earth-fixed or earth-moving NTN cells serve the UE at times T=t0 and T=t1.

This issue is even more serious for mobile UEs, as illustrated in Fig. 2 below. At T=t0, the UE (located in ISA #0) is served by cell #0. At T=t1, the UE is located in ISA #1, where the UE may be served by cell #1 or cell #2.

Fig. 2 – Different fast moving quasi-earth-fixed or earth-moving NTN cells serve the UE at times T=t0 and T=t1.

Observation 1: In scenarios with quasi-earth-fixed or earth-moving cells, which are under the scope of RAN2 agreements, NTN cell(s) serving an intended MBS service area may change constantly over time, which will result in frequent updates in MBS service(s) and the associated ISAs.

Thus, the network should provide further guidance to the UE for MBS service selection in an intended MBS service area. For example, in addition to broadcasting the intended MBS service area, the network provides MBS-specific estimated service times for intended MBS service areas, where serving NTN cell(s) change continuously. Based on those MBS-specific service time estimates, the UE decides whether and when to initiate MBS service.

Proposal 1: The network provides, in addition to broadcasting the intended MBS service area, MBS-specific service time estimates for intended MBS service areas, where serving NTN cell(s) continuously change.

# Conclusions

The following proposals have been made in this contribution:

Observation 1: In scenarios with quasi-earth-fixed or earth-moving cells, which are under the scope of RAN2 agreements, NTN cell(s) serving an intended MBS service area may change constantly over time, which will result in frequent updates in MBS service(s) and the associated ISAs.

Proposal 1: The network provides, in addition to broadcasting the intended MBS service area, MBS-specific service time estimates for intended MBS service areas, where serving NTN cell(s) continuously change.

# References

- RP-234076, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN1 Vice-Chair, RAN#102, December 11-15, 2023, Edinburgh, UK.

- R2-2403733, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#125bis, Changsha, China, April 15th – 19th, 2024.

- R2-2405703, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#126, Fukuoka, Japan, May 20th-24th, 2024.

- R2-2409213, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#127bis, Hefei, China, Oct. 14th-18th, 2024.

- R2-2410913, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#128, Orlando, USA, Nov. 18th-22nd, 2024.

- R2-2502983, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#129bis, Wuhan, China, April 7th-11th, 2025.