3GPP TSG-RAN WG2 Meeting #130	R2-2504208

St. Julian’s, Malta,  May 19th – 23rd , 2025

Agenda item:	8.8.4

Source:	Samsung

Title:	Open issues on Broadcast service area

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NTN, one objective is to support MBS broadcast service for smaller areas in an NTN cell. The detailed justification and objective in WID are provided as follows.

MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service. Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this contribution, we continue to share our views on open issues for the MBS broadcast intended service area.

# Discussion

## Area-specific intended service area

Regarding the property of ISA, the following aspects have been agreed.

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

- RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

- RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

As agreed, the ISA(s) of MBS broadcast service(s) can include portions of one or more NTN cells (e.g., the current serving cell and neighbor cells) and is semi-static which does not cause frequent updates. Therefore, considering the ISA(s) across multiple NTN cells, the SIBxx containing the geographic information for ISA(s) can be area-specific and common to a cluster of cells. The update and acquisiton of SI can be reduced for area-specific SIB. This is same as SIB20/21 for MBS.

Observation 1: RAN2 has agreed the ISA(s) of MBS broadcast service(s) can include portions of one or more NTN cells (e.g., the current serving cell and neighbor cells) and is semi-static which does not cause frequent updates.

Proposal 1: The new SIB of the geographic information for ISA(s) can be either area-specific or cell-specific (no spec. Impact).

Based on the above agreements, one possible scenario is that for some interested MBS broadcast services, the ISAs cover only portions of the serving cell, say type-1 ISA, while for some other interested services the ISAs cover the whole current serving cell and portions of neighbor cells, say type-2 ISA, as shown in the Figure below. In this case, if the NW does not provide the type-2 ISA for the serving cell and only provide type-1 ISA, the UE may determine it is not in any ISA in the serving cell and do not acquire MCCH. However, UE is still in type-2 ISA and should acquire MCCH to receive the interested services of type-2 ISA. To avoid such wrong UE behavior, type-2 ISA should be provided.

One approach is the serving cell’s SIB include explicit geographic area of type-1 ISA and the SIB is area-specific (i.e., common for the current serving cell and relevant neighbor cells). This can reduce SI update and acquisition. Another approach is the serving cell’s SIB includes a one-bit indicator for type-1 ISA without explicit geographic area. This allows smaller overhead for the serving cell SIB, but the SIB is cell-specific.

Observation 2: If the serving cell SIB only include the ISA that covers a small portion of the serving cell (type-1 ISA) and does not include the ISA that covers the whole current serving cell and portions of neighbor cells (type-2 ISA), UE may wrongly determine it is not in any type-1 ISA in the serving cell and do no acquire MCCH, however UE may still in type-2 ISA and should acquire MCCH.

Proposal 2: For the ISA that covers the whole current serving cell and portions of neighbor cells, the serving cell’s SIB can

- Option 1: include explicit geographic area for this type of ISA and be area-specific.

- Option 2: include a one-bit indicator to indicate this type of ISA and be cell-specific.

## Service continuity

In the last meeting, it is agreed that UE can prioritize or deprioritize a certain frequency in cell reselection based on ISAs of MBS broadcast services. The open issue is whether/how the association between intended service area and frequencies is acquired.

UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. - FFS whether and how the association between intended service area and frequencies is acquired.

It has been discussed that UE may know the frequency-ISA mapping from the current provided information, by combining the service-FSAI mapping in USD, the FSAI-frequencies mapping in SIB21, and the ISA-service mapping in MCCH. However, the association between each two is not 1-1 mapping.

Based on the higher layer message User Service Description (USD), see appendix, UE can know the MBS frequency selective area (FSA) ID for a MBS service, which identifies a preconfigured area within which, and in proximity to, the cell(s) are announcing the MBS FSA ID and the associated frequency corresponding to this MBS Distribution Session Announcement. However, it is not clear what is the granularity of “the preconfigured area”.

For a MBS service, if “the preconfigured area” identified by FSA ID is small, UE knows the frequency at the ISA with no ambiguity. If “the preconfigured area” identified by FSA ID is large, e.g., in the form of a Tracking Area Identifier (TAI) list and/or Cell ID list, multiple frequencies can map to one MBS FSA ID, UE cannot determine the frequency at the ISA, for which case the association between ISA and frequency should be provided additionally.

Observation 3: It is not clear what is the granularity of “the preconfigured area” identified by MBS FSA ID. If “the preconfigured area” identified by FSA ID is large, e.g., in the form of a Tracking Area Identifier (TAI) list and/or Cell ID list, multiple frequencies can map to one MBS FSA ID.

Due to this ambiguity, UE may end up with one service associated to multiple frequencies (e.g., for multiple cells) and associated to multiple ISAs. For instance, by combining the service-FSAI mapping in USD and the FSAI-frequencies mapping in SIB21, UE knowns service #1 is provided in f1 and f2. By the ISA-service mapping in MCCH, UE knows service #1 is provided in ISA #1 and ISA #2. Although UE knows it is in ISA #1 based on its location, UE cannot determine ISA #1 is provided in f1 or f2 so it may prioritize a wrong frequency. Hence, it is necessary to provide the association between ISA and frequency.

Observation 4: By combining the existing information of service-FSAI mapping in USD, FSAI-frequencies mapping in SIB21, and ISA-service mapping in MCCH, UE will consider one service associated to multiple frequencies (e.g., for multiple cells) and associated to multiple ISAs, but cannot know which frequency is used in the ISA UE located in.

Proposal 3: Provide the association between ISA ID and frequency in SIB21.

# Conclusion

In this contribution we discussed the following proposals.

Observation 1: RAN2 has agreed the ISA(s) of MBS broadcast service(s) can include portions of one or more NTN cells (e.g., the current serving cell and neighbor cells) and is semi-static which does not cause frequent updates.

Proposal 1: The new SIB of the geographic information for ISA(s) can be either area-specific or cell-specific (no spec. Impact).

Observation 2: If the serving cell SIB only include the ISA that covers a small portion of the serving cell (type-1 ISA) and does not include the ISA that covers the whole current serving cell and portions of neighbor cells (type-2 ISA), UE may wrongly determine it is not in any type-1 ISA in the serving cell and do no acquire MCCH, however UE may still in type-2 ISA and should acquire MCCH.

Proposal 2: For the ISA that covers the whole current serving cell and portions of neighbor cells, the serving cell’s SIB can

- Option 1: include explicit geographic area for this type of ISA and be area-specific.

- Option 2: include a one-bit indicator to indicate this type of ISA and be cell-specific.

Observation 3: It is not clear what is the granularity of “the preconfigured area” identified by MBS FSA ID. If “the preconfigured area” identified by FSA ID is large, e.g., in the form of a Tracking Area Identifier (TAI) list and/or Cell ID list, multiple frequencies can map to one MBS FSA ID.

Observation 4: By combining the existing information of service-FSAI mapping in USD, FSAI-frequencies mapping in SIB21, and ISA-service mapping in MCCH, UE will consider one service associated to multiple frequencies (e.g., for multiple cells) and associated to multiple ISAs, but cannot know which frequency is used in the ISA UE located in.

Proposal 3: Provide the association between ISA ID and frequency in SIB21.

# Reference

RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

# Appendix

TS 38.300

##### 16.10.6.5.1	Service Continuity in RRC_IDLE or RRC_INACTIVE

Mobility procedures for MBS reception allow the UE to start or continue receiving MBS service(s) when changing cells. The gNB may indicate in the MCCH the list of neighbour cells providing the same MBS broadcast service(s) as provided in the serving cell. This allows the UE, e.g., to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission. To avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies, the UE is made aware of which frequency is providing which MBS broadcast services via PTM, through User Service Description, as defined in TS 26.517 [46], or the combination of the following:

-	User Service Description (USD);

-	SIB21, as defined in clause 7.3.1.

NOTE:	UE can request unicast reception of the service after moving to a cell not providing the MBS broadcast service(s) using PTM transmission.

- TS 26.517

## 5.0	Overview

MBS User Service Announcement is needed in order to advertise MBS User Services in advance of, and potentially during, the MBS User Service Sessions described. MBS User Service Announcement (as defined in clauses 4.5.7 and 4.5.8 of TS 26.502 [3]) is provided by means of an MBS User Service Description, the syntax of which is defined in this clause.

- TS 26.502

### 4.5.7	MBS User Service Announcement parameters

This entity models an MBS User Service Announcement, which is compiled by the MBSF and used to advertise the current or imminent availability of an MBS User Service in the MBS System. The baseline parameters for an MBS User Service Announcement are listed in table 4.5.7-1 below:

Table 4.5.7-1: Baseline parameters of MBS User Service Announcement entity

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| External service identifiers | 1..* | MBS Application Provider | A unique identifier used by the MBSF Client to distinguish between MBS User Services. If assigned in a globally unique manner, this identifier may be useful to the MBSF Client in correlating the MBS User Service with the same service delivered by a different system. |
| Service class | 1..1 |  | The class of the MBS User Service, expressed as a term identifier from a controlled vocabulary. |
| Start date–time | 0..1 |  | The point in time from which this MBS User Service Announcement is valid. If not present, the announcement is already valid. |
| End date–time | 0..1 |  | The point in time after which this MBS User Service Announcement is no longer valid. If not present, the announcement is valid indefinitely. |
| Service names | 1..* |  | A set of distinguishing names for the MBS User Service, one per language. |
| Service descriptions | 1..* |  | A set of descriptions of the MBS User Service, one per language. |
| Main service language | 0..1 |  | The main language of the MBS User Service. |
| MBS Distribution Session Announcements | 1..* | MBSF | The set of MBS Distribution Session Announcements (see clause 4.5.8) currently associated with this MBS User Service Announcement. |

### 4.5.8	MBS Distribution Session Announcement parameters

This entity models an MBS Distribution Session Announcement, which is compiled by the MBSF and used to advertise the current or imminent availability of an MBS Distribution Session in the MBS System. The baseline parameters for an MBS Distribution Session Announcement are listed in table 4.5.8-1 below:

Table 4.5.8-1: Baseline parameters of MBS Distribution Session Announcement entity

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| MBS Session Identifier | 1..1 | MB-SMF | The Temporary Mobile Group Identity (TMGI) or Source-Specific Multicast (SSM) IP address of the MBS Distribution Session from which this announcement is derived. |
| MBS Frequency Selection Area (FSA) Identifier | 0..1 | MBS Application Provider or MB-SMF | (Broadcast MBS Session only.) Identifies a preconfigured area within which, and in proximity to, the cell(s) are announcing the MBS FSA ID and the associated frequency corresponding to this MBS Distribution Session Announcement (see NOTE) |
| Radio parameters | 0..* | MBSF | (Broadcast MBS Session only.) Radio transmission parameters of this MBS Distribution Session in its target service areas. The values are obtained by the MBSF from the OAM using the MBS Frequency Selection Area (FSA) Identifier (see above) as the lookup key. |
| Target UE classes | 0..* | MBS Application Provider | (Broadcast MBS Session only.) Indicates whether the MBS Distribution Session described by this announcement is suitable for consumption by NR RedCap UEs and/or non-NR RedCap UEs as defined in clause 6.19 of TS 23.247 [5]. |
| Distribution method | 1..1 | MBS Application Provider | The distribution method (as defined in clause 6) of the MBS Distribution Session from which this announcement is derived. |
| Session Description parameters | 1..* | MBSF | Additional parameters needed to receive the MBS Distribution Session from which this announcement is derived, including relevant User Plane traffic flow parameters. |
| Service protection description | 0..1 | MBSSF | The security parameters for the MBS Distribution Session (see clause W.4.2 of TS 33.501), including: - Which form of transport security protection is in force, whether UICC key management (see TS 33.246 [19]) is selected and/or whether 2G GBA security (see TS 33.246 [19]) is selected. - The MBS Session Key (MSK) identifier and key domain. - The address of the key management server (FQDN of the MBSSF) when user plane security is in force. |
| Time service endpoints | 0..N | MBSF | A set of endpoints provided by the MBS AS and used by the MBS Client to synchronise its clock with the needed precision. |
| NOTE: Used to guide frequency selection by the UE for a broadcast MBS Session. |  |  |  |