3GPP TSG-RAN WG2 Meeting #130	R2-2504211

St. Julian’s, Malta, May 19th – 23rd, 2023

Agenda item:	8.12.2

Source:	Samsung

Title:	Open issues on Asymmetric DL sTRP/UL mTRP

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In the so-called “asymmetric DL sTRP/UL mTRP” scenario, we consider dense UL-only TRPs, where UEs receive DL from a macro DL/UL TRP (a.k.a. anchor TRP) and can transmit UL to micro UL-only TRPs, as shown in the Figure below.

In this scenario, we consider a pathloss offset between the DL pathloss RS from the DL TRP and the actual pathloss for UL-only TRP.  Due to this pathloss offset, UL power control to the UL-only TRP are determined differently from the legacy power control. The PL offset is used in UL power control in a way that UE reuses the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. The PL offset is applied in PUCCH, PUSCH, SRS, PDCCH-order PRACH as RAN1 agreed.

The objective of this topic is specified in WID [1].

Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this paper, we discuss the open issues on this objective [2].

# Discussion

## When PL offset is not configured

In the last meeting, regarding the 2TA operation for asymmetric TRP with PL offset configured, the following agreement is made.

For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes: - • remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value; - • a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

On the other hand, as in LS R1-2503091 RAN1/4 also consider the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), which is for the case UE is configured with SSB-MTC-additionalPCI (i.e., Rel-17 inter-cell beam management (ICBM) scenario). For this scenario, RAN1 has made the following agreement.

Agreement - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG. - Above applies for the case UE is configured with SSB-MTC-additionalPCI.

Based on the objective in WID and the above RAN1 agreement, we can see Rel-19 2TA is not limited to the scenario of asymmetric DL sTRP/UL mTRP with PL offset. Rel-19 2TA is for single-DCI mTRP operation where coresetPoolIndex may be not configured or configured with only one value, and sDCI mTRP 2TA is applied to both the scenarios that PL offset is configured (i.e., asymmetric DL sTRP/UL mTRP) and PL offset is not configured (i.e., ICBM).

Observation 1: RAN1/4 also consider the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), i.e., ICBM. Rel-19 sDCI mTRP 2TA is applied in this scenario.

Proposal 1: 2TA operation is supported for single-DCI mTRP without the restriction that coresetPoolIndex needs to be configured with more than one value, and single-DCI mTRP 2TA is applied to both the scenarios that PL offset is configured (i.e., asymmetric DL sTRP/UL mTRP) and PL offset is not configured (i.e., ICBM).

In the current tag2 configuration for Rel-18 mDCI mTRP 2TA, the following field description have an restriction on coresetPoolIndex.

tag2 This field is used to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex.

To support Rel-19 sDCI mTRP 2TA, the restriction is not applicable, meaning coresetPoolIndex can be not configured or configured with only one value. However, for Rel-18 mDCI mTRP 2TA, the restriction is still needed for backward compatibility. We need to discuss how to keep the configuration restriction for Rel-18 mDCI mTRP 2TA and also make the field applicable to Rel-19 sDCI mTRP 2TA. Some options are listed for discussion.

Proposal 1-1: Discuss how to capture field description for tag2 to keep backward compatibility for Rel-18 mDCI mTRP 2TA and also make the field applicable to Rel-19 sDCI mTRP 2TA.

- Option 1: as a clean solution, introduce a new RRC parameter per BWP that explicitly enables the Rel-19 sDCI-mTRP 2TA, and clarify in FD of tag2 to include all cases where tag2 is configured that “it is optionally configured in a serving cell for mDCI mTRP 2TA if coresetPoolIndex for a BWP is configured with more than one value, and for sDCI mTRP 2TA if [the new parameter is configured].”;

- Option 2: use coresetPoolIndex to implicitly enable Rel-18 mDCI mTRP 2TA or Rel-19 sDCI mTRP 2TA, i.e., update the FD of tag2 that “it is optionally configured for a serving cell for mDCI mTRP 2TA if coresetPoolIndex for a BWP is configured with more than one value, and for sDCI mTRP 2TA if coresetPoolIndex for a BWP is not configured or coresetPoolIndex is configured with no more than one value”;

- Option 3: use UE capability to distinguish Rel-19 feature, i.e., update the FD of tag2 that “For UE not supporting 2TA for sDCI mTRP, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex for a BWP. For UE supporting 2TA for sDCI mTRP, it is optionally configured in a serving cell for mTRP operation if coresetPoolIndex for a BWP is not configured or configured with only one value.”.

For sDCI mTRP 2TA operation, in the scenario PL offset is configured, as agreed n-TimingAdvanceOffset2 is not needed and only n-TimingAdvanceoffset is configured since there is only one single downlink reference timing; in the scenario PL offset is not configured (i.e., UE is configured with SSB-MTC-additionalPCI), RAN1 has agreed UE maintains two downlink reference timings. However, it is not clear how to configure n-TimingAdvanceoffset or n-TimingAdvanceOffset2.

If we follow Rel-18 mDCI mTRP 2TA operation for inter-cell mTRP (i.e., UE is configured with SSB-MTC-additionalPCI), both n-TimingAdvanceoffset and n-TimingAdvanceOffset2 are configured. RAN2 can assume the same for Rel-19 sDCI mTRP 2TA unless RAN1 informed something different. Alternatively, we can send LS to ask RAN1.

Observation 2: In sDCI mTRP 2TA operation for the scenario PL offset is not configured (UE is configured with SSB-MTC-additionalPCI), UE maintains two downlink reference timings as RAN1 agreed.

Proposal 2: Regarding sDCI mTRP 2TA operation for the scenario PL offset is not configured (UE is configured with SSB-MTC-additionalPCI), discuss how to configure n-TimingAdvanceoffset or n-TimingAdvanceOffset2.

Option 1: RAN2 assumes both n-TimingAdvanceoffset and n-TimingAdvanceOffset2 are configured unless RAN1 has different agreement.

Option 2: send LS to ask RAN1.

## TCI state activation MAC CE for asymmetric DL sTRP/UL mTRP

One open issue is whether/how Rel-18 TCI state activation MAC CEs for sDCI mTRP can be applied for asymmetric DL sTRP/UL mTRP. The difference from Rel-18 sDCI mTRP is that only one joint/DL TCI state, instead of two, is needed in asymmetric DL sTRP/UL mTRP, as RAN1 concluded below.

Conclusion RAN1#117 For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

We see different views on this issue from companies’ contribution.

- Option-1: Rel-18 TCI state activation MAC CEs for sDCI mTRP cannot work for asymmetric DL sTRP/UL mTRP. Send an LS to RAN1 to ask whether the Rel-18 MAC CEs can be applied or enhancement is needed.

- Option-2: The Rel-18 MAC CEs can work for asymmetric DL sTRP/UL mTRP with no enhancement, e.g., NW always activates one joint/DL TCI state for each codepoint (by setting a Fi,j field as 0), or provides same QCL information for the two indicated DL TCI states so that from UE perspective it is DL sTRP operation.

- Option-3: Reuse the Rel-18 MAC CEs, for separate mode the Fi,j field for the DL TCI state for the UL-only TRP is set to 0 and the UE ignores this Fi,j field.

Based on our investigation of the issue, RAN1 has discussed and concluded the existing uTCI signaling can work, as concluded above. In our view, Rel-18 TCI state activation MAC CE for sDCI mTRP can work in couple of ways without enhancement as mentioned in Option 2 and 3.

Note in the current Rel-18 MAC CEs, Fi,j field can be set to 0 to activate only one joint/DL TCI state for a DCI codepoint. It can leave to NW implementation to always set Fi,j to 0 when PL offset is configured. How to use the information in the MAC CEs should be specified or enhanced if needed in RAN1 specification similar as Rel-18. We do not see any need to enhance the MAC CEs.

- Fi,j: This field indicates for the TCI state ID fields associated with the codepoint i of the DCI Transmission Configuration Indication field whether the j-th joint TCI state is present or not, where j=1, 2. If Fi,j field is set to 1, it indicates the j-th joint TCI state for codepoint i is present. If Fi,j field is set to 0, it indicates the j-th joint TCI state for codepoint i is absent. The codepoint to which a TCI state is mapped is determined by its ordinal position among all the TCI state ID fields; - Fi,j: This field indicates for the TCI state ID fields associated with the codepoint i of the DCI Transmission Configuration Indication field whether the j-th DL TCI state is present or not, where j=1, 2. If Fi,j field is set to 1, it indicates the j-th DL TCI state for codepoint i is present. If Fi,j field is set to 0, it indicates the j-th DL TCI state for codepoint i is absent;

Observation 3: The existing Rel-18 TCI state activation MAC CEs for sDCI mTRP already support to always activate one joint/DL TCI state for a DCI codepoint.

Proposal 3: Rel-18 TCI state activation MAC CEs for sDCI mTRP are applied for asymmetric DL sTRP/UL mTRP. NW can always set Fi,j field to 0 to activate only one DL TCI state for a codepoint. No enhancement is needed from RAN2 perspective.

## Co-existence with LTM

Rel-18 mDCI mTRP 2TA is supported in LTM, where the TA included in the LTM cell switch command is applied to the TAG configured for the TCI state indicated in LTM cell switch command. Rel-19 sDCI mTRP 2TA can be applied for LTM in the same way. Since only intra-DU is considered for Rel-19 sDCI mTRP 2TA, we consider to support it only for intra-DU LTM.

Proposal 4: Rel-19 sDCI mTRP 2TA is supported for intra-DU LTM, the TA included in the LTM cell switch MAC CE is applied to the TAG configured for the TCI state indicated in LTM cell switch MAC CE (same as Rel-18 mDCI mTRP 2TA, no spec. impact).

For the scenario that PL offset is configured for asymmetric DL sTRP/UL mTRP, since the PL offset is configured per TCI state, the PL offset configured for the TCI state indicated in the LTM cell switch command can be directly applied for the first PUSCH transmission in RACH-less LTM cell switch for the target cell.

Regarding whether it is possible for NW to pre-configure PL offset in LTM configuration, it is totally up to NW implementation and the scenario of LTM. For example, PL offset can be pre-configured if the UL-only TRP is from the additional PCI in ICBM and the additional PCI is the LTM target cell. For the case PL offset is not configured, we think the current LTM cell switch procedure is applied and there is no need to introduce PL offset in the LTM cell switch MAC CE since UE can always transmit to the anchor DL TRP without applying PL offset. If PL is not pre-configured, it implies there is no need of UL-only TRP or NW may not know the PL offset for the UL-only TRP of the target cell.

Proposal 5: If PL offset is configured in LTM configuration for the target cell, the PL offset configured for the TCI state indicated in the LTM cell switch MAC CE is applied for the first PUSCH transmission in RACH-less LTM for the target cell. No enhancement for the case PL offset is not pre-configured. No spec. impact.

## How to configure/apply PL offset in HO/RRCresume

In the last meeting, we have agreed from RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE. Here we consider details for HO and RRC resume.

As the objective in WID, the scenario of asymmetric DL sTRP/UL mTRP is limited to intra-frequency intra-DU. For HO, if the target and source are from different DUs, since PL offset is per TCI state and the configuration of TCI states is per BWP per cell, the target cell’s PL offset will be anyway different from the source cell. If delta configuration is used in HO, as legacy operation for delta configuration UE set the target cell PL offsets based on the RRC-maintained PL offsets of the source cell, and UE sets the RRC-maintained PL offset of target cell to this latest configured value and apply the PL offset in PHY layer.

In RRC inactive, if the stored PL offsets become out-of-date, NW can reconfigure PL offsets in servingCellConfig in RRC resume message when resume to the same cell or a different cell. UE sets the RRC- maintained PL offset to the latest configured value and apply the PL offset in PHY layer.

Hence, we don’t see any issue or spec. impact regarding UE applies the latest PL offset value received in RRC or MAC CE. UE maintains RRC configured PL offset in RRC and informs to PHY layer, and receives MAC CE updated PL offset (one-shot) and also informs to PHY layer. The latest received PL offset value for a TCI state is applied in PHY layer.

Proposal 6: For the current HO/RRCresume procedure, there is no spec. impact that UE applies the latest PL offset value received in RRC or MAC CE.

# Conclusion

Based on the discussion, we have the following proposals.

Observation 1: RAN1/4 also consider the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), i.e., ICBM. Rel-19 sDCI mTRP 2TA is applied in this scenario.

Proposal 1: 2TA operation is supported for single-DCI mTRP without the restriction that coresetPoolIndex needs to be configured with more than one value, and single-DCI mTRP 2TA is applied to both the scenarios that PL offset is configured (i.e., asymmetric DL sTRP/UL mTRP) and PL offset is not configured (i.e., ICBM).

Proposal 1-1: Discuss how to capture field description for tag2 to keep backward compatibility for Rel-18 mDCI mTRP 2TA and also make the field applicable to Rel-19 sDCI mTRP 2TA.

- Option 1: as a clean solution, introduce a new RRC parameter per BWP that explicitly enables the Rel-19 sDCI-mTRP 2TA, and clarify in FD of tag2 to include all cases where tag2 is configured that “it is optionally configured in a serving cell for mDCI mTRP 2TA if coresetPoolIndex for a BWP is configured with more than one value, and for sDCI mTRP 2TA if [the new parameter is configured].”;

- Option 2: use coresetPoolIndex to implicitly enable Rel-18 mDCI mTRP 2TA or Rel-19 sDCI mTRP 2TA, i.e., update the FD of tag2 that “it is optionally configured for a serving cell for mDCI mTRP 2TA if coresetPoolIndex for a BWP is configured with more than one value, and for sDCI mTRP 2TA if coresetPoolIndex for a BWP is not configured or coresetPoolIndex is configured with no more than one value”;

- Option 3: use UE capability to distinguish Rel-19 feature, i.e., update the FD of tag2 that “For UE not supporting 2TA for sDCI mTRP, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex for a BWP. For UE supporting 2TA for sDCI mTRP, it is optionally configured in a serving cell for mTRP operation if coresetPoolIndex for a BWP is not configured or configured with only one value.”.

Observation 2: In sDCI mTRP 2TA operation for the scenario PL offset is not configured (UE is configured with SSB-MTC-additionalPCI), UE maintains two downlink reference timings as RAN1 agreed.

Proposal 2: Regarding sDCI mTRP 2TA operation for the scenario PL offset is not configured (UE is configured with SSB-MTC-additionalPCI), discuss how to configure n-TimingAdvanceoffset or n-TimingAdvanceOffset2.

Option 1: RAN2 assumes both n-TimingAdvanceoffset and n-TimingAdvanceOffset2 are configured unless RAN1 has different agreement.

Option 2: send LS to ask RAN1.

Observation 3: The existing Rel-18 TCI state activation MAC CEs for sDCI mTRP already support to always activate one joint/DL TCI state for a DCI codepoint.

Proposal 3: Rel-18 TCI state activation MAC CEs for sDCI mTRP are applied for asymmetric DL sTRP/UL mTRP. NW can always set Fi,j field to 0 to activate only one DL TCI state for a codepoint. No enhancement is needed from RAN2 perspective.

Proposal 4: Rel-19 sDCI mTRP 2TA is supported for intra-DU LTM, the TA included in the LTM cell switch MAC CE is applied to the TAG configured for the TCI state indicated in LTM cell switch MAC CE (same as Rel-18 mDCI mTRP 2TA, no spec. impact).

Proposal 5: If PL offset is configured in LTM configuration for the target cell, the PL offset configured for the TCI state indicated in the LTM cell switch MAC CE is applied for the first PUSCH transmission in RACH-less LTM for the target cell. No enhancement for the case PL offset is not pre-configured. No spec. impact.

Proposal 6: For the current HO/RRCresume procedure, there is no spec. impact that UE applies the latest PL offset value received in RRC or MAC CE.

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.

- R2-2504210 MAC open issues for MIMO, Samsung