3GPP TSG-RAN WG2 Meeting #130	R2-2504212

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.12.3

Source:	Samsung

Title:	Open issues on UE-initiated Reporting and RRC parameters

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the open issues on UE-initiated report and some RRC parameters based on the post-129bis email discussion reports.

# Discussion

## UL skipping for mode-A UE-initiated reporting

Regarding UL skipping, it is specified currently that UL skipping is not applied to UCI multiplexed on the PUSCH.

TS 38.321 clause 5.7 … The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

In the post-129bis email discussion, the common understanding is that UL skipping is not applicable to mode-B type-1 CG event-triggered beam report. For UL skipping enhancedSkipUplinkTxDynamic, the UCI for mode-A DG-based UE-initiated report follows the existing procedure (i.e., MAC PDU is generated), there is no MAC impact.

The remaining issue is when skipUplinkTxDynamic is configured, whether to generate MAC PDU for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH. If a UE supports UE-initiated report and only supports skipUplinkTxDynamic (i.e., does not support enhancedSkipUplinkTxDynamic), according to the current procedure the UCI for mode-A UE-initiated report cannot be multiplexed in PUSCH when there is no UL-SCH data to be transmitted. In order to allow UE to transmit UE-initiated report regardless of UL-SCH data, UE should not perform UL skipping (i.e., MAC PDU should be generated) if UEI-initiated report is requested on PUSCH; otherwise, UL skipping should be performed. This is similar to aperiodic CSI report.

Proposal 1: For skipUplinkTxDynamic (enhancedSkipUplinkTxDynamic is not configured), MAC PDU should be generated if there is UE-initiated report requested on the PUSCH regardless of UL-SCH data availability.

## Overlapping rule for mode-A UE-initiated report

One open issue is whether the existing rule in MAC can be applied to handle the overlapping/prioritization between the PUSCH of mode-A report and SR/other PUSCH. We have current rules in MAC procedure to handle the overlapping/prioritization between PUSCH and SR/other PUSCH. For simplicity, the mode-A UE-initiated report in PUSCH can follow the existing rule. This means the UL grant for mode-A UE-initiated report can be deprioritized. Since RAN1 is discussing overlapping rules in PHY, it may have potential impact to RAN1 since the UL grant for UEI report can be deprioritized by MAC entity. It would be good to consider if LS to RAN1 is needed for potential RAN1 impact.

Proposal 2: The existing rule in MAC that handles the overlapping/prioritization between PUSCH and SR/other PUSCH is applied to handle the overlapping/prioritization between the PUSCH of mode-A report and SR/other PUSCH.

Proposal 2-1: Discuss whether LS to RAN1 is needed to inform RAN2 agreement on handling the overlapping/prioritization between the PUSCH of mode-A report and SR/other PUSCH.

## No valid PUCCH/PUSCH resource for UE-initiated report

Another open issue is UE behaviour when UE-initiated report is triggered but there is no valid PUCCH/PUSCH resource to transmit UE-initiated report (e.g., PUCCH resource is released by RRC and type-1 CG is cleared due to TAT expired).

Here we need to consider two cases.

Case-1: a UEI report is triggered for a UEI report configuration and UE has no valid PUCCH resource for this report. This happens when TAT has expired and UE has released PUCCH resources on this serving cell (TAT(s) for STAG(s) expired) or on all serving cells (TAT(s) for PTAG(s) expired).

Case-2: a UEI report is triggered for a UEI report configuration and UE has no PUSCH resource for this report. This happens when TAT is expired after transmitting UCI in PUCCH and before transmitting report in PUSCH and UE has cleared configured uplink grant on this serving cell (TAT(s) for STAG(s) expired) or on all serving cells (TAT(s) for PTAG(s) expired). Note there is no issue for mode-A UE-initiated report since PUSCH is scheduled by DCI and NW can initiate PDCCH-order RACH to recover UL synchronization.

In the current procedure for SR, UE initiates RACH if there is no valid PUCCH resource for pending SR. For UE-initiated report the UCI in PUCCH is similar to SR. Thus, for case-1, the same principle can be applied, meaning UE initiates RACH if there is no valid PUCCH resource for the triggered UE-initiated report. For case-2, it is same to the case that UE has UL data but not UL synchronized. In this case, UE initiates RACH to obtain TA in the legacy operation, which should also be applied for UE-initiated reporting.

Proposal 3-1: When UE-initiated reporting is triggered but UE has no valid PUCCH resource or no valid type-1 CG for this report, UE initiates RACH.

For the actual report transmission, UE can transmit the report in Msg3/MsgA or after Msg3/MsgA. The uplink grant after Msg3/MsgA grant can be the one scheduled by PDCCH addressed to C-RNTI based on which RA procedure is completed, or can be the one scheduled by PDCCH addressed to C-RNTI which is provided after the completion of RA procedure.

Proposal 3-2: When UE-initiated reporting is triggered but UE has no valid PUCCH resource or no valid type-1 CG for this report, discuss how UE transmits the actual report.

- Option 1: in Msg3/MsgA;

- Option 2: using the uplink grant scheduled by PDCCH addressed to C-RNTI based on which RA procedure is completed;

- Option 3: using the uplink grant scheduled by PDCCH addressed to C-RNTI which is provided after the completion of RA procedure.

## Rules for UE-initiated report transmission

In the current MAC specification, whether to transmit CSI report is specified for different cases, e.g., for DRX, for activated/deactivated SCell/SCG, handling of measurement gaps, handling of FR2 UL gaps, BWP operation, cell DRX operation. We need to discuss whether the same rule of CSI report transmission is applied to UEI report.

As currently specified for DRX, aperiodic CSI is transmitted regardless of PDCCH monitoring.

TS 38.321 clause 5.7 … Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

Similarly, upon a UEI report is triggered, UE has to transmit the first PUCCH using the configured periodic PUCCH resource, and transmits the second PUSCH using the DG/CG, which is regardless of whether UE is monitoring PDCCH when UEI report is triggered.

Proposal 4: Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits UE-initiated report on PUCCH and PUSCH.

In the current MAC specification, “CSI report” is used to refer to any P/SP/AP CSI report in activated/deactivated SCell/SCG (clause 5.9, 5.29), handling of measurement gaps (clause 5.14), FR2 UL gaps (clause 5.30), BWP operation (clause 5.15.1), cell DRX operation (clause 5.34.3). We need to discuss whether “UE-initiated report” belongs to CSI report. If yes, “CSI report” is also used to refer to “UE-initiated report” and no MAC spec. change is needed, however, a clarification (e.g., stage-2) is needed. If we consider “UE-initiated report” is a different report than CSI report, then the rule of transmitting UE-initiated report should be specified separately, and the rule for CSI report should be applied.

Proposal 5: Discuss whether “UE-initiated report” belongs to CSI report.

- If yes, “CSI report” is also used to refer to “UE-initiated report” and a clarification (e.g., stage-2) is needed;

- otherwise, specify the rule of transmitting UE-initiated report separately and the rule for CSI report is reused at least in activated/deactivated SCell/SCG (clause 5.9, 5.29), handling of measurement gaps (clause 5.14), FR2 UL gaps (clause 5.30), BWP operation (clause 5.15.1), cell DRX operation (clause 5.34.3).

## Other RRC parameters

- pusch-ResourceOfModeB

| configuredPUSCHResourceForSecondChannelOfModeB-r19 | This parameter is used to inform Type-1 CG PUSCH resource for second channel in mode-B | configuredGrantConfigIndex, UL bwp-Id, serving cell index |
| --- | --- | --- |

pusch-ResourceOfModeB-r19       SEQUENCE {

configuredGrantConfigIndex-r19                ConfiguredGrantConfigIndex-r16                            OPTIONAL,    -- Need R

ul-BWP-Id-r19                                BWP-Id,

servCellIndex-r19							 ServCellIndex

}                                                                                                          OPTIONAL,    -- Need R

Based on the following RAN1 agreement, PUSCH for mode-B should be a type-1 CG in the same serving cell where the report configuration is configured. Thus, the ServCellIndex is not needed and a restriction should be added to the configuredGrantConfigIndex-r19.

Agreement RAN1#118bis On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. - Note: Above is applied to the case that second UL channel is PUSCH. - FFS: Whether the first and second channels can be from the same/different CC.

Proposal 6: For pusch-ResourceOfModeB, the ServCellIndex is not needed, clarify for the configuredGrantConfigIndex that it indicates a type-1 CG in the serving cell where the report configuration is configured.

- additionalOneSlotOffsetDoppler

| CSI-Type-I/II | NZP-CSI-RS-ResourceSet | additionalOneSlotOffsetDoppler | additionalOneSlotOffset Dopp | For Rel-19 Type-II based on Rel-18 Type-II Doppler: 1-slot offset (per NZP-CSI-RS-Resource Group) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet | {enabled} |
| --- | --- | --- | --- | --- | --- |
| CSI-Type-I/II | NZP-CSI-RS-ResourceSet | kdopp-r19 | Kdopp | The number of configured resource groups, each resource group comprises K NZP CSI-RS resources, where K={2,3,4} | 4, 8, 12 |

In the current running CR, additionalOneSlotOffsetDoppler has only one-bit enabled (not per resource group).

kdopp-r19                           SEQUENCE {

numberOfResourceGroups-r19           ENUMERATED {n4, n8, n12},

numberOfResourcesPerGroup-r19        ENUMERATED {n2, n3, n4}

}                                                                                                      OPTIONAL,   -- Need R

additionalOneSlotOffsetDoppler      ENUMERATED{enabled}                                                    OPTIONAL   -- Need R

However, as RAN1 informed the one-bit enabled is per NZP-CSI-RS-Resource Group, and the number of configured resource groups is {4,8,12}. Thus, the additionalOneSlotOffsetDoppler should include a sequence of {enabled}, where the sequence size is configured by numberOfResourceGroups-r19, or a bitmap of length equal to the value of numberOfResourceGroups-r19.

Proposal 7: Discuss how to configure additionalOneSlotOffsetDoppler per NZP-CSI-RS-Resource Group.

- Option 1:  a sequence of {enabled}, where the sequence size is configured by numberOfResourceGroups-r19.

- Option 2: a bitmap of length equal to the value of numberOfResourceGroups-r19.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: For skipUplinkTxDynamic (enhancedSkipUplinkTxDynamic is not configured), MAC PDU should be generated if there is UE-initiated report requested on the PUSCH regardless of UL-SCH data availability.

Proposal 2: The existing rule in MAC that handles the overlapping/prioritization between PUSCH and SR/other PUSCH is applied to handle the overlapping/prioritization between the PUSCH of mode-A report and SR/other PUSCH.

Proposal 2-1: Discuss whether LS to RAN1 is needed to inform RAN2 agreement on handling the overlapping/prioritization between the PUSCH of mode-A report and SR/other PUSCH.

Proposal 3-1: When UE-initiated reporting is triggered but UE has no valid PUCCH resource or no valid type-1 CG for this report, UE initiates RACH.

Proposal 3-2: When UE-initiated reporting is triggered but UE has no valid PUCCH resource or no valid type-1 CG for this report, discuss how UE transmits the actual report.

- Option 1: in Msg3/MsgA;

- Option 2: using the uplink grant scheduled by PDCCH addressed to C-RNTI based on which RA procedure is completed;

- Option 3: using the uplink grant scheduled by PDCCH addressed to C-RNTI which is provided after the completion of RA procedure.

Proposal 4: Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits UE-initiated report on PUCCH and PUSCH.

Proposal 5: Discuss whether “UE-initiated report” belongs to CSI report.

- If yes, “CSI report” is also used to refer to “UE-initiated report” and a clarification (e.g., stage-2) is needed;

- otherwise, specify the rule of transmitting UE-initiated report separately and the rule for CSI report is reused at least in activated/deactivated SCell/SCG (clause 5.9, 5.29), handling of measurement gaps (clause 5.14), FR2 UL gaps (clause 5.30), BWP operation (clause 5.15.1), cell DRX operation (clause 5.34.3).

Proposal 6: For pusch-ResourceOfModeB, the ServCellIndex is not needed, clarify for the configuredGrantConfigIndex that it indicates a type-1 CG in the serving cell where the report configuration is configured.

Proposal 7: Discuss how to configure additionalOneSlotOffsetDoppler per NZP-CSI-RS-Resource Group.

- Option 1:  a sequence of {enabled}, where the sequence size is configured by numberOfResourceGroups-r19.

- Option 2: a bitmap of length equal to the value of numberOfResourceGroups-r19.

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.