3GPP TSG-RAN2 Meeting #130	R2-2504221

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.2.3

Source:	Qualcomm Incorporated

Title:	Views on Random Access Aspects of Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

RAN2 #129 made the following agreements regarding the A-IoT random access.

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

RAN2 #129bis made the following agreements regarding the A-IoT random access.

- Agreements on msg1

      - In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

      - RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Agreements on msg 2

- A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

- Same Msg2 format is used for initial transmission and retransmission of Msg2.

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

In this paper, we will discuss the random-access aspects of Ambient IoT (A-IoT).

# Discussion

## Contention-Based A-IoT Random Access

During the A-IoT MAC spec running CR review [2], questions were raised on what should be the name of the messages? Should they be Msg1, Msg2 etc. or Random ID Message, Random ID Response Message etc.

In our view, Msg1 and Msg2 are quite commonly understood terminologies in the Random Access procedure, not only in A-IoT context, but also in NR and LTE. So, we think it is ok to keep the names A-IoT Msg1 and A-IoT Msg2 as currently captured in the Stage-2 spec CR. There is no need to call them with more restrictive names like ‘Random ID Message’.

It is fine to call the MAC CE that carries Random ID as ‘Random ID MAC CE’. Then A-IoT Msg1 contains Random ID MAC CE in addition to any other L1 items based on RAN1 agreements. Similarly, the response message can be called A-IoT Msg2.

Proposal 1: Use A-IoT Msg1 and A-IoT Msg2 as message names in the CBRA procedure.

In addition, Rapporteur suggested using ‘Access Occasion Trigger message’, while some other companies suggested to use ‘R2D trigger message’. We prefer to call it ‘R2D trigger message’ since that is easier to understand and has been used in RAN2 agreements before.

Proposal 2: For the new R2D message indicating the start of a set of MSG1 resources that were configured in the paging message, keep the name ‘R2D trigger message’ (i.e. no need to change to ‘Access Occasion Trigger message’).

RAN2 #129 has agreed that 3-step contention-based A-IoT random access is supported in Rel-19. In contention-based A-IoT random access, the A-IoT device randomly selects one frequency and/or time access occasion from the D2R scheduling information configured by the A-IoT paging message.

RAN2 #129bis further agreed that a new R2D message other than the paging message is introduced for A-IoT device to determine A-IoT Msg1 resources and the R2D message indicates the start of a set of A-IoT Msg1 resources that were configured in paging message.

In our view, only the paging message should provide the A-IoT access resource (i.e., D2R scheduling information), i.e. the D2R scheduling information should not be configured by the new R2D message (or subsequently referred as R2D trigger message). The R2D trigger message is only used to indicate the start point of the A-IoT access occasion. Thus, the A-IoT resource selection on the access occasion is not based on and is not impacted by the R2D trigger message.

Proposal 3: The A-IoT device randomly selects the access occasion from the A-IoT access resources configured by the A-IoT paging message. The resource selection is not based on the R2D trigger message.

Thus, no D2R scheduling information is included in the R2D trigger message but only the message type is in the R2D trigger message.

Proposal 4: No D2R scheduling information is included in the R2D trigger message.

Proposal 5: Only the message type is included in the R2D trigger message.

After A-IoT device receives the A-IoT paging message with the D2R scheduling information, the A-IoT device can already have the information on the first set of A-IoT Msg1 access occasions. There is no need to have additional R2D trigger message to indicate the start of the first set of access occasion. Only the subsequent sets of A-IoT Msg1 resource configured by A-IoT paging message needs the indication by the R2D trigger message.

Proposal 6: The first set of A-IoT access occasions for Msg1 can be directly indicated from the A-IoT access resource configuration in the A-IoT paging message. The R2D trigger message indicates the start of the subsequent set(s) of A-IoT Msg1 access occasion(s).

The A-IoT device should monitor the access response from A-IoT Msg2 after random ID transmission in the contention-based A-IoT random access. If no echoed random ID is received in A-IoT Msg2 or no A-IoT Msg2 is received before the next R2D trigger message or the subsequent A-IoT paging message, A-IoT device should consider the contention resolution as failed. The A-IoT device with contention resolution failure should perform A-IoT re-access upon receiving the next A-IoT paging message.

Proposal 7: If no echoed random ID is received in A-IoT Msg2 or A-IoT Msg2 is not received, A-IoT device considers the contention resolution as failed and performs A-IoT re-access upon receiving next A-IoT paging message.

Another question to be answered in the context of discussion of A-IoT CBRA is the TB size of the D2R grant for A-IoT Msg3 (i.e. the grant indicated by A-IoT Msg2). One can argue that the TB size is indicated by the Reader therefore can be any size as defined by RAN1. However, practically speaking, in a given deployment, the Msg3 size from different A-IoT devices performing CBRA is likely to be the same. That is because for a given deployment, the A-IoT device ID, at least among the devices that are sent A-IoT paging in one round, are likely to have the same length Device ID. Even when devices of different ID lengths are deployed, Reader can by implementation select the devices of same ID length for a round of inventory. Therefore, we propose the following:

Proposal 8: During a CBRA procedure, the TB size is the same for all A-IoT Msg3 access occasions indicated by a single A-IoT Msg2 (i.e. A-IoT Msg2 can include a single TB size indication even for the case when contention is resolved for multiple devices by a single Msg2).

## Contention-Free A-IoT Access

For contention-free A-IoT access, the scheduled A-IoT device should be configured with a dedicated D2R resource. From A-IoT device perspective, if the resource is dedicatedly configured or the resource configured can be one-to-one mapped to the device, the A-IoT device selects the indicated D2R resource and performs contention-free A-IoT access. When A-IoT paging indicates a device specific identifier and the paging message includes an A-IoT access resource (i.e., D2R resource) for that device, contention-free A-IoT access is performed.

Since the access resource is dedicatedly configured by Reader for the target A-IoT device, the A-IoT Msg1 resource is unique to the target A-IoT device. There is no use case for R2D trigger message to indicate the start of A-IoT Msg1 resource in contention-free case. Thus, we propose that the R2D trigger message is not needed for contention-free A-IoT access.

Proposal 9: The R2D trigger message for CFRA is not supported.

For contention-free A-IoT access, if Reader does not receive the first D2R message from the target A-IoT device after sending the A-IoT paging message, Reader can re-send the paging message again to request the target A-IoT device to respond again. From A-IoT device perspective, the device simply monitors and responds to any R2D message (e.g., either a R2D data message including command or the re-sent A-IoT paging message). There is no need for special handling for the ‘re-access’ of contention-free A-IoT access.

Proposal 10: The ‘re-access’ of contention-free A-IoT access is achieved by Reader sending the paging message again to the target A-IoT device.

For the multiple devices contention-free A-IoT access, it can be achieved by Reader implementation to trigger the A-IoT devices in sequence in Rel-19. The A-IoT paging message and AS ID assignment design can consider the forward compatibility to support multiple devices contention-free case in future release.

Proposal 11: In Rel-19, for multiple devices contention-free A-IoT access, it can be achieved by Reader implementation to trigger each A-IoT device in sequence.

## A-IoT Msg2

In the A-IoT SI phase, it has been studied that the Reader can use an optional explicit R2D failure/success feedback indication to trigger the re-access. The R2D failure indication can be used to address the A-IoT Msg3 transmission failure. Given that the A-IoT Msg3 is scheduled by A-IoT Msg2, Reader can retransmit the A-IoT Msg2 including the random ID to trigger A-IoT Msg3 retransmission. The A-IoT device associated with the same random ID may consider the previous A-IoT Msg3 was unsuccessfully (re)transmitted and should respond to the A-IoT Msg2 with the A-IoT Msg3 retransmission.

Proposal 12: Reader can trigger A-IoT Msg3 retransmission by retransmitting A-IoT Msg2 with the previously used ID (either the random ID or AS ID, if assigned in A-IoT Msg2).

Proposal 13: The A-IoT device should retransmit the A-IoT Msg3 if the A-IoT Msg2 including the random ID or AS ID is received again.

In contention-based A-IoT random access, it is possible that multiple devices using the different A-IoT access resources generate the same random ID. If Reader responds to A-IoT device with the echoed random ID only (i.e., no additional information related to the used A-IoT resource), it might cause ambiguity to those A-IoT devices and the A-IoT device may misunderstood the response from Reader.

In A-IoT SI phase, a similar issue was discussed and RAN2 has drawn the following conclusion.

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

In A-IoT WI phase, the fixed 16-bit random ID was agreed. That means RAN2 confirmed the 16-bit fixed size is sufficient for the contention resolution. If there are concerns raised about new cases, RAN2 should first revisit the decision on whether 16-bit fixed size is sufficient for all cases instead of considering additional information in A-IoT Msg2 which may introduce unnecessary complexity.

Proposal 14: A-IoT Msg2 does not include additional information other than the echoed random ID to address the contention resolution of A-IoT devices.

## NACK feedback indication

In RAN2#129bis, there were discussions on whether the NACK feedback indication can be received before the R2D trigger message or the paging message. No matter whether the NACK feedback indication is received before a subsequent R2D message, RAN2 should first confirm that the A-IoT re-access due to the reception of NACK feedback indication can only be triggered by the next paging message. The R2D trigger message is not used for A-IoT re-access. If the R2D trigger message is used for A-IoT re-access, it may cause collision with those A-IoT devices using the R2D trigger message for initial access.

Proposal 15: The R2D trigger message is not used for A-IoT re-access. Only the subsequent A-IoT paging message is used for re-access.

It is possible that the NACK feedback indication and/or other R2D message like A-IoT Msg2 are missed by the A-IoT device(s) when the R2D link is poor. Then, it can cause ambiguity between Reader and A-IoT device, e.g., Reader requires A-IoT device to re-access by sending NACK feedback indication. However, A-IoT device assumes random access is successful due to failed reception of NACK indication. Thus, an explicit ACK indication should be introduced to indicate the end of the whole procedure for the particular A-IoT device. Only if A-IoT device receives the ACK indication, the random access is determined as successful. Otherwise (if the ACK is missing), A-IoT device should perform re-access in the next A-IoT paging message.

Proposal 16: An explicit ACK indication is introduced to indicate the end of the whole procedure for the particular A-IoT device.

Regarding the NACK feedback indication, it is good for A-IoT device to receive the NACK feedback indication before the next R2D trigger message instead of waiting for the next A-IoT paging message. When A-IoT device has transmitted the random ID message and expects to receive the echoed random ID in A-IoT Msg2 and further transmits the A-IoT data including the device ID. It is straightforward for Reader to directly inform the A-IoT Msg3 failed device by NACK feedback indication before the next R2D trigger message.

Proposal 17: NACK feedback indication can be sent before the next R2D trigger message.

Similar to A-IoT Msg2, the NACK indication message can include ID information for more than one failed A-IoT device(s).

Proposal 18: The NACK indication can include ID information of multiple failed A-IoT device(s).

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: Use A-IoT Msg1 and A-IoT Msg2 as message names in the CBRA procedure.

Proposal 2: For the new R2D message indicating the start of a set of MSG1 resources that were configured in the paging message, keep the name ‘R2D trigger message’ (i.e. no need to change to ‘Access Occasion Trigger message’).

Proposal 3: The A-IoT device randomly selects the access occasion from the A-IoT access resources configured by the A-IoT paging message. The resource selection is not based on the R2D trigger message.

Proposal 4: No D2R scheduling information is included in the R2D trigger message.

Proposal 5: Only the message type is included in the R2D trigger message.

Proposal 6: The first set of A-IoT access occasions for Msg1 can be directly indicated from the A-IoT access resource configuration in the A-IoT paging message. The R2D trigger message indicates the start of the subsequent set(s) of A-IoT Msg1 access occasion(s).

Proposal 7: If no echoed random ID is received in A-IoT Msg2 or A-IoT Msg2 is not received, A-IoT device considers the contention resolution as failed and performs A-IoT re-access upon receiving next A-IoT paging message.

Proposal 8: During a CBRA procedure, the TB size is the same for all A-IoT Msg3 access occasions indicated by a single A-IoT Msg2 (i.e. A-IoT Msg2 can include a single TB size indication even for the case when contention is resolved for multiple devices by a single Msg2).

Proposal 9: The R2D trigger message for CFRA is not supported.

Proposal 10: The ‘re-access’ of contention-free A-IoT access is achieved by Reader sending the paging message again to the target A-IoT device.

Proposal 11: In Rel-19, for multiple devices contention-free A-IoT access, it can be achieved by Reader implementation to trigger each A-IoT device in sequence.

Proposal 12: Reader can trigger A-IoT Msg3 retransmission by retransmitting A-IoT Msg2 with the previously used ID (either the random ID or AS ID, if assigned in A-IoT Msg2).

Proposal 13: The A-IoT device should retransmit the A-IoT Msg3 if the A-IoT Msg2 including the random ID or AS ID is received again.

Proposal 14: A-IoT Msg2 does not include additional information other than the echoed random ID to address the contention resolution of A-IoT devices.

Proposal 15: The R2D trigger message is not used for A-IoT re-access. Only the subsequent A-IoT paging message is used for re-access.

Proposal 16: An explicit ACK indication is introduced to indicate the end of the whole procedure for the particular A-IoT device.

Proposal 17: NACK feedback indication can be sent before the next R2D trigger message.

Proposal 18: The NACK indication can include ID information of multiple failed A-IoT device(s).

# References

- RP-250796, Solutions for Ambient IoT (Internet of Things) in NR.

- R2-2503997, MAC running CR for Ambient IoT, Huawei.