3GPP TSG-RAN WG2 Meeting #130	R2-2504224

St.Julians, Malta, 19th – 23rd May 2025

Agenda item:	8.10.2

Source:	Samsung

Title:	Remaining issues for MRO enhancements

Document for:	Discussion and Decision

# Introduction

As per [1], one of the objectives of R19 SON WI is to support the MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs and subsequent CPAC. From RAN2, this involves identification and specification of necessary UE reporting to enhance the mobility parameter tuning as given below. We discuss the remaining issues for MRO for LTM and CHO with candidate SCGs in this contribution.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for LTM

Open issue in LTE RRC CR

In 36.331, rapporteur has captured an open issue regarding the LTM based recovery as below.

=>Proposal 2: It is proposed RAN2 to discuss the open issue LTERRC-2, i.e. when the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection need to refer to the first failure and this should be captured in running 36.331 CR.

When the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection is updated in NR RLF report. RAN2 has already agreed to reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures. So we think it is reasonable that  timeUntilReconnection refers to the first failure, and this should be captured in 36.331 CR.

Proposal 1: When the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection refers to the first failure. This need to be captured in running 36.331 CR.

Issue with logging L1 measurements for LTM in RRC CR

LTM may be based on L3 measurements or L1 measurements. When the LTM is based on L3 measurements, LTM-SSB-Config-r18 in the LTM-Candidate-r18 will not be configured, and UE will not perform L1 measurements for the LTM, based on this configuration. However the L1 measurements which are used for generating L3 measurements will still be available at the UE.

As UE already reports beam level measurements (L3 filtered) in the current RLF report, L1 measurements for LTM need to be reported only when the network wants to trigger to LTM based on L1 measurements. There is no need/benefit for reporting L1 measurement results when the network configures LTM based on L3 measurements.

Observation 1: L3 filtered beam measurements are included for all the candidates in RLF report and SHR.

In the current running CR, UE reports the L1 measurement results based on the SS/PBCH block-based L1 measurement quantities.

1> if the UE supports RLF-Report for LTM and UE is LTM configured and the SS/PBCH block-based L1 measurement quantities are available: 2> set the resultsSSB-Indexes in measResultL1LastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block L1-RSRP is listed first, based on the available SS/PBCH block-based L1 measurements collected up to the moment the UE detected failure;

It seems the intention is to report the L1 measurements if the SS/PBCH block based configuration is present in quantity configuration. However, the quantity configurations are also associated to L3 filters (and are already reported) based on the definition of QuantityConfig in TS 38.331. In the ltm-Candidate configuration, UE will be configured for reporting L1 measurements based on the presence of LTM-SSB-Config-r18. So the reporting also may be performed accordingly.

Proposal 2: L1 measurements for LTM are logged only for the candidates for which LTM-SSB-Config-r18 is configured (i.e for LTM based on L1 measurements).

MRO for SCG LTM

In RAN2#126, during the initial discussions for Rel-19 SON/MDT, RAN2 decided to focus on MCG LTM first, and made several agreements in the subsequent meetings. We think that MCG LTM is sufficiently stable now and we can include some of the agreed information for SCG LTM also. We note there was wide consensus on reporting L1 measurements for LTM in RLF report/SHR and it is the most critical information for the network to do any optimisation. So it makes sense to include the same in SCGFailureInformation/SPR also.

Proposal 3: L1 measurements for LTM candidates can be reported in SCGFailureInformation/SPR.

Capability for LTM

Considering that the LTM is an entirely different mobility procedure than the L3 mobility and includes additional information such as the L1 measurements which may need to be processed by different network entities such as DU, it would be beneficial to define separate capabilities for the reports for LTM and inform them to the network. It makes sense to have separate capabilities for RLF and SHR as well. The capabilities need to be reported to the network as it may be required for the configurations from the network. For e.g. if the UE supports SHR for LTM, the network may include the thresholds in LTM candidate configuration or the LTM reference configuration.

Proposal 4: Define separate UE capability parameters with signalling for RLF report and SHR for MRO of LTM cell switch.

## MRO for CHO with Candidate SCG(s)

Open issue in NR RRC CR

In the RRC CR, there is an open issue as below:

Editor’s Note: FFS whether it is needed to avoid duplication of information in case of two reports being generated CHO with candidate SCGs, any redundancy (e.g., measurements) are recorded in the reports for PCell (i.e., in SHR, SPR).

We observe that there are no new measurements logged for CHO with Candidate SCG(s) in SHR/SPR. The only new change in the running CR is moving existing serving PCell/PSCell measurements to a new structure.

Observation 2: There is no new measurement added in SHR or SPR due to CHO with candidate SCG(s).

There may be some duplication of time information or some additional flags (which is first triggered event etc.). It may not be beneficial to have added UE behaviour for avoiding this duplication considering that the overhead is minimal and the occurrence of SHR/SPR at the same time is a corner case (more so for CHO with candidate SCG(s) as it depends on A4 event and hence the target PSCell’s measurements only). We also note that RAN3 decided to not consider near failure scenarios. So we prefer not to do further optimisation for this case for now.

Proposal 5: No special handling is needed to avoid duplication of information in SHR and SPR.

RLF due to the CHO and associated CPAC conditions cannot be met simultaneously.

For CHO with candidate SCG(s), network may configure multiple execution condition for CHO and CPAC.

So there can be cases where several execution conditionals for CHO and CPC are met but the associated candidate PCell and PSCell cannot be met at the same time (case 9 in the RAN3 agreed use case for CHO with candidate SCG(s)), as below.

For e.g. CHO condition, CHO1 is met initially, and later a CPA/CPC condition is met, but at that moment CHO1 is not met. So the UE doesn’t perform CHO with candidate SCG. Later another CHO condition CHO2 can be met, but the associated CPA/CPC may not be met and the UE doesn’t perform CHO with candidate SCG. In another case CHO1 may be met at a later point of time, but the CPAC condition may not be met at this point of time. An RLF can occur in all these cases.

RAN2 has agreed to log the type of the first fulfilled condition and the time elapsed between fulfilled conditions. Thus UE may report that CHO is fulfilled in the above example. But when CHO1 is fulfilled first and CHO2 is fulfilled thereafter, the network can’t understand which one is the fulfilled condition. Similarly the time elapsed also doesn’t provide much helpful information as none of the CHO conditions were fulfilled when CPAC is fulfilled. This may even give a wrong direction to the network, as CHO1 condition was not fulfilled when CPAC is fulfilled, and network may assume that CHO1 is fulfilled based on the first fulfilled condition and the time information.

For the network to optimise this scenario, UE need to provide the list of fulfilled execution conditions before the RLF is encountered and the time elapsed between each of the fulfilled execution conditions and RLF.

Proposal 6: For CHO with candidate SCG(s), to optimise RLF due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously, UE logs the following in RLF report:

a)  List of fulfilled execution conditions before the RLF is encountered.

b) Time elapsed between each of the fulfilled execution conditions and RLF.

# Conclusion

Based on the discussion, we have the following observation and proposals.

MRO for LTM

Open issue in LTE RRC CR

Proposal 1: When the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection refers to the first failure. This need to be captured in running 36.331 CR.

Issue with logging L1 measurements for LTM in RRC CR

Observation 1: L3 filtered beam measurements are included for all the candidates in RLF report and SHR.

Proposal 2: L1 measurements for LTM are logged only for the candidates for which LTM-SSB-Config-r18 is configured.

MRO for SCG LTM

Proposal 3: L1 measurements for LTM candidates can be reported in SCGFailureInformation/SPR.

Capability for LTM

Proposal 4: Define separate UE capability parameters with signalling for RLF report and SHR for MRO of LTM cell switch.

MRO for CHO with Candidate SCG(s)

Open issue in NR RRC CR

Observation 2: There is no new measurement added in SHR or SPR due to CHO with candidate SCG(s).

Proposal 5: No special handling is needed to avoid duplication of information in SHR and SPR.

RLF due to the CHO and associated CPAC conditions cannot be met simultaneously.

Proposal 6: For CHO with candidate SCG(s), to optimise RLF due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously, UE logs the following in RLF report:

a)  List of fulfilled execution conditions before the RLF is encountered.

b) Time elapsed between each of the fulfilled execution conditions and RLF.

# References

[1] RP-234038: New WID on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4