3GPP TSG-RAN WG2 Meeting #130	R2-2504237

St Julian's, Malta, 19 – 23 May, 2025

Title: 	Discussion on SONMDT for others

Source: 	Huawei, HiSilicon

Agenda item:	8.10.3

Document for:	Discussion and Decision

- Introduction

In this paper, we mainly analyze possible RAN2 impacts in terms of SON/MDT for intra-NTN mobility and Network slicing

- Discussion

Other issues on SON/MDT for intra-NTN mobility

There is an incoming LS [1] on SON/MDT for NTN from RAN3 as listed below:

RAN3 discussed the MRO enhancement for location-based CHO in NTN for earth moving scenario. In order to optimize the distance threshold for location-based CHO, RAN3 thinks it is beneficial to support either option provided below. - Option 1: to introduce the absolute UTC time when RLF/HOF happens, based on which the network computes the distance. - Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens. RAN3 would like to check with RAN2 which option to be adopted in the UE RLF report.

In our understanding, if the goal is to optimize the distance threshold for serving cell and each candidate cell, it is better for UE to straightforward report the measure distance when RLF/HOF happens, which assist the network to make adjustment to the maximum extent in an efficient manner. Some companies have concerned about that the UE is not mandated to keep calculating the distance with the target(candidate) cell after the location-based trigger is fulfilled in NTN mobility. However, it is no big issue since the UE always has a valid GNSS position in RRC connected, and is totally capable of calculating the distance with any candidate cell for SON purpose regardless of whether location-based CHO is triggered/executed or not.

Observation 1: The UE always has a valid GNSS position in RRC connected, and is totally capable of calculating the distance with any candidate cell for SON purpose regardless of whether location-based CHO is triggered/executed or not.

Proposal 1: It is proposed RAN2 to agree on introducing the distance from UE to serving cell and candidate cell.

Other issues on MDT enhancements for network slicing

There is an incoming LS [2] on SON/MDT for network slicing from RAN3 as listed below:

RAN3 has discussed how to optimize slice-based cell reselection via e.g. logged MDT and identified the following scenario: - A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell. RAN3 would like to ask RAN2 to take the above scenario into account, and to discuss whether and how it can be addressed and whether any standard changes are needed.

As for slice-based cell reselection enhancement, RAN3 has identified two use case as follows:

Use case 1: Optimize the mapping of S-NSSAI to NSAG or optimize the frequency priorities

- Sub-case a: Highest ranked cell or best cell support none of the intended NSAGs

- Sub-case b: Highest ranked cell or best cell does not support all of the intended NSAGs

Use case 2: inform the network that UE performs slice-based re-selection instead of general reselection.

We further analyse the use cases to discuss whether any MDT enhancements are needed. According to TS 38.304, in order to perform slice-based cell reselection, the UE need to derive the reselection

priorities by using the following parameters: - NAS provided NSAG information, only for NSAG(s) associated with the network slice(s) provided by NAS for cell reselection - sliceInfoList and/or sliceInfoListDedicated per frequency with nsag-CellReselectionPriority per NSAG, if provided in SIB16 and/or in RRCRelease message - cellReselectionPriority per frequency provided in SIB16 and/or in RRCRelease message.

Besides, with the aforementioned parameters, the UE performs the cell (re)selection according to the following rules:

TS 38.304: The UE shall derive reselection priorities for slice-based cell reselection according to the following rules: - Frequencies that support at least one prioritized NSAG received from NAS have higher reselection priority than frequencies that support none of the NSAG(s) received from NAS. - Frequencies that support at least one NSAG provided by NAS are prioritised in the order of the NAS-provided priority for the NSAG with highest priority supported on the frequency. - Among the frequencies (one or multiple) that support the highest prioritised NSAG(s) with the same NAS-provided priorities, the frequencies are prioritized in the order of their highest nsag-CellReselectionPriority given for these NSAG(s). If no nsag-CellReselectionPriority is given for a NSAG at a frequency, the lowest priority value is used (i.e, lower than any of the network configured values for these frequencies). - Frequencies that support none of the NSAG(s) provided by NAS are prioritized in the order of their cellReselectionPriority. For a UE performing slice-based cell reselection, if the highest ranked cell or best cell in a frequency fulfils the inter- freqeuency cell reselection criteria (see clause 5.2.4.5) based on reselection priority for the frequency and NSAG derived according to this clause or fulfils intra-frequency and equal priority inter-frequency cell reselection criteria (see clause 5.2.4.6), but this cell does not support the NSAG according to this clause: - if this cell supports any other NSAG(s) according to this clause, the UE shall re-derive a reselection priority for the frequency by considering the NSAG(s) supported by this cell (rather than those of the corresponding NR frequency); - Otherwise, the UE shall re-derive a reselection priority for the frequency as if none of the NSAG(s) provided by NAS is supported.

For sub-cases a and b, based on the NAS provided NSAG information and slice Info List in the SIB message/RRC release message, if the UE cannot camp on a cell supporting any or all of the intended NSAGs, it means the effect of slice-based cell reselection is compromised. In this case, we think the network parameters may need to be adjusted, and the UE can make the network aware of the issue so that the UE to find the correct cell during the slice-based cell (re)selection procedure.

Observation 2: The UE may not camp on a cell supporting any or all of the intended NSAGs, and the network parameters may need to be adjusted.

Observation 3: To address the issue mentioned in observation 1, the UE can make the network aware of the issue so that the UE to find the correct cell during the slice-based cell (re)selection procedure.

The logged MDT feature involves measurement logging by UE in Idle and Inactive state, so it can be used for slicing purpose, i.e. the UE could make the network aware of the NSAG information from the NAS via logged MDT. Currently, the NW can configure reportType in LoggedMeasurementConfiguration, which includes periodical and eventTriggered types. In our opinion, the following report type can be addended:

- highest ranked cell or best cell does not support any or all of the intended NSAGs

When the UE triggers the slicing specific report type, we think the UE can also include the NSAG information received from NAS.

Proposal 2: It is proposed RAN2 to agree on introducing the following report type in logged MDT configuration:

- highest ranked cell or best cell does not support any or all of the intended NSAGs

Proposal 3: It is proposed RAN2 to agree on including the NSAG information received from NAS in logged MDT report.

For use case 2, if the UE is unable to find suitable cell in the frequency corresponding to the prioritized NSAG, UE falls back to legacy cell reselection procedure, i.e., without considering the NSAG(s) and their priorities. It delays the cell (re)selection process and consumes the UE battery. Therefore, we propose:

Proposal 4: It is proposed RAN2 to discuss whether to address the case where the UE fall backs to legacy cell reselection procedure from slice-based cell reselection.

Proposal 5: It is proposed RAN2 to discuss possible solutions, including:

- addition of a new report type related to P3 in logged MDT configuration

- addition of a fallback indicator in logged MDT report

Other issues on MHI for SCG deactivation

Since RAN3 agreed on a network based solution and is pending on RAN2’s progress on the format of time spent in SCG deactivated/activated state, we suggest RAN2 to adopt the percentage ratio or accumulated time for UE in SCG deactivated state in MHI.

Proposal 6: It is proposed RAN2 to adopt the percentage ratio or accumulated time for UE in SCG deactivated state in MHI.

- Conclusion

Based on the discussions above, we have the following observations and proposals:

Observation 1: The UE always has a valid GNSS position in RRC connected, and is totally capable of calculating the distance with any candidate cell for SON purpose regardless of whether location-based CHO is triggered/executed or not.

Observation 2: The UE may not camp on a cell supporting any or all of the intended NSAGs, and the network parameters may need to be adjusted.

Observation 3: To address the issue mentioned in observation 1, the UE can make the network aware of the issue so that the UE to find the correct cell during the slice-based cell (re)selection procedure.

Proposal 1: It is proposed RAN2 to agree on introducing the distance from UE to serving cell and candidate cell.

Proposal 2: It is proposed RAN2 to agree on introducing the following report type in logged MDT configuration:

- highest ranked cell or best cell does not support any or all of the intended NSAGs

Proposal 3: It is proposed RAN2 to agree on including the NSAG information received from NAS in logged MDT report.

Proposal 4: It is proposed RAN2 to discuss whether to address the case where the UE fall backs to legacy cell reselection procedure from slice-based cell reselection.

Proposal 5: It is proposed RAN2 to discuss possible solutions, including:

- addition of a new report type related to P3 in logged MDT configuration

- addition of a fallback indicator in logged MDT report

Proposal 6: It is proposed RAN2 to adopt the percentage ratio or accumulated time for UE in SCG deactivated state in MHI.

- Reference

- R3-252382, LS on SONMDT for NTN, RAN3

- R3-250914, LS on SON for Network Slicing, RAN3