3GPP TSG RAN WG2 Meeting #130                          	   R2-2504245

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.12.2

Source:	Qualcomm Incorporated

Title:	Discussion on UL only mTRP

WID/SID:	NR_MIMO_Ph5-Core – Release 19

Document for:	Discussion

# Introduction

For asymmetric DL sTRP/UL mTRP deployment scenarios, the updated subject (addition of two TA) was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources.

In this contribution, we discuss the remaining design consideration for supporting 2 TAs when PL offset is not configured.

# Discussion

As shown in Figure 1, a UE may have two TAs where one TA (e.g., TA1) is for the uplink transmission to DL sTRP and the other TA (e.g., TA2) is for the uplink transmission to UL mTRP.

SSB1

SSB2

SSB2

Figure 1. Two TAs for asymmetric DL sTRP and UL mTRP

At RAN1 #118bis, the following agreement was made for 2TA [2].

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

At RAN2 #129bis, the following agreement was made for 2 TAs based on the above RAN1’s agreement [5].

Agreement For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes: - remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value; - a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

However, at RAN1 #119, the following agreement was made for ULmTRP [3].

Agreement The answer to the Question 1 in LS R1-2409353 is: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

This agreement allows ULmTRP to transmit SSB if the PL offset is not configured (i.e., as an implicit indication).

Observation 1. SSB transmission from ULmTRP is supported if PL offset is not configured.

Additionally, at RAN1 #120bis, the following agreements were made for ULmTRP [4].

Agreement Answer the question of RAN4 as follows: - The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). Agreement - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI

Based on the above agreements, ULmTRP may support SSB transmissions on downlink if PL offset is not configured. In this case, each TA of the 2 TAs may have its respective downlink reference (e.g., referenced from the respective SSB transmission as shown in Figure 1), similar to Release 18 multi-TRPs. However, there is only one sTRP for transmitting DCI (i.e., one CORESET for monitoring the DCI).

Observation 2. If SSB transmission from ULmTRP is allowed, each TA of the 2 TAs may have its respective downlink reference.

Observation 3. If SSB transmission from ULmTRP is allowed, there is still one sTRP transmitting DCI.

Based on the above observations, we propose the following proposal for 2 TAs when SSB transmission from mTRP is allowed.

Proposal 1. RAN2 support the follows for 2 TAs when PL offset is not configured, based on Rel-18 2TA configuration:

- remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- keep both single n-TimingAdvanceoffset and n-TimingAdvanceOffset2.

# Conclusion

In this contribution, we discussed the remaining design consideration for supporting 2 TAs when PL offset is not configured and concluded with the following observations and proposals.

Observation 1. SSB transmission from ULmTRP is supported if PL offset is not configured.

Observation 2. If SSB transmission from ULmTRP is allowed, each TA of the 2 TAs may have its respective downlink reference.

Observation 3. If SSB transmission from ULmTRP is allowed, there is still one sTRP transmitting DCI.

Proposal 1. RAN2 support the follows for 2 TAs when PL offset is not configured, based on Rel-18 2TA configuration:

- remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- keep both single n-TimingAdvanceoffset and n-TimingAdvanceOffset2.

# References

- RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- “Chair notes RAN1#118bis eom0” RAN1 #118bis, Hefei, China, October 14th – 18th, 2024

- “Chair notes RAN1 #119 eom0”, RAN1 #119, Orlando, US, November 18th – 22nd, 2024

- “Chair notes RAN1#120bis eom0”, RAN1 #120bis, Wuhan, China, Apr. 7th – 11th, 2025

- “R2_129b R18 MIMO R19 MIMO LPWUS SBFD NR_Others (Erlin) 2025-04-11 eom”, RAN2 #129bis, Wuhan, China, Apr. 7th – 11th, 2025