3GPP TSG-RAN WG2 Meeting #130	R2-2504266

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	6.1.3.3

Source:	Nokia, Huawei, HiSilicon

Title:	Discussion on emergency services and eDRX

WID/SID:	NR_redcap-Core

Document for:	Discussion and Decision

# 1	Introduction

In reply LS R2-2501752 (S2-2502427), SA2 confirmed that RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exists in the RAN node.

SA2 analysed the identified issue above and would like to provide the following feedback:

- Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node.

- In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10.

Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN.

Also, in reply LS R2-2503318 (R3-252402), RAN3 confirms that the NG-RAN is able to be aware of the presence of emergency PDU session when PDU session resources are established in the NG-RAN node based on the received special ARP value from CN.

RAN3 has discussed the scenario raised by RAN2 and checked the response from SA2. According to SA2’s clarification, RAN3 agrees that the NG-RAN is able to be aware of the presence of emergency PDU session when PDU session resources are established in the NG-RAN node based on the received special ARP value from CN. Regarding the open issues, RAN3 has endorsed the attached CRs to TS 38.300.

RAN3 CR R3-252400 mentioned above adds the following note to TS 38.300 Clause 9.2.10:

NOTE:	If emergency PDU session resources are established, the gNB should not configure the Extended DRX when sending the UE to RRC_INACTIVE state. The gNB recognizes the emergency PDU session resources based on special ARP value as specified in TS 23.501 [3].

Both LSs, from SA2 and RAN3, confirm that RAN can be aware of PDU Session for emergency services via ARP values ​​received from CN, which was never considered a problem during discussions in RAN2.

However, both LSs, from SA2 and RAN3 indicate that:

- It is just gNB ability (“can be aware”, “is able to be aware”) of determining existence of PDU Session for emergency services based on existence of U-plane connection (aka UP context, aka UP resources) of an PDU Session for emergency services,

- The potential solution is applicable only “when/if” U-plane connection / PDU Session resources existence condition is fulfilled.

Additionally, in SA2 LS, it is stated that “there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated”.

Due to the above lack of consensus, it should be assumed that there may be scenarios in which U-plane connection / PDU Session resources for emergency services are deactivated or remain not activated even though PDU Session for emergency services between UE and CN still exists (based on TS 23.501 5.6.8 Selective activation and deactivation of UP connection of existing PDU Session mechanism).

Observation 1:	Due to no consensus in SA2 whether the UP of emergency PDU session can be deactivated, it should be assumed that there may be scenarios in which U-plane connection / PDU Session resources for emergency services are deactivated or remains not activated even though PDU Session for emergency services between UE and CN still exists (based on TS 23.501 5.6.8 Selective activation and deactivation of UP connection of existing PDU Session mechanism).

The SA2 LS also questiones the existence of scenarios where the gNB aware of the existence of a PDU Session for emergency services transitions UE to RRC_ILDE or RRC_INACTIVE.

In our paper we discuss:

- scenarios where the gNB is aware of the of a PDU Session for emergency service and transitions the UE to RRC_IDLE or RRC_INACTIVE (Section 2,1).

- Problematic scenarios (Section 2.2) and solution (Section 2.3) to the eDRX configuration problem when UE has a PDU Session for emergency services but gNB is not aware of it.

# 2	Scenarios and solutions

## 2.1	Transition to RRC_IDLE/RRC_INACTIVE

Apart from several abnormal cases (e.g. AS security is not activated while reconfiguration with sync, T311 expiry after RRC re-establishment initiation), there are two ways to trigger the UE going to RRC_IDLE/INACTIVE state.

One way is initiated by the RAN node. The RAN node transmits the RRCRelease message to the UE, indicating that the UE releases or suspends the connection and performs the actions going to the RRC_IDLE/INACTIVE state. The RAN node can decide on whether to release/suspend the connection with the UE based on an inactivity timer (i.e. ue-InactiveTime), which is up to network implementation. This timer is maintained by the RAN node and used to record the duration while the UE has not received or transmitted any user data. The RAN node can also indicate the ue-InactiveTime to the target RAN node in the HandoverPreparationInformation message during the handover preparation stage, to help the target RAN node in managing UE RRC states, as specified in the TS 38.331 clause 11.2.2. When the ue-InactiveTime expires, the RAN node initiates the connection releasing/suspending procedure and transmits the RRCRelease message to the UE, regardless of whether there is an emergency PDU session or not for the UE.

Observation 2:	The RAN node can release the UE to the RRC_IDLE or RRC_INACTIVE state upon the expiry of ue-InactiveTime, regardless of whether the UE has an emergency PDU session.

The other way is that the UE goes to the RRC_IDLE state automatically upon the expiry of DataInactivityTimer. The RAN node configures DataInactivityTimer by RRC message and the UE in RRC_CONNECTED state maintains the timer to control data inactivity operation. The UE starts or restarts this timer when the UE receives or transmits a MAC SDU from any dedicated channels and goes to the RRC_IDLE state upon the timer expiry, as specified in TS 38.321, clause 5.19 and TS 38.331, clause 5.3.8.5.

Observation 3:	The UE goes to the RRC_IDLE state upon the expiry of DataInactivityTimer, regardless of whether the UE has an emergency PDU session.

Therefore, from the RAN2 perspective, the scenario that the UE is released to the RRC_IDLE or RRC_INACTIVE state from the RRC_CONNECTED state when the UE has an emergency PDU session does exist. Based on this consensus, the functionality that eDRX should not be used when the UE in RRC_IDLE or RRC_INACTIVE state has an emergency PDU session makes sense, since the emergency callback may be initiated. SA2 also indicated that there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated. If the UP of emergency PDU session can be deactivated, then gNB is not aware whether the UE has an emergency PDU session.

Proposal 1:	RAN2 confirms and replies SA2 that the RAN node may release the UE to RRC_IDLE or RRC_INACTIVE state from RRC_CONNECTED when the UE has an emergency PDU session.

## 2.2	eDRX configuration by unaware gNB – scenarios

In this section we present several scenarios where gNB is unaware of the existence of PDU Session for emergency serivices and configures the UE with eDRX.

Scenario 1:

In this scenario PDU Session resources for emergency services on Uu and NG-U i/f are released (without release of the PDU Session for emergency services) and after some time CN attempts to establish them back.

Figure 1. Problematic scenario 1 – EM PDU Session resources selectively released

Scenario 2:

In this scenario UE that has a PDU Session for emergency services is in RRC_IDLE and establishes new RRC connection for the purpose of UL non-emergency data transfer, i.e., without indicating presence of emergency data in its buffer during NAS Service Request procedure. Therefore, the PDU Session resources for emergency services are not established and once the non-emergency data inactivity is detected gNB transitions UE to RRC_INACTIVE with eDRX. Later the CN attempts to establish PDU Session resources for emergency services.

Figure 2. Problematic scenario 2 – EM PDU Session resources not established during ICS

Scenario 3:

In this scenario PDU Session resources for emergency services on Uu and NG-U i/f are released (without release of the PDU Session for emergency services).

After some time, due to inactivity on the remaining non-emergency PDU Session the UE is transitioned to RRC_INACTIVE with eDRX.

Later, gNB pages the UE with eDRX but the UE does not receive paging due to lack of coverage (at this stage the usage of eDRX is not a problem).

Due to unsuccessful RAN paging, gNB releases UE context.

After short time, thanks to application-level request retry (e.g., SIP INVITE message is send once again in DL direction), the CN pages the UE with regular non-eDRX paging, however the UE that considers itself to be in RRC_INACTIVE follows the eDRX cycle.

Figure 3. Problematic scenario 3 – UE-gNB RRC state mismatch

Observation 4:	There are scenarios where gNB is not aware that UE has a PDU Session for emergency services and gNB may configure the UE with eDRX.

In our view UE in Scenarios 1-3 follows eDRX, however we understand that the other companies may have a different opinion since TS 23.501 Clause 5.31.7.2.1 is not clear in this matter.

When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure.

We find no reason to believe that the above applies to RAN paging and/or eDRX in RRC_INACTIVE, based on the used terminology ”idle mode”, the fact that the gNB is not being mentioned and that TS 23.501 describes UE reachability in CM-IDLE states in Clause 5.4.1 but when it comes to reachability in CM-CONNECTED / RRC_INACTIVE described in Clause 5.4.2 it refers to RAN specification TS 38.300.

UE RAN reachability management is used by RAN for UEs in RRC_INACTIVE state, see TS 38.300 [27]. The location of a UE in RRC_INACTIVE state is known by the RAN on a RAN Notification area granularity. A UE in RRC_INACTIVE state is paged in cells of the RAN Notification area that is assigned to the UEs. The RAN Notification area can be a subset of cells configured in UE's Registration Area or all cells configured in the UE's Registration Area. UE in RRC_INACTIVE state performs RAN Notification Area Update when entering a cell that is not part of the RAN Notification area that is assigned to the UE.

Observation 5:	It is not clear whether TS 23.501 Clause 5.31.7.2.1 ”When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX.” is applicable to RAN and/or CN eDRX paging in RRC_INACTIVE state.

## 2.2	eDRX configuration by unaware gNB – problem solution

Since, as mentioned above, TS 23.501 Clause 5.31.7.2.1 is not clear and TS 23.501 Clause 5.4.1 refers to RAN specification we propose to add a note to TS 38.304 Clause 7.1 clarifying that in RRC_IDLE and RRC_INACTIVE states, the UE shall operate in DRX, not in eDRX, when the UE has a PDU session associated with emergency services.

The above clarification solves the problems described in the previous section.

Scenario 4 (after CR):

This scenario shows the solution to problematic Scenario 1 and 2 in case NAS-PDU is absent in NGAP signaling.

When the incoming NGAP signaling contains emergency ARP values and NAS-PDU is absent (it cannot be NAS layer establishment of the new PDU Session for emergency services and it must be related to an existing PDU Session for emergency services), gNB follows DRX cycle and the same DRX cycle is followed by the UE.

Figure 4. Solution scenario 4 – EM ARP and NAS-PDU in NGAP signaling

Scenario 5 (after CR):

This scenario shows the solution to problematic Scenario 1 and 2 in case NAS-PDU is present in NGAP signaling.

When incoming NGAP signaling contains emergency ARP values and NAS-PDU is present (it may be NAS layer establishment of the new PDU Session, but it also may be a modification of an existing PDU Session for emergency services), gNB initiates DRX paging.

If UE has a PDU Session for emergency services, it also follows DRX, and the paging is successful. If UE has no PDU Session for emergency services the DRX paging is unsuccessful (or the outcome of the paging procedure is unknown at the time of the next eDRX paging occasion, the gNB initiates eDRX paging. Assuming sufficient coverage, one of those pagings should be successful.

Figure 5. Solution scenario 5 – EM ARP and no NAS-PDU in NGAP signaling

Scenario 6 (after CR):

This scenario shows the solution to problematic Scenario 3. The UE in RRC_INACTIVE that has a PDU Session for emergency services follows DR. Thus, the CN DRX paging is successful.

Figure 6. Solution scenario 6 – UE-gNB RRC state mismatch

Observation 6:	Even without support from SA2, all observed problems can be solved (although not necessarily in an optimal way) if a UE configured with eDRX uses DRX.

Proposal 2:	RAN2 agrees the UE side solution, which does not require any SA2 solution/conclusion: Clarify in TS 38.304 Clause 7.1 that in RRC_IDLE and RRC_INACTIVE states, the UE shall operate in DRX, not in eDRX, when the UE has a PDU session associated with emergency services. We provide a draft TP for Rel-17 TS 38.304 in the Annex.

# 3	Conclusion

This document has made the following observations:

Observation 1:	Due to no consensus in SA2 whether the UP of emergency PDU session can be deactivated, it should be assumed that there may be scenarios in which U-plane connection / PDU Session resources for emergency services are deactivated or remains not activated even though PDU Session for emergency services between UE and CN still exists (based on TS 23.501 5.6.8 Selective activation and deactivation of UP connection of existing PDU Session mechanism).

Observation 2:	The RAN node can release the UE to the RRC_IDLE or RRC_INACTIVE state upon the expiry of ue-InactiveTime, regardless of whether the UE has an emergency PDU session.

Observation 3:	The UE goes to the RRC_IDLE state upon the expiry of DataInactivityTimer, regardless of whether the UE has an emergency PDU session.

Observation 4:	There are scenarios where gNB is not aware that UE has a PDU Session for emergency services and gNB may configure the UE with eDRX.

Observation 5:	It is not clear whether TS 23.501 Clause 5.31.7.2.1 ”When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX.” is applicable to RAN and/or CN eDRX paging in RRC_INACTIVE state.

Observation 6:	Even without support from SA2, all observed problems can be solved (although not necessarily in an optimal way) if a UE configured with eDRX uses DRX.

And proposed the following:

Proposal 1:	RAN2 confirms and replies SA2 that the RAN node may release the UE to RRC_IDLE or RRC_INACTIVE state from RRC_CONNECTED when the UE has an emergency PDU session.

Proposal 2:	RAN2 agrees the UE side solution, which does not require any SA2 solution/conclusion: Clarify in TS 38.304 Clause 7.1 that in RRC_IDLE and RRC_INACTIVE states, the UE shall operate in DRX, not in eDRX, when the UE has a PDU session associated with emergency services. We provide a draft TP for Rel-17 TS 38.304 in the Annex.

# 4	Annex: TP for TS 38.304

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where paging DCI can be sent (TS 38.213 [4]). One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO. A L2 U2N Relay UE monitors the paging occasions of its PC5-RRC connected L2 U2N Remote UEs. In this case, the DRX cycle and UE ID mentioned in this clause refer to those of the L2 U2N Remote UE.

In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN initiated paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS. However, if a L2 U2N Relay UE in RRC_INACTIVE state receives a CN initiated paging for a L2 U2N Remote UE, the L2 U2N Relay UE does not move to RRC_IDLE state.

NOTE 0a:	The L2 U2N Remote UE does not need to monitor the PO in order to receive the paging message.

NOTE 0b:	While the SDT procedure is ongoing in RRC_INACTIVE state, the UE monitors the PO in order to receive only the Short Message as specified in TS 38.331 [3].

In RRC_IDLE and RRC_INACTIVE states, the UE shall operate in DRX, not in eDRX, when the UE has a PDU session associated with emergency services, as specified in clause 5.31.7.2.1 in TS 23.501 [10].

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace as specified in TS 38.213 [4] and firstPDCCH-MonitoringOccasionOfPO and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213 [4].

When SearchSpaceId = 0 is configured for pagingSearchSpace, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts from the first PDCCH monitoring occasion for paging in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF.

When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

NOTE 1:	A PO associated with a PF may start in the PF or after the PF.

NOTE 2:	The PDCCH monitoring occasions for a PO can span multiple radio frames. When SearchSpaceId other than 0 is configured for paging-SearchSpace the PDCCH monitoring occasions for a PO can span multiple periods of the paging search space.

The following parameters are used for the calculation of PF and i_s above:

T: DRX cycle of the UE.

If the UE does not operate in eDRX as defined in clause 7.4:

-	T is determined by the shortest of the UE specific DRX value configured by RRC (if any), the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information. For L2 U2N Relay UE, T for a L2 U2N Remote UE is determined by the shortest of the UE specific DRX value provided in PC5-RRC signalling and a default DRX value broadcast in system information.

In RRC_IDLE state, if the UE operates in eDRX and eDRX is configured by upper layers, i.e., TeDRX, CN, according to clause 7.4:

-	If TeDRX, CN is no longer than 1024 radio frames:

-	T = TeDRX, CN;

-	else:

-	During CN configured PTW, T is determined by the shortest of UE specific DRX value, if configured by upper layers, and the default DRX value broadcast in system information.

In RRC_INACTIVE state, if the UE operates in eDRX and eDRX is configured by RRC, i.e., TeDRX, RAN (if any), and upper layers, i.e., TeDRX, CN, as defined in clause 7.4:

-	If both TeDRX, CN and used TeDRX, RAN are no longer than 1024 radio frames, T = min{TeDRX, RAN, TeDRX, CN}.

-	If TeDRX, CN is no longer than 1024 radio frames and no TeDRX, RAN is configured or used, T is determined by the shortest of UE specific DRX value configured by RRC and TeDRX, CN.

-	If TeDRX, CN is longer than 1024 radio frames:

-	If TeDRX, RAN is not configured or used:

-	During CN configured PTW, T is determined by the shortest of the UE specific DRX value configured by RRC, the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by the UE specific DRX value configured by RRC;

-	else if used TeDRX, RAN is no longer than 1024 radio frames:

-	During CN configured PTW, T is determined by the shortest of the UE specific DRX value, if configured by upper layers and TeDRX, RAN, and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by TeDRX, RAN.

N: number of total paging frames in T

Ns: number of paging occasions for a PF

PF_offset: offset used for PF determination

UE_ID:

If the UE operates in eDRX as specified in clause 7.4:

-	5G-S-TMSI mod 4096

else:

-	5G-S-TMSI mod 1024

Parameters Ns, nAndPagingFrameOffset, nrofPDCCH-MonitoringOccasionPerSSB-InPO, and the length of default DRX Cycle are signaled in SIB1. The values of N and PF_offset are derived from the parameter nAndPagingFrameOffset as defined in TS 38.331 [3]. The parameter firstPDCCH-MonitoringOccasionOfPO is signalled in SIB1 for paging in the BWP configured by initialDownlinkBWP. For paging in a DL BWP other than the BWP configured by initialDownlinkBWP, the parameter first-PDCCH-MonitoringOccasionOfPO is signaled in the corresponding BWP configuration.

If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

5G-S-TMSI is a 48 bit long bit string as defined in TS 23.501 [10]. 5G-S-TMSI shall in the formulae above be interpreted as a binary number where the left most bit represents the most significant bit.

In RRC_INACTIVE state, if the UE supports inactiveStatePO-Determination and the network broadcasts ranPagingInIdlePO with value "true", the UE shall use the same i_s as for RRC_IDLE state. Otherwise, the UE determines the i_s based on the parameters and formula above.

In RRC_INACTIVE state, if used eDRX value configured by upper layers is no longer than 1024 radio frames, the UE shall use the same i_s as for RRC_IDLE state.

In RRC_INACTIVE state, if used eDRX value configured by upper layers is longer than 1024 radio frames, during CN PTW, the UE shall use the same i_s as for RRC_IDLE state. Outside CN PTW, the UE shall use the i_s for RRC_INACTIVE state.