3GPP RAN WG2 Meeting #130	R2-2504272

St Julian’s, Malta May 19th – 23rd, 2025

Agenda Item:	8.13.1

Source:	Huawei, HiSilicon, CATT

Title:	Open issues for Multi hop Sidelink Relay in TS 38.331

Document for:	Discussion, Decision

# Introduction

The following document includes a list of open issues according identified during the following email discussion:

[Post129bis][401][Relay] Rel-19 relay service continuity CR to 38.331 (CATT)

Scope: Update the service continuity running CR from the baseline of R2-2503071 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR and open issue list for RAN2#130

Deadline: Long

[Post129bis][405][Relay] Rel-19 relay main CR to 38.331 (Huawei)

Scope: Update the 38.331 running CR from the baseline of R2-2503075 to take into account decisions of RAN2#129bis.

Intended outcome: Updated CR for RAN2#130

Deadline: Long

# Remaining open issues for specification 38.331

Open issue RRC-1: Measure Events for Service Continuity Scenarios C and D

For scenario A/B, the following agreements on measurement event were agreed.

RAN2#128/129

- Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

- The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching:

- Event Y2

- Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

RAN2#128/129bis

- Measurement event X2 could be applied to scenario B with multi-hop indirect to single-hop indirect path switching.

Next, we suggest to further discuss that for Rel-19 intra-gNB direct/single-hop indirect to multi-hop indirect path switching (scenario C/D), whether the legacy measurement event could be reused. For example, for scenario C, whether the UE can perform the legacy measurement event Y1/Y2. For scenario D, whether the UE can perform the legacy measurement event X2/Y2/Z1.

Open issue RRC-2: Contents of report for Service Continuity Scenarios C and D

For scenario A/B, the following agreements on measurement event contents were agreed.

RAN2#129

For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

For multi-hop i2d, first relay UE evaluates adjacent upstream relay link and Uu link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

For multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

Next, we suggest to further discuss that for Rel-19 intra-gNB direct/single-hop indirect to multi-hop indirect path switching (scenario C/D), what is the contents of measure report? For example, whether Candidate UE’s cell info, Candidate relay UE’s source ID, Measurement result and information indicating whether the measurement result is SL-RSRP or SD-RSRP are reported to the gNB as with legacy is enough. For the enhancement part, whether potential remote UE reports to the gNB the PC5 link qualities of each hops of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

Open issue RRC-3: Source relay becomes target for Service Continuity Scenario D

For scenario B, the following agreement on path switch scenarios were agreed.

RAN2#128

For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.

For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.

For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

Next, we suggest to further discuss that for Rel-19 intra-gNB single-hop indirect to multi-hop indirect path switching (scenario D), which scenario should be supported and potential spec impacts. For example, for scenario D, whether RAN2 supports the case that the target first/intermediate/last relay UE are all new relay UEs which are not the source single-hop U2N relay UE. Whether RAN2 needs to specify anything to support the case that that either the target first/intermediate/last relay UE is the source single-hop U2N relay UE.

Open issue RRC-4: Extending T300, T301 and T319

The T300, T301 and T319 are relevant Multi hop U2N Relays and their handling is shown below

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T300 | Upon transmission of RRCSetupRequest. | Upon reception of RRCSetup or RRCReject message, cell re-selection, relay (re)selection or cell selection by a L2 U2N Remote UE, and upon abortion of connection establishment by upper layers. | Perform the actions as specified in 5.3.3.7. |
| T301 | Upon transmission of RRCReestabilshmentRequest | Upon reception of RRCReestablishment or RRCSetup message as well as when the selected cell becomes unsuitable or the (re)selected L2 U2N Relay UE becomes unsuitable, upon reception of NotificationMessageSidelink indicating relayUE-HO or relayUE-CellReselection. | Go to RRC_IDLE |
| T319 | Upon transmission of RRCResumeRequest or RRCResumeRequest1 when the resume procedure is not initiated for SDT. | Upon reception of RRCResume, RRCSetup, RRCRelease, RRCRelease with suspendConfig or RRCReject message, upon cell re-selection or upon relay (re)selection. | Perform the actions as specified in 5.3.13.5. |

Rapporteur recommend further discussion on whether these timers should be extended to reduce the risk of failures, considering they were originally designed for the Uu hop and we will now also be supporting three additional SL hops.

Open issue RRC-5: NotificationMessageSidelink message handling

In the previous meeting, there was considerable discussion about sending the NotificationMessageSidelink to child UEs. However, we were still unable to reach a conclusion on whether this message can be sent in all scenarios, and whether it should be a forwarded copy of the original notification (with the same cause) or a regenerated message from the intermediate relay, reflecting its own cause based on its reaction.

The relevant agreements from the previous meeting are listed below.

RAN2#129 bis

- When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE).  FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment).  This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message.

- FFS detailed information in the indication and child UE handling.

- FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

- FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

In the current specification, for example, when an L2 U2N Relay UE receives a reconfigurationWithSync, it either notifies the upper layers (to trigger the release of the PC5 unicast link) or sends a NotificationMessageSidelink message to the connected L2 U2N Remote UE(s). It is then up to the remote UE to take appropriate action to recover from the situation.

Upon L2 U2N Relay UE receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.

The rapporteur's view is that, first and foremost, the same principle should be extended to the multi-hop scenario as outlined below.

Upon L2 U2N Relay UE receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release with its connected downstream child UE(s)) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) or to the connected downstream child UE(s) in accordance with 5.8.9.10.

When an intermediate relay UE receives a NotificationMessageSidelink message from the parent, it has two options

Option 1 – Forward a copy of the original notification (with the same cause) to its child UEs.
Option  2  –   Generate a new notification message and send it to the child UEs, with an updated cause that reflects its own response, such as relay reselection or re-establishment.

The issue with Option 1 is that it will lead to duplicate notifications being sent by the intermediate relay UE: first, a forwarded copy of the parent’s original notification, and then a second notification reflecting the intermediate relay’s own actions. This redundancy is clearly undesirable as it does not provide the child UE with any additional information.

To avoid this, the Rapporteur suggests adopting Option 2, where the intermediate relay UE generates a new notification message. This message would reflect the relay's own status or reaction (e.g., relay reselection or re-establishment) and is always sent, allowing the Remote UE to take appropriate action based on the most relevant and current context of its parent UE.

In Rapporteur view this approach also aligns with the principles established in Rel-17 and helps minimize the impact on the specifications. The current CR is drafted based on this baseline approach to support the essential handling. If any enhancements are deemed necessary beyond the baseline, companies are encouraged to discuss them further along with text proposal for the enhancements at the next meeting

Open issue RRC-6: Discovery Model B with no PC5 Link in multi hop scenario

In the previous meeting were discussions on  Discovery Model B with no PC5 Link established with the parent Relay UE. The relevant agreements from the previous meeting are listed below.

RAN2#129 bis

- If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration.  If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages.

- For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

- For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE.

- For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

It was agreed that in Discovery Model B, when there is no established PC5 link between the Remote UE and the Intermediate Relay UE, the Intermediate Relay UE should forward the solicitation message only if the PC5 RSRP between the Remote UE (or an Intermediate Relay UE) and the Intermediate Relay UE is above a defined threshold. This has been implemented in the current running CR.

However further discussion is needed regarding the scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.

First, we need to discuss whether this scenario can actually occur, or if the Intermediate Relay UE always establishes itself as a Remote UE beforehand. If this scenario is indeed possible, the next question is whether the same SD-RSRP threshold—used by the Intermediate Relay UE to decide whether to forward the solicitation message—can also be applied by the Last Relay UE when deciding whether to send the discovery response message to the Intermediate Relay UE.

Rapporteur recommends further discussion on this in the next meeting with contributions on these aspects.

Open issue RRC-7: Introduce a new field sl-L2U2N-MH-Relay in SIB 12

In R17 we had introduced new field sl-L2U2N-Relay-r17  indicating the support for NR sidelink Layer-2 U2N relay operation in SIB 12. Similarly, in R18 we had introduced new field sl-L2-U2U-Relay indicating the support for NR sidelink Layer-2 U2U relay operation.

In rapporteur’s view it is straight forward to add similar field sl-L2U2N-MH-Relay for indicating the support of NR sidelink Layer-2 U2N multi hop relay operation in SIB 12.

Companies can look at the running CR that proposes adding sl-L2U2N-MH-Relay  in the running CR and provide any comments.

Proposal 1: Introduce sl-L2U2N-MH-Relay for indicating the support of NR sidelink Layer-2 U2N multi hop relay operation in SIB 12.

Open issue RRC-8: introduce a new field relayUE-Operation-L2-r17   and remoteUE-Operation-L2-r17  in  SidelinkParameters

In R17 we had introduced new field relayUE-Operation-L2-r17 and remoteUE-Operation-L2-r17 in  SidelinkParameters which is used to convey capabilities related to NR and V2X sidelink communications/ positioning.

In rapporteur’s view it is straight forward to add similar capabilities for R19 e.g.  relayUE-MH-Operation-L2-r19 and remoteUE-MH-Operation-L2-r19 in SidelinkParameters IE.

Proposal 2: Introduce capabilities for R19 e.g.  relayUE-MH-Operation-L2-r19 and remoteUE-MH-Operation-L2-r19 in SidelinkParameters IE.

# Other identified open issues

Companies are invited to describe any other identified open issues not currently included within this document

| Company | Other identified open issues? (please describe) |
| --- | --- |

# Conclusions

The following proposals have been provided based on open issues identified above in the document:

Proposal 1: Introduce sl-L2U2N-MH-Relay for indicating the support of NR sidelink Layer-2 U2N multi hop relay operation in SIB 12.

Proposal 2: Introduce capabilities for R19 e.g.  relayUE-MH-Operation-L2-r19 and remoteUE-MH-Operation-L2-r19 in SidelinkParameters IE.

# References

R2_129b_Positioning_Relay_2025-0410-1040

R2_129_Positioning_Relay_2025-02-21-0845_eom