3GPP TSG-RAN WG2 Meeting #130	R2-2504277

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	8.9.2

Source:	Huawei, HiSilicon, Turkcell

Title:	Further consideration on Store and Forward

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, the following agreements were made regarding the Store & Forward (S&F) satellite operation [1]:

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

In this contribution, we provide some further considerations on the Store and Forward satellite operation.

# 2	Discussion

## 2.1 Transition Timing from Normal Mode to S&F Mode

In the last meeting [1], it was agreed that an indication for the satellite mode switch from normal mode to S&F mode would be included in system information, at least for NAS use, while the details of the transition time indication remain FFS (e.g., whether it can be linked to other exiting information).

When a serving cell switches from normal mode to S&F mode, it indicates that the feeder link will become unavailable at a certain point in time. In the legacy mechanism, the t-Service in SIB19 indicates when the current serving cell will stop providing service in the area it is currently covering. For cells supporting both normal and S&F mode, switching to S&F mode may render the cell unusable for legacy UEs. In this case, NW should set the t-Service to the enable the legacy UEs to start the measurement for mobility in a timely manner. During the previous discussions, a concern was raised that this will change the meaning of t-Service. However, it does not alter its interpretation for legacy UE. Because for legacy UE, the t-Service still represents the time at which the cell stops service, and it continues to follow the legacy behaviour. On the contrary, if t-Service is not set to the actual transition time, legacy UE(s) would be unable to start measurement in advance, even though the cell has already stopped serving them - this would deviate from the expected  legacy behaviour.

Observation 1: Setting t-Service to the transition time does not change the legacy principles and the behaviour of the legacy UEs.

On the other hand, after switching to the S&F, even if the NW releases all the legacy UEs in RRC_CONNECTED to RRC_IDLE which causes a lot of signalling overhead, legacy UEs will not perform neighbouring cell measurement before t-Service expires. And for the S&F UEs, we can use a new t-service time (e.g., t-serviceSF) to indicate the stop time for the SF mode.

In that case, the UEs that do not support S&F will begin measurements and transition to normal cells by the time of the mode switch (i.e., t-service). Meanwhile UEs that support S&F will continue to be served by the satellite until the new t-service time (e.g., t-serviceSF) is reached.

Proposal 1:(RRC-1) It is up to NW implementation to set the legacy t-Service to the transition timing from normal mode to “S&F operation” mode in some cases and a new t-ServiceSF can be introduced to indicate the stop-serving time of the SF mode.

## 2.2	Paging in S&F satellite operation

In the legacy paging procedure, the arrival of MT data triggers the network to page UE. In the S&F scenario, when feeder link is available, the CN sends MT data to satellite, which receives and stores it. When service link is available, the satellite pages UE or the UE initiates RRC connection, at which point the network sends MT data to UE. However, if the satellite does not receive any MT data when feeder link is available, there is no need for the satellite to page UE when service link is available. In this case, the UE doesn’t need to monitor paging or initiate RRC connection.

Therefore, if UE is informed when the NW does not have any MT data buffered for an area, the UE does not need to monitor the paging or initiate RRC connection, which helps to save UE power.

Proposal 2:(RRC-4) If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

## 2.3	Access control

For both S&F architectures, the UE’s DL NAS data can be stored at the MME on board the satellite serving the UE at next pass. In this case, if the UE is unable to access the intended satellite, the stored data will be lost. Given that the current access control mechanism may lead to rejection of the UE’s RRC connection request due to the load status, it is beneficial for the eNB to prioritize the access of the UE, if it is subsequently re-attempting the access to the NW, to avoid potential data loss. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

Proposal 3: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

## 2.4	Issue of satellite ID

During RAN2#129 meeting, RAN2 received the reply LS on satellite IDs from SA2 as follows [2]:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

[SA2 response]: SA2 has discussed the scenario and clarified this in the TS 23.401 clause 4.13.9.

Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s):

- the satellite has the UE context but does not necessarily support store-and-forward operation; or

- the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)?

[SA2 Answer]: SA2 confirms that option 2 is the correct understanding.

According to the LS content and the updated TS 23.401, the following RAN2 understanding is confirmed by SA2: “the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.”

In order to camp on the satellites that are not included in the NAS satellite ID list, the UE may need to continuously perform measurement for cell selection evaluation after being released to RRC_IDLE. This is to ensure that the UE will not miss any passing satellites. However, in this case, the NAS wait timer becomes ineffective, leading to increased power consumption for the UE. To alleviate this issue, when determine whether to perform measurements, the UE should also take into account the information for the subsequent satellites in SIB32, which was introduced for discontinuous coverage in Rel-17. In other words, UE if the t-ServiceStart for a satellite in SIB32 occurs earlier than the NAS wait timer, UE need to start performing measurement based on the t-ServiceStart to ensure UE can access that satellite.

Proposal 4: When determining whether to perform measurement in IDLE, the UE should consider both NAS information (i.e., Satellite list and NAS wait timer) and the information in SIB32 (i.e., t-ServiceStart and ephemeris).

However, if the UE detects that the satellite indicated in SIB32 (but not contained in NAS satellite list) is operating in SF mode, it should not access this satellite, as there would be no UE context on this satellite. If there were, the CN would have included this satellite into the NAS satellite list.

Proposal 5: If a satellite indicated in SIB32 but not included in the NAS satellite list is operating in SF operation, the UE should not access it.

# 3	Conclusion

This contribution discusses the store and forward operation for IoT NTN and we have the following proposals:

Remaining time of current operating mode

Observation 1: Setting t-Service to the transition time does not  change the legacy principles and behaviour of the legacy UEs.

Proposal 1:(RRC-1) It is up to NW implementation to set the legacy t-Service to the transition timing from normal mode to “S&F operation” mode in some cases and a new t-ServiceSF can be introduced to indicate the stop-serving time of the SF mode.

Paging in S&F satellite operation

Proposal 2: (RRC-4) If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

Access control

Proposal 3: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

Issue of satellite ID

Proposal 4: When determining whether to perform measurement in IDLE, the UE should consider both NAS information (i.e., Satellite list and NAS wait timer) and the information in SIB32 (i.e., t-ServiceStart and ephemeris).

Proposal 5: If a satellite indicated in SIB32 but not included in the NAS satellite list is operating in SF operation, the UE should not access it.

# 4	References

- R2-2502983, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 129bis.

- S2-2502450, Reply LS on Satellite IDs for store-and-forward operation, SA2.

- R2-2410913, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 128.