3GPP TSG-RAN WG2 Meeting #130	R2-2504284

St Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Functionality management for AI mobility

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#129bis:

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

Figure 1-1 Example: 3GPP TS 38.300 Measurement Model

The 5G measurement framework is shown in Figure 1-1. Related to it, in RAN#125bis the following cases were agreed for predicting cell-level measurements:

- Sub-Case 1: To predict beam level results, then generate cell level results based on the predicted beam results

- Sub-Case 2: To directly predict cell level results based on cell level results.

- Sub-Case 3: To directly predict cell level results based on beam level results

# 2	Discussion

## 2.1	Temporal Domain Case B RRM Measurement Reduction

The ML model for the time domain case B temporal RRM measurement prediction for measurement reduction may implement one of the three Sub-Cases. Regardless of the subcase, we can assume that the skipped measurements contain skipping actual L1 samples to bring meaningful energy saving in the UE and to allow reference signal optimization in the NW.

Figure 2.1-1 Sliding L1 filtering [1]

Figure 2.1-2 Non-Sliding L1 filtering [1]

As observed in [2] and shown in the two examples in Figures 2.1-1 and 2.1-2, using a sliding L1 filter does not result in any completely skipped measurement periods after applying the L1 filter. Therefore, with a sliding L1 filter we assume completely skipped measurements or samples are predicted only, when the time domain Case B measurement reduction is implemented using unfiltered beam-level results as input in either Sub-Case 1 or 3.

An example of a non-sliding L1 filter is shown in Figure 2.1-2. As discussed in [2], in time domain Case B measurement reduction, all L1 samples within a non-sliding L1 filter window may either be measured or all of them skipped, at least when Sub-Case 2 is used.

Observation 1: In time domain Case B measurement reduction all L1 samples within a non-sliding L1 filter window may either be measured or all of them are skipped.

The following options for L3 filtering in time domain Case B were agreed [1]:

Filtering option 1: L3 filtering is based on its L1 filtered result and the immediate last skipped measurement result Filtering option 2: L3 filtering is based on its L1 filtered result i.e. no L3 filtering Filtering option 3: L3 filtering is based on the L1 filtered result and last actual measurement result i.e. the skipped result(s) in between is ignored

Figure 2.1-3 Measurement Skipping Pattern (MSP) with non-sliding L1 filter and L3 filter option 3

Figure 2.1-3 shows the Measurement Skipping Pattern (MSP) with non-sliding L1 filter (length of 5 samples) and L3 filter option 3. We observe that due to filtering the MSP can be different for unfiltered L1 samples and filtered L1 and L3 measurements.

Observation 2: Due to filtering, the measurement skipping pattern can be different for unfiltered L1 samples and filtered L1 and L3 measurements.

If the measurement reduction does not require coordination with, e.g., measurement gaps scheduled at the NW side etc., the UE can choose the MSP independently. However, if the measurement reduction is intended to enable, for example, also reduction in the required measurement gaps to increase throughput and reduce latency, the MSP may need to be coordinated between NW and the UE. The coordinated MSP corresponds to the MSP of the unfiltered L1 samples.

Observation 3: In some scenarios, the used measurement skipping pattern may need to be coordinated between NW and the UE, for example to align the configured measurement gap pattern.

It was agreed in RAN2#129bis that:

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

Figure 2.1-4 Measurement configuration procedure overview

Figure 2.1-4 shows an overview of the measurement configuration process. Specifically, with AI/ML measurement saving the steps are:

- The UE indicates the supported measurement reduction methods and ratios, filter options and optionally also skipping patterns in its UE Capability Indication. Note that UE may indicate supporting more than one skipping pattern.

- Based on the indicated capabilities, the determines the measurement and measurement prediction configuration This may include selecting a skipping pattern and for example the measurement gap configuration.

- NW provides the UE with a measurement configuration, including the activation of the measurement saving feature and potentially the MRRT. If necessary, the selected skipping pattern and any related MG pattern or reference signals are also configured.

- The UE takes, predicts, filters and reports the measurements and predictions as configured.

As was observed above, in some cases MSP may need to be coordinated between the UE and the NW for the L1 samples that are skipped. The L1 sample skipping pattern can, however, depend on the UE proprietary L1 filter implementation.

Observation 4: The L1 sample skipping pattern can depend on the UE proprietary L1 filter implementation.

Proposal 1: Discuss on how the UE and NW can align on the measurement skipping patterns.

Proposal 2: Study how the NW can configure the measurement reduction feature, potentially including also the selected measurement skipping pattern.

Figure 2.1-5 Measurement Skipping Pattern (MSP) with non-sliding L1 filter and L3 filter option 3 with monitoring

To establish ground truth for the monitoring, the UE may need to take measurements during the configured measurement periods, which would be otherwise skipped. This may also need to be considered, in case an MSP is coordinated between the UE and the NW. Alternatively, a UE may predict a period it is configured to measure, but this may not be suitable for many implementations, which can require specific OW measurement period and length and specific PW.

The additional monitoring frames should be only a subset of the skipped measurements to avoid compromising the measurement reduction. Figure 2.1-5 shows an example, where every third skipped measurement frame in the MSP is measured for monitoring. If coordinated between the UE and the NW, the MSP should also support configuring additional monitoring frames, which are both predicted and measured.

Observation 5: To establish ground truth for the monitoring, the UE may need to measure selected otherwise skipped measurement periods.

Proposal 3: If coordinated between the UE and the NW, the MSP should also support configuring additional monitoring frames, which are both predicted and measured.

## 2.2	Time Domain Case A Measurement Prediction and Measurement Event Prediction

The following agreements were made in RAN2#129bis:

Agreements

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

- For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

For evaluation assumptions, two options for handover execution decision have been documented in TR 38.744 [1]:

- Relying on legacy measurement event

- Relying on predicted measurement event

In both options, the decision for an early handover preparation may be done based on the predicted measurement event. The options are for evaluation purpose only, any mobility decisions made based on reported measurements or predictions is network implementation, as agreed in the SID [3]:

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively.

Observation 6: Considering the SID, what mobility decisions network makes based on predicted or legacy reported event or measurements is a matter of NW implementation.

Figure 2.2-1 Sequence diagram of Option 1

Figure 2.2-1 shows an example sequence diagram, how network may use the reported predictions and measurements considering evaluation Option 1:

- UE indicates in its UE Capability Information that it is capable of measurement event prediction.

- Network configures UE with a measurement configuration including a measurement event for event-based reporting. Network additionally chooses to configure UE to report a prediction for the event.

- The UE measures the serving and the target candidate cells as specified in the measurement configuration.

- When the UE evaluates that the measurement event prediction trigger condition is fulfilled for one of the candidate cells, it infers the measurement event prediction model.

- If the measurement event prediction reporting condition is fulfilled, the UE reports the prediction to the network as configured. This may include one or more of the following:

  - Reporting predicted measurement(s)

  - Reporting actual measurement(s)

  - Reporting predicted measurement event information (occurrence, probability, predicted exact time)

- The network may decide to act based on the report. For example. in case of Option 1, the network may decide on an early handover preparation.

- The UE continues to measure the serving cell and the target candidate cells as configured.

- If the configured measurement event condition is met,

- The UE sends a measurement report as configured.

- The network decides if handover execution should be triggered based on the measurement repot

- If the network decides to execute the handover, it sends the HO command to the UE.

Observation 7: The UE needs indicate its capabilities to do measurement or measurement event prediction.

Observation 8: The network can configure, what the UE predicts, and reports based on the predictions.

Proposal 4: Consider the sequence diagram in Figure 2.2-1 as baseline for the measurement event prediction use case.

## 2.3	Frequency-Domain Measurement Event Prediction

Inter-frequency measurement event prediction was agreed in RAN2#129:

Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required.

In scenario 3, FR1 to FR1 inter-frequency (frequency domain) measurement prediction for measurement reduction has been studied and evaluated. Similar to indirect time domain measurement event prediction, the predicted inter-frequency measurements can be used for inter-frequency measurement event prediction, e.g., predicting an inter-frequency A4 measurement event based on measurements of the serving cell frequency. The predicted inter-frequency measurement events can be reported to the network, which may, for example, decide to configure the UE to measure the other frequency only when the prediction indicates that the UE may be able to connect to it. This can reduce the inter-frequency measurements the UE needs to take, reducing also the need for measurement gaps.

Considering the co-located inter-frequency scenario, the strongest cell in the target frequency may be known, but to support also non-co-located scenarios, the inter-frequency measurement prediction may in this case be configured to predict the measurements of not a specific cell but whichever is the strongest in the target frequency, which can be used to determine if the A4 condition is fulfilled for a cell in the target frequency. The specific cell to connect to is determined with a measurement, which is only configured, when the predicted A4 indicates that a cell with a strong-enough signal is likely to be available.

Observation 9: The predicted inter-frequency measurements can be used for inter-frequency measurement event prediction

Observation 10: The predicted inter-frequency measurement events can be used to reduce the inter-frequency measurements.

Observation 11: The inter-frequency measurement prediction may be configured to predict the measurements a specific cell or the strongest cell in the target frequency.

Proposal 5: RAN2 to consider predicting measurement events in the frequency domain for measurement reduction goal.

Proposal 6: Study configuration and reporting of predicted measurement events in the frequency domain for a specific cell or for any cell in the target frequency.

## 2.4	ML Framework Impacts

The following agreements on definitions were made in RAN2#129bis, that following the 3GPP TR 38.843 definitions:

1. Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) 2. Applicable functionalities refers to functionalities that the UE is ready to apply for inference 3. Activated functionalities refers to functionalities already enabled for performing inference

Additionally, the following agreements were made in RAN2#129bis:

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

In mobility scenarios, the applicability of trained ML models may depend on, for example:

- Network deployment

- UE/device

- Radio environment

- UE movement/mobility profiles

Each of these may also experience changes, for example:

- Network: New or removed sites, hardware or software changes, reconfigurations (transmission power, down-tilt etc.)

- UE/device: New devices, software upgrades, reconfigurations

- Radio environment: New or removed shadowing or blocking objects, weather

- UE movement/mobility profiles: New streets etc.

What features are the relevant to be considered in the applicability framework depends on the generalization level and design of the ML functionality. For example, an area-specific ML model may be affected by individual changes in the radio environment, e.g., changes in objects causing shadowing and NLOS, or changes in the typical UE trajectories, whereas a more generalized ML model may be applicable to specific types of environments, like urban or suburban. However, changes in the radio environment and UE trajectories may be assumed not to be known upfront and if necessary, must be detected with model monitoring. For NW-side ML, no applicability framework is expected.

Observation 12: Changes in the radio environment and UE trajectories, if necessary, must be detected, e.g., by model monitoring.

Proposal 7: For NW-side ML, no applicability framework is expected.

Similar generalization level considerations affect the applicability to different network configuration. For example, cell configuration changes such as transmission power or down-tilt changes may affect site- or area-specific models, but the applicability of more generic models depends on, for example, the type of the site (macro, micro). In the study item, the RRM measurement prediction generalization to UE speeds and different site types and ISDs was evaluated, and it was observed that the models generalized well. However, it has not been studied, if a generalization level can be agreed for all UE-side implementations per use case.

Observation 13: The relevant features to be considered in the applicability framework depend on the generalization level and design of the ML functionality.

Observation 14: If additional information is required to be provided by the network to the UE to assess the UE-side ML functionality applicability, the relevant information should be studied per use case and the ML functionality generalization level agreed.

If any additional configuration is required to be provided by the network to the UE to assist the UE-side ML functionality applicability determination, the relevant configuration should be studied per use case and the ML functionality generalization level agreed. This applies also if an associated ID is used instead of specific configuration parameters, because the same considerations are required to provide associated ID values, which are meaningful for the UE-side ML functionality generalization level.

Observation 15: The previous observation applies also if an associated ID is used, because the same considerations are required to provide associated ID values, which are meaningful for the UE-side ML functionality generalization level.

Proposal 8: RAN2 to consider applicability-related additional configuration only if the required configuration has been studied and the ML generalization level agreed.

# 3	Conclusion

This document has made the following observations:

Observation 1: In time domain Case B measurement reduction all L1 samples within a non-sliding L1 filter window may either be measured or all of them are skipped.

Observation 2: Due to filtering, the measurement skipping pattern can be different for unfiltered L1 samples and filtered L1 and L3 measurements.

Observation 3: In some scenarios, the used measurement skipping pattern may need to be coordinated between NW and the UE, for example to align the configured measurement gap pattern.

Observation 4: The L1 sample skipping pattern can depend on the UE proprietary L1 filter implementation.

Observation 5: To establish ground truth for the monitoring, the UE may need to measure selected otherwise skipped measurement periods.

Observation 6: Considering the SID, what mobility decisions network makes based on predicted or legacy reported event or measurements is a matter of NW implementation.

Observation 7: The UE needs indicate its capabilities to do measurement or measurement event prediction.

Observation 8: The network can configure, what the UE predicts, and reports based on the predictions.

Observation 9: The predicted inter-frequency measurements can be used for inter-frequency measurement event prediction

Observation 10: The predicted inter-frequency measurement events can be used to reduce the inter-frequency measurements.

Observation 11: The inter-frequency measurement prediction may be configured to predict the measurements a specific cell or the strongest cell in the target frequency.

Observation 12: Changes in the radio environment and UE trajectories, if necessary, must be detected, e.g., by model monitoring.

Observation 13: The relevant features to be considered in the applicability framework depend on the generalization level and design of the ML functionality.

Observation 14: If additional information is required to be provided by the network to the UE to assess the UE-side ML functionality applicability, the relevant information should be studied per use case and the ML functionality generalization level agreed.

Observation 15: The previous observation applies also if an associated ID is used, because the same considerations are required to provide associated ID values, which are meaningful for the UE-side ML functionality generalization level.

And proposed the following:

Proposal 1: Discuss on how the UE and NW can align on the measurement skipping patterns.

Proposal 2: Study how the NW can configure the measurement reduction feature, potentially including also the selected measurement skipping pattern.

Proposal 3: If coordinated between the UE and the NW, the MSP should also support configuring additional monitoring frames, which are both predicted and measured.

Proposal 4: Consider the sequence diagram in Figure 2.2-1 as baseline for the measurement event prediction use case.

Proposal 5: RAN2 to consider predicting measurement events in the frequency domain for measurement reduction goal.

Proposal 6: Study configuration and reporting of predicted measurement events in the frequency domain for a specific cell or for any cell in the target frequency.

Proposal 7: For NW-side ML, no applicability framework is expected.

Proposal 8: RAN2 to consider applicability-related additional configuration only if the required configuration has been studied and the ML generalization level agreed.

# 4	References

[1]	3GPP Rel-19 TR 38.744 “AI/ML for Mobility in NR”

[2] R2-2407979, “Simulation results for RRM measurement prediction”, vivo, RAN2#127bis, Hefei, China, Oct 14th – 18th, 2024