3GPP TSG-RAN WG2 Meeting #130	R2-2504285

St Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.3.5

Source:	Nokia

Title:	Discussion on performance monitoring

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#129bis:

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

In this document, we present our views on performance monitoring. First, we introduce several definitions which would be useful in aligning the understanding of different terms. Then, we present our views on the signaling impacts, particularly focusing on the ground truth label acquisition, monitoring time window and the detection of ML failures.

# 2	Discussion

## 2.1	Model Drift

We begin by laying out some key definitions which will be used throughout this discussion:

- Data drift: It occurs when the distribution of the input features changes over time. This can happen when the statistical properties (mean, variance, correlations, etc.) of the input features change without the relationship between them and the output data changing.

- Concept drift: It occurs when the relationship between the input features and target variables changes over time, which invalidates the patterns and input-output mappings that the model learned during the training phase.

- Model drift: A term that refers to data drift and concept drift collectively.

Proposal 1: Capture the definitions for data, concept, and model drift in the TR.

## 2.2	Signaling Impacts

To monitor the performance of a machine learning model for drift, we consider the following two cases:

- UE-sided models: When the ML model is on the UE, the UE may continuously calculate and monitor the performance metrics of its ML model, such as precision, recall, F1 score, etc. When the UE detects drift, it may require coordination with the network to e.g. disable or switch to a certain feature. This may have some specification impact.

- NW-sided models: When the ML model is deployed on the NW, it can calculate its own performance metrics and autonomously decide how to reconfigure the UEs in case of drift. In our understanding, this case is largely supported by current specifications. If needed, additional specification that may be required to report data needed by the network for establishing the ground truth, should be studied on a case-by-case basis.

Observation 1: To enable performance monitoring, performance metrics need to be collected by the UE and/or the network.

Proposal 2: As baseline, discussions on monitoring consider UE-sided models.

Proposal 3: Identify signaling impacts for supporting the reporting of UE-sided model metrics.

Another important aspect to consider is how the network or the UE should react to a detected degradation in the model’s performance. This also depends on the use-case, but a more general approach involves a fallback mechanism to another configured feature. The network may want to signal to the UE to fall back to a different feature or switch to another functionality when it detects a performance degradation based on the UE’s own reports.

Proposal 4: In case of detected performance degradation, fallback to a NW-configured functionality is supported.

### Ground-Truth Labels Acquisition

Calculating intermediate KPIs requires acquiring the ground truth values which correspond to the model's predictions. In some use-cases, obtaining the ground truth labels can be straightforward, but for others, extra measures need to be taken to acquire them, which can also require coordination between the NW and the UE. The necessary actions required to establish the ground truth need to be considered on a per use case basis.

In time domain RRM measurement prediction case A, measurements can be used as a ground truth for predictions for the same measurement period. Similarly, in measurement event prediction, measurement events determined based on the measurements can be used as a ground truth for the predictions. The ground truth is often available to the UE without any specification impact. The ground truth cannot be obtained only in scenarios, where the predicted measurements are not measured at a future time or the actual measurement event that would have occurred based on the measurements is not reported, for example due to executing a handover based on the predictions. These scenarios can be relevant for monitoring, however.

In measurement reduction use cases, i.e., in time domain RRM measurement prediction time domain case B and in inter-frequency measurement prediction, the ground-truth labels, i.e., values of the skipped measurements, are not readily available. Acquiring the ground truth for the skipped measurement periods incurs a cost, since it reduces the measurement reduction ratio. Also, coordination between NW and the UE may be required if the measurements for monitoring require, e.g., scheduling additional measurement gaps.

Consider a time-domain measurement reduction use-case, with 50% measurement reduction rate, as shown in Figure 2.2.1-1. Here, measurements are predicted (not measured) by a machine learning model for every other sample. To obtain the ground truth labels (the true physical measurements), the UE would need to physically perform and obtain the measurements in some of those slots where it should normally predict instead of measuring them. The network may need to coordinate with the UE regarding the periodicity of these slots and on which specific frames they occur.

Figure 2.2.1-1: Acquiring ground-truth labels for time-domain measurement reduction Case B with 50% MRRT.

Observation 2: Additional specification effort may be needed for establishing the ground truth in UE-side monitoring also when monitoring metrics are not reported.

Observation 3: In measurement reduction use cases, acquiring the ground truth can incur a cost, e.g., reduced measurement reduction ratio, and the measurements required for monitoring can require coordination between the network and the UE.

Observation 4: In time domain case A and measurement event prediction, in most scenarios the ground truth can be acquired with lower cost and without coordination between the network and the UE.

Proposal 5: Study the configuration aspects needed so that the network and the UE can coordinate for establishing the ground truth in case of ML monitoring.

### 2.2.2 Monitoring Time Window

Performance metrics are statistical quantities that are calculated over a large set of samples. To calculate the F1 score for a binary classifier, for example, it requires collecting a large enough dataset of test samples where both the ground truth labels, and the predictions are known. This creates a challenge for calculating performance metrics for continuous online monitoring of machine learning models deployed on the UE or the network, because it involves calculating the performance metrics over a rolling window of recent predictions or time instances. The length of this window is a key design choice that presents a fundamental tradeoff between robustness and responsiveness.

Observation 5: Online performance monitoring requires calculating metrics over a time window of recent predictions or data instances. The length of this window is a key design choice that presents a tradeoff between robustness and responsiveness.

Short windows are sensitive to changes and allow for a timely detection of model drift and performance degradation, but they are also prone to noise and variance, which increases the occurrence of false alarms or overreaction to irrelevant outliers. Long windows, on the other hand, smooth out such noise, so they provide a more stable and robust estimate of the model’s performance, but they delay the detection of model drift, as the metrics get “diluted” by older data.

Observation 6: Short windows are sensitive to changes and allow for a timely detection of model drift, but they are also prone to noise and variance. Long windows provide a more stable estimate of the model’s performance, but they delay the detection of model drift.

This is an inherent and unavoidable tradeoff. The optimal window size that balances robustness and responsiveness depends on factors that are specific to the use-case. For some use-cases, the cost of a false alarm may be relatively higher, so longer windows are more fitting. An example for such a use-case is A3 event prediction, where a false positive may trigger the UE to perform a handover too early, which can result in a handover failure. For other use-cases, such as RLF prediction, swift detection of drift may be more important, so shorter windows are more fitting. This may require that the length of the monitoring window be coordinated between the network and the UE.

Proposal 6: Discuss the tradeoff between robustness and responsiveness in performance monitoring and consider whether the length of the monitoring window needs to be coordinated between the network and the UE.

### 2.2.3 Detection of ML failures

In our understanding, the most important use-case for monitoring is the detection of cases where the AIML-related configuration cannot be supported from the UE. For instance, due to drifts in the data and/or model, the UE cannot support a high MRRT in case of measurement prediction, or the UE starts to (mis)detect the different events that are configured towards it.

Observation 7: AIML configuration support faces challenges such as managing data drift, ensuring timely model updates, and maintaining continuous inference as UE transitions between coverage areas.

Considering legacy systems, examples of detection that the given configuration is no longer applicable are RLF and HOF detection. In case of the former, the UE’s physical layer monitors the DL quality of the primary cell for indicating sync status (i.e., in/out of sync). Upon receiving N310 consecutive out of sync indications, UE starts the T310 timer and upon expiry of timer, UE declares RLF and triggers the subsequent procedures (e.g., re-establishment or any recovery procedure such LTM or CHO recovery). In a similar manner, in case of handover UE monitors for T304 expiry to declare HOF and trigger the subsequent procedures.

Observation 8: Legacy systems utilize RLF and HOF detection mechanisms to identify when a configuration is no longer applicable, triggering procedures like re-establishment or recovery based on specific timer expirations.

Considering the current applicability framework, it is possible that in case UE may not have models to support a configured functionality, in such cases, UE would indicate to NW that the configured functionality is inapplicable. Another case where the UE might signal inapplicable functionalities is the case where the performance of the models in the UE has drifted. In case UE-side monitoring of the features is configured, UEs need to have a mechanism to detect such failures and indicate the relevant failure information to the NW. From NW perspective, a scenario that would be nice to avoid is the case where UE identifies and signals failures (or inapplicability) of the functionalities either too early or too late as that would require more frequent reconfigurations towards the UE and introduce unnecessary complexity and potentially interruption time. In the case of periodic predictions, as in RRM measurement prediction, the RLF detection method can serve as an example of how drift may be detected.

Observation 9: To minimize the number of reconfigurations, UEs need a mechanism to detect and report performance drift or inapplicability of functionalities to the network, while avoiding too early or too late signaling.

Observation 10: In case of periodic predictions, as in RRM measurement prediction, the RLF detection method can serve as an example of how drift may be detected.

Proposal 7: Study the detection and reporting of inapplicability from UE to NW for AI Mobility use-cases. In the case of periodic predictions, the legacy RLF detection mechanism can be considered as starting point.

# 3	Conclusion

This document has made the following observations:

Observation 1: To enable performance monitoring, performance metrics need to be collected by the UE and/or the network.

Observation 2: Additional specification effort may be needed for establishing the ground truth in UE-side monitoring also when monitoring metrics are not reported.

Observation 3: In measurement reduction use cases, acquiring the ground truth can incur a cost, e.g., reduced measurement reduction ratio, and the measurements required for monitoring can require coordination between the network and the UE.

Observation 4: In time domain case A and measurement event prediction, in most scenarios the ground truth can be acquired with lower cost and without coordination between the network and the UE.

Observation 5: Online performance monitoring requires calculating metrics over a time window of recent predictions or data instances. The length of this window is a key design choice that presents a tradeoff between robustness and responsiveness.

Observation 6: Short windows are sensitive to changes and allow for a timely detection of model drift, but they are also prone to noise and variance. Long windows provide a more stable estimate of the model’s performance, but they delay the detection of model drift.

Observation 7: AIML configuration support faces challenges such as managing data drift, ensuring timely model updates, and maintaining continuous inference as UE transitions between coverage areas.

Considering legacy systems, examples of detection that the given configuration is no longer applicable are RLF and HOF detection. In case of the former, the UE’s physical layer monitors the DL quality of the primary cell for indicating sync status (i.e., in/out of sync). Upon receiving N310 consecutive out of sync indications, UE starts the T310 timer and upon expiry of timer, UE declares RLF and triggers the subsequent procedures (e.g., re-establishment or any recovery procedure such LTM or CHO recovery). In a similar manner, in case of handover UE monitors for T304 expiry to declare HOF and trigger the subsequent procedures.

Observation 8: Legacy systems utilize RLF and HOF detection mechanisms to identify when a configuration is no longer applicable, triggering procedures like re-establishment or recovery based on specific timer expirations.

Observation 9: To minimize the number of reconfigurations, UEs need a mechanism to detect and report performance drift or inapplicability of functionalities to the network, while avoiding too early or too late signaling.

Observation 10: In case of periodic predictions, as in RRM measurement prediction, the RLF detection method can serve as an example of how drift may be detected.

And proposed the following:

Proposal 1: Capture the definitions for data, concept, and model drift in the TR.

Proposal 2: As baseline, discussions on monitoring consider UE-sided models.

Proposal 3: Identify signaling impacts for supporting the reporting of UE-sided model metrics.

Proposal 4: In case of detected performance degradation, fallback to a NW-configured functionality is supported.

Proposal 5: Study the configuration aspects needed so that the network and the UE can coordinate for establishing the ground truth in case of ML monitoring.

Proposal 6: Discuss the tradeoff between robustness and responsiveness in performance monitoring and consider whether the length of the monitoring window needs to be coordinated between the network and the UE.

Proposal 7: Study the detection and reporting of inapplicability from UE to NW for AI Mobility use-cases. In the case of periodic predictions, the legacy RLF detection mechanism can be considered as starting point.