3GPP TSG-RAN WG2 Meeting #130	R2-2504286

St Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.3.4

Source:	Nokia

Title:	Considerations on data collection

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#129bis, the following agreements are made regarding to data collection aspects for AI/ML mobility.

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

- Model transfer will not be discussed in this study item

7	For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

In accordance with the RAN2 agreement, the data collection framework will apply the AI PHY as its foundational baseline. Consequently, this contribution is organized around the recent agreements from the AI PHY work items 8.1.3 (network-sided data collection) and 8.1.4 (UE-sided data collection). Key aspects discussed in AI PHY include mainly: data collection configuration, data collection mechanism, reporting mechanism, and UE behavior during handover for network-sided data collection. We will further explore UE-sided data collection in subsequent sections. Detailed agreements will be referenced in the relevant sections below. Building on the previously discussed scopes and content from AI PHY, this contribution is organized to explore similar directions for AIML mobility on data collection aspects.

# 2	Discussions

## 2.1	Network sided data collection

## 2.1.1	Data collection configuration

In the AIML mobility study item, two use cases have been evaluated: RRM measurement prediction (across temporal, spatial, and frequency domains) and measurement event prediction. Unlike AI PHY use cases, such as beam management, which primarily focus on measurement data, AI mobility use cases may necessitate the collection of additional data beyond measurements for network-side model training, inference, or monitoring. For RRM measurement prediction, the regression ML model can be trained and evaluated with RRM measurement data. Therefore, the configuration of RRM measurement data collection may still adhere to the AI PHY agreement. For the measurement event prediction use case,  additional information may be required, in particular when configured actual measurement events have occurred.

Proposal 1: Extend UE-side logging and related configurations to collect the relevant measurements and labels as configured. Details FFS for each use case.

In the following, we discuss configurations related to the collection of measurement-specific data, beginning with analysis and agreements of AI PHY use cases.

As discussed in [1], for AI beam management use cases, the configuration for data collection in offline training will depend on UEs receiving CSI measurement configurations for RS resources and resource sets, such as NZP-CSI-RS or SSB-RS resources, as specified in CSI-MeasConfig. The CSI-MeasConfig framework serves as a prerequisite for configuring measurements and logging data collection for network-side models. Within CSI-MeasConfig, the CSI-ReportConfig IE specifies the CSI ResourceConfig to be utilized for measurement purposes.

According to TS38.331, it is important to note that CSI-MeasConfig is cell-specific and configured per serving cell through ServingCellConfig. For AI beam management use cases, it is logical to consider adding data collection configuration IE within the cell-level configuration, as inter-cell mobility is not a factor here. However, for mobility purposes, this per-cell configuration for CSI-based measurements may not be applicable to all AI mobility use cases, particularly when L3 cell-level measurements are of interest.

In AI mobility use cases, the RSs utilized for RRM measurements and mobility purposes are configured through MeasObjectNR, which is a component of MeasConfig. Notably, MeasConfig IEs are not configured per serving cell; instead, they reside directly under the top-level RRCReconfiguration IEs. This configuration applies universally to the entire UE, encompassing all its serving cells and carriers for handover and mobility-related measurements.

Observation 1: Compared to the AI PHY beam management use case, AI mobility emphasizes RRM measurement over CSI measurement. Therefore, MeasConfig appears to be more appropriate for defining the logging configuration.

Proposal 2: For NW-side deployments, RAN2 to consider the data collection configuration within MeasConfig IEs for AI mobility use cases.

## 2.1.2	Data collection mechanism

During RAN2#129, it was agreed in AI PHY to support the use of L3 measurement event triggers, specifically when L3 serving cell measurements become worse or better than a threshold for TTT, to determine whether the UE performs logging. L1 measurement event triggers will not be supported, and further clarification is needed of what should be logged.

All agreements for NW side data collection (AI PHY)

Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

Low power bit indication is supported

Data availability indication is supported.  FFS when this would be triggered

As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

FFS how to handle idle/inactive and RLF cases and whether we have a unified.

Based on this agreement, the existing measurement events, e.g., A1 (serving cell becomes better than absolute threshold), A2 (serving cell becomes worse than absolute threshold), and A3 (neighbor becomes offset better than SpCell), can be reused with related configurations, such as reportInterval and reportAmount as specified in TS 38.331. In addition, s-MeasConfig establishes a threshold for NR SpCell RSRP measurement, determining when the UE must perform measurements on non-serving cells. Utilizing this threshold for logging initiation ensures the collection of meaningful data related to mobility, encompassing both serving and neighboring cells.

Observation 2: Initiating data collection at the UE using the s-MeasConfig threshold can ensure that measurements from both serving and neighboring cells are included.

Based on the previous discussion, data collection can be initiated either periodically or in response to specific measurement events. Additionally, a dedicated indication signaling from the network may be employed to trigger data collection at the UE, utilizing a mechanism similar to semi-persistent CSI-RS reporting with DCI or MAC CE as per TS38.214. Upon receiving this indication, the UE can choose to either log or report the measurement results based on its specific logging requirements. However, further discussion is required for RAN2.

Proposal 3: Consider the following mechanism to initiate the data logging for AI mobility use cases:

- Periodic logging: logging interval or periodicity can be a new RRC parameter.

- Event triggered logging: measurement Event A* or s-MeasureConfig threshold can be reused.

- Network indicated logging: an existing or dedicated DL indication signal can be used. Details FFS.

## 2.1.3	Logging interval

In legacy logged MDT operations, when the report type is periodic, the UE logs data at the configured logging intervals. Similarly, for periodic RRM measurement logging, which is based on CSI-RS or SSB, the UE can log data at each interval, with the minimum logging interval corresponding to the transmission periodicity of the CSI-RS or SSB.

Proposal 4: For periodic logging, the minimum logging interval should align with the CSI-RS/SSB transmission periodicity.

Prioritization of logged data

The ongoing RAN2 discussions on AI PHY, along with previous agreements, have not adequately addressed the prioritization of data to be logged in buffers or the selection of corresponding logging intervals for measurement data with varying prioritization levels within the context of AI mobility procedures. The UE may accumulate low‐priority or irrelevant data for network‐side model training, inference or monitoring. This not only degrades the overall performance of predictions but also forces operators to initiate additional data collection efforts, which yields little benefit. Furthermore, transmitting non-critical data to the network consumes memory and signaling resources on the operator side, while unnecessarily occupying over-the-air bandwidth.

Observation 3: The current agreements lack mechanisms for prioritizing specific types of data to be logged by the UE.

Several prioritization aspects can be considered for data collection by the UE. For instance, prioritization can be determined based on UE radio conditions, such as measurements taken at the cell edge or in higher speed categories, which should be assigned higher priority. Additionally, one of the most critical use cases for AI/ML-based RRM measurement prediction is assessing the accuracy of AI/ML in predicting the frequency domain. As the frequency gap between two bands increases, the differences in radio propagation characteristics become more pronounced, making prediction more challenging. Consequently, a larger frequency gap relative to the serving cell carrier should be considered for higher prioritization

Proposal 5: RAN2 to study different data prioritization levels for logging at the UE.

## 2.1.4	Report mechanism

RAN2 #129bis on AI PHY agrees that the availability indication can be triggered by several conditions, including 1) a full buffer being reached, 2) a buffer threshold being met, or 3) low power, provided these configurations are set. When any of these triggers occur, the UE sends an UAI to signal that data is available. This UAI includes the reason for the trigger, such as a full buffer or threshold, and may also contain a low power indication.

Agreements on availability indication (AI PHY) - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE sends a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

In our view, sending an indication when the UE buffer is already full risks data loss due to overwriting, as the UE may continue logging data while the network collects existing buffer data. To address this, it is important to clarify the implications for different UEs based on buffer sizes and management logic when indicating before the buffer is full. Additionally, frequent data availability indicators for minimal data in the buffer may prove inefficient.

Observation 4: Triggering the availability indication when a buffer threshold is met is more robust (minimizing loss of data) than waiting for the buffer to be completely full.

Furthermore, when the UE is configured with logging intervals of different lengths and buffers to collect data with differing priorities, relying on a single buffer threshold to trigger reporting is unsuitable. To address this issue, the network can configure multiple buffer thresholds that align with different logging intervals and buffer sizes for triggering reporting. This approach ensures more efficient handling of data prioritization and reporting.

Proposal 6: Study the use of different buffer thresholds to enable data availability indication.

RAN2 #129bis on AI PHY agrees that a new SRB can be configured for network-side data collection with lower priority.

Agreements on SRB to be used

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

- New SRB can be configured for NW-side data collection (with lower priority)

However, according to 3GPP TS 38.331 clause 4.2.2, there are multiple options available for different SRB types, even when assigned lower priorities.

"Signalling Radio Bearers" (SRBs) are defined as Radio Bearers (RBs) that are used only for the transmission of RRC and NAS messages. More specifically, the following SRBs are defined: - SRB0 is for RRC messages using the CCCH logical channel (except SRB0 of L2 U2N Remote UE); - SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel (except SRB1 of L2 U2N Remote UE); - SRB2 is for NAS messages and for RRC messages which include logged measurement information, all using DCCH logical channel (except SRB2 of L2 U2N Remote UE). SRB2 has a lower priority than SRB1 and may be configured by the network after AS security activation; - SRB3 is for specific RRC messages when UE is in (NG)EN-DC or NR-DC, all using DCCH logical channel; - SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 has a lower priority than SRB1 and can only be configured by the network after AS security activation. - SRB5 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB5 has a lower priority than SRB1 and SRB3 and can only be configured by the SN serving the SCG when the UE is in NR-DC, after AS security activation.

Observation 5: SRB3 and SRB5 are configured exclusively for dual connectivity (EN-DC or NR-DC), making them unsuitable for the current use case.

Despite the agreement for AI PHY to utilize low priority SRBs for data transmission, they may not effectively address mobility use cases. Mobility-related measurement data can have varying priorities based on radio propagation conditions, UE mobility procedures, and specific use case data. In particular, different cells may have different requirements for data collection and reporting. Thus, it remains uncertain whether using a single common SRB with low priority can effectively manage the transmission of data for AI mobility use cases, particularly when non-measurement data, such as event labels or flags, or data with varying prioritization levels, are involved.

Proposal 7: Discuss the need of a common or different SRBs for network sided data collection to facilitate transmission of data with different priorities.

## 2.1.4	UE behavior during HO, IDLE/ INACTIVE and RLF

RAN2 #129bis on AI PHY agrees to use 1-bit indication to determine whether un-retrieved data should be released or retained within the RRCReconfiguration phase. This decision is made by the source gNB, which assesses whether the data should be preserved. The indication is incorporated into the RRCReconfiguration, rather than being included in the RRC Reconfiguration from the target cell, with specific signaling details to be finalized. Additionally, when the UE transitions to RRC_IDLE, RLF, or RRC_INACTIVE states, it automatically discards any logged data to ensure efficient data handling and resource management.

Agreements on Idle/inactive and HO (AI PHY)

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

For AI mobility use cases, as previously discussed, the UE may be configured with varying logging intervals and buffer sizes to manage collected data with different priorities, especially in response to changes in radio conditions or the need to measure different carriers. Consequently, a simplistic binary approach to either release or retain logged data during handover, idle/inactive states, and RLF may prove ineffective. Flushing all collected data during handover execution could result in the loss of critical data upon re-establishing the connection to the target. Conversely, retaining all data, particularly low-priority data, could lead to inefficient memory usage by the UE.

Observation 6: The current agreements for AI Phy use-cases suggest using a binary indication, which prevents selective discarding of low-prioritized data. This can lead to inefficient buffer use and resource handling.

Observation 7: AI mobility studies and RLF processes that automatically discard data during UE transitions to RLF could result in the loss of critical features necessary for data collection.

Proposal 8: Study whether the UE can retain a certain level of prioritized data during the handover process.

## 2.2	UE sided data collection

Both configuration and transfer of UE-side data collection have been discussed at length throughout the AI Phy SI and WI. In RAN2#129bis, for AI Mobility we agreed the following:

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

The relevant procedures for AIM Phy that would extend to mobility would be the ones defined for the BM use-case. As discussed in Section 2.1 of this contribution, using the CSI measurement configuration for collecting mobility data would suffer from the limitations associated with the need for collecting data from more than one serving cell. Extending the framework to collect mobility data would also incur larger specification effort. On the other hand, re-using the RSs utilized for RRM measurements and mobility purposes configured through MeasConfig would mitigate such drawbacks associated with collecting data from multiple cells pertaining to multiple gNBs.

Proposal 9: For UE-side deployments, enhance the MeasConfig IEs for collecting data for AI mobility use cases.

In case of AI Phy BM, the emphasis is mainly on intra-gNB configuration. However, in case of mobility, another important aspect to consider when collecting data for UE-side models relates to the potential inter-operability issues that arise in different deployments. We think it is important to identify and address such issues as part of the SI.

Proposal 10: Identify and study potential inter-operability issues that arise when collecting data for UE-side models.

If allowed by the network, an idle UE may potentially want initiate an RRC connection for data collection reasons by an RRC setup message and request a specific measurement configuration, for example to collect measurement and event data as it crosses a cell border, without endangering a degradation in the data plane traffic, for example if additional measurement gaps are required. The RRC setup message can indicate the specific purpose of data collection or exploration and refer to a measurement configuration the UE is requesting for its experiment. If the network is able to accommodate the request, it may configure the UE with the requested configuration.

Observation 8: RRC connection for data collection should enable measurement and event data collection also when otherwise not connected and without endangering a degradation in the data plane traffic, for example if additional measurement gaps are required.

Proposal 11: RAN2 to consider an RRC connection set up specifically for measurement and event data collection.

# 3	Conclusion

This document has made the following observations:

Observation 1: Compared to the AI PHY beam management use case, AI mobility emphasizes RRM measurement over CSI measurement. Therefore, MeasConfig appears to be more appropriate for defining the logging configuration.

Observation 2: Initiating data collection at the UE using the s-MeasConfig threshold can ensure that measurements from both serving and neighboring cells are included.

Observation 3: The current agreements lack mechanisms for prioritizing specific types of data to be logged by the UE.

Observation 4: Triggering the availability indication when a buffer threshold is met is more robust (minimizing loss of data) than waiting for the buffer to be completely full.

Observation 5: SRB3 and SRB5 are configured exclusively for dual connectivity (EN-DC or NR-DC), making them unsuitable for the current use case.

Observation 6: The current agreements for AI Phy use-cases suggest using a binary indication, which prevents selective discarding of low-prioritized data. This can lead to inefficient buffer use and resource handling.

Observation 7: AI mobility studies and RLF processes that automatically discard data during UE transitions to RLF could result in the loss of critical features necessary for data collection.

Observation 8: RRC connection for data collection should enable measurement and event data collection also when otherwise not connected and without endangering a degradation in the data plane traffic, for example if additional measurement gaps are required.

And proposed the following:

Proposal 1: Extend UE-side logging and related configurations to collect the relevant measurements and labels as configured. Details FFS for each use case.

Proposal 2: For NW-side deployments, RAN2 to consider the data collection configuration within MeasConfig IEs for AI mobility use cases.

Proposal 3: Consider the following mechanism to initiate the data logging for AI mobility use cases:

- Periodic logging: logging interval or periodicity can be a new RRC parameter.

- Event triggered logging: measurement Event A* or s-MeasureConfig threshold can be reused.

- Network indicated logging: an existing or dedicated DL indication signal can be used. Details FFS.

Proposal 4: For periodic logging, the minimum logging interval should align with the CSI-RS/SSB transmission periodicity.

Proposal 5: RAN2 to study different data prioritization levels for logging at the UE.

Proposal 6: Study the use of different buffer thresholds to enable data availability indication.

Proposal 7: Discuss the need of a common or different SRBs for network sided data collection to facilitate transmission of data with different priorities.

Proposal 8: Study whether the UE can retain a certain level of prioritized data during the handover process.

Proposal 9: For UE-side deployments, enhance the MeasConfig IEs for collecting data for AI mobility use cases.

Proposal 10: Identify and study potential inter-operability issues that arise when collecting data for UE-side models.

Proposal 11: RAN2 to consider an RRC connection set up specifically for measurement and event data collection.

# 4	Reference

[1] R2-2502594, Data Collection for Training of network-side ML models, RAN2 #129bis, Nokia, April 2025.