3GPP TSG-RAN2 Meeting #130	R2-2504287

St Julian’s, Malta, 19 – 23 May 2025	Revision of R2-2502909

Agenda Item:	6.1.3.3 Other (NR Rel-17 CP)

Source: 	Ericsson

Title:  	eDRX and emergency PDU session

Document for:	Discussion and Decision

# Introduction

In this contribution eDRX when there is an emergency PDU session is discussed further.

# Discussion

RAN2#129bis

RAN2 briefly discussed the topic [1-6], and o.a. it was mentioned that RAN3 was discussing a NOTE in 38.300. It was decided to postpone the topic and wait for RAN3 progress.

RAN3#127bis

RAN3 indicates in the reply LS [7]:

… RAN3 agrees that the NG-RAN is able to be aware of the presence of emergency PDU session when PDU session resources are established in the NG-RAN node based on the received special ARP value from CN …

RAN3 endorsed the following NOTE from Rel-17 in 38.300 [8, 9]:

NOTE:	If emergency PDU session resources are established, the gNB should not configure the Extended DRX when sending the UE to RRC_INACTIVE state. The gNB recognizes the emergency PDU session resources based on special ARP value as specified in TS 23.501 [3].

When there is an emergency call the gNB can decide to not configure eDRX for the UE in RRC_INACTIVE or keep the UE in RRC_CONNECTED until the emergency PDU session is released by AMF:

When the UE has an emergency call, then the gNB does not configure eDRX when the UE is suspended to RRC_INACTIVE or the gNB releases the emergency call until the AMF indicates that the emergency PDU session is released.

After the emergency call has been released in the upper layers, there can be an emergency call back, i.e. the emergency service centre requires further information from the caller that initiated the emergency call. An emergency call back is treated as a normal call in the network, i.e. there is no special ARP value used for emergency call back. To prevent that an emergency call back is delayed, the UE should not be operating in eDRX when there is a call back. The eDRX delay can be avoided by:

- The UE with an emergency call is not released to RRC_INACTIVE with eDRX configured, i.e. normal DRX is used for RAN paging.

- The UE in RRC_IDLE does not use eDRX when the UE has a emergency PDU session, see section 5.3.25 in 24.501. When there is CM-IDLE paging the AMF omits the eDRX info from the PAGING message to RAN, which causes the gNB to use normal DRX for paging.

- The gNB does not release the RRC connection with special ARP value until the emergency PDU session inactivity timer expires and the AMF sends UE CONTEXT RELEASE COMMAND.

From RAN2 perspective there is no need for further clarification concerning eDRX and emergency PDU session:

No further clarifications are needed for eDRX and emergency PDU session from RAN2 perspective.

# Summary

RAN2 is kindly asked to discuss eDRX and emergency PDU session:

Observation 1	When the UE has an emergency call, then the gNB does not configure eDRX when the UE is suspended to RRC_INACTIVE or the gNB releases the emergency call until the AMF indicates that the emergency PDU session is released.

Proposal 1	No further clarifications are needed for eDRX and emergency PDU session from RAN2 perspective.

# References

- R2-2501752, Reply LS on emergency call back and paging, SA2, LS in, Rel-17, To:RAN2, Cc:CT1, RAN3, RAN2#129-bis

- R2-2502337, On eDRX operation with emergency PDU session, ZTE, DISC, RAN2#129-bis

- R2-2502504, Discussion on emergency services and eDRX, Nokia, Huawei, DISC, RAN2#129-bis

- R2-2502909, Emergency callback and paging, Ericsson, DISC, RAN2#129-bis

- R2-2502976, Discussion on the LP-WUS handling for Emergency call back, NTT DOCOMO INC, DISC, RAN2#129-bis

- S2-2502428, Removing unnecessary restrictions for emergency PDU sessions, Qualcomm, CR 23.501, Rel-19, SA2#167

- R3-252402, Reply LS on emergency call back and paging, ZTE, LS out, Rel-17, To: SA2, RAN2, Cc: CT1, RAN3#127-bis

- R3-252400, Correction of Emergency Call and eDRX, Nokia, ZTE, Huawei, Ericsson, Qualcomm, CR 38.300, Rel-17, RAN3#127bis

- R3-252401, Correction of Emergency Call and eDRX, Nokia, ZTE, Huawei, Ericsson, Qualcomm, CR 38.300, Rel-18, RAN3#127bis