3GPP TSG-RAN WG2 #130	R2-2504294

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.10.3

Source:	Ericsson

Title:	Discussion on SON-MDT enhancements for Slicing and NTN

Document for:	Discussion/Decision

# Introduction

RAN#103 decided to support further data collection for Self-organizing Networks (SON). One of the topics that was endorsed was the SON and MDT enhancements for mobility in non-terrestrial networks and slicing.

# Discussion

## SON-MDT for Slicing

### Logging slice-based cell reselection information in logged MDT report

After RAN3#127 meeting, RAN3 has sent the following LS (R3-250613):

Overall description RAN3 has discussed how to optimize slice based cell reselection via e.g. logged MDT and identified the following scenario: A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell. RAN3 would like to ask RAN2 to take the above scenario into account, and to discuss whether and how it can be addressed and whether any standard changes are needed.

The LS comprises of a scenario where a UE is unable to find a suitable cell in the frequencies corresponding to the highest ranked NSAG and thereby has to camp on a cell belonging to the lower ranked NSAG or use legacy cell reselection policies to find a cell.

For example, assume the network supports slice A in frequency F1. Hence, the network may configure the NSAG priorities at the UE stipulating the UE to reselect to frequency F1. The UE, however, is unable to find a suitable cell in frequency F1 and is forced to re-select to a cell served by frequency F2. It should be noted that the UE selected a cell in frequency F2 using legacy cell reselection priorities.

From network perspective, this may happen only if there is a coverage hole at F1 in the location of the UE.

The scenario described in the LS may occur only if there is a coverage hole in the frequencies corresponding to the highest ranked NSAG.

In current logged MDT procedure, the UE logs measurements for up to 3 inter-frequency neighbour cell per frequency. Thus, not finding a suitable cell in a given frequency F1 may result in below different behaviours from the UE.

- UE may log measurements of some neighbour cells in the F1. However, the measurement results would show that the quantities are poor.

- UE may not log any information regarding F1, since it was unable to measure any cell in the frequency.

In both cases, however, network can deduce coverage problem in F1. Since the UE may also provide location information in the logged MDT report, network would be able to identify any random coverage hole also.

Moreover, in legacy, network can configure the UE with an interfrequencyTargetInfo IE to perform measurements in a specific frequency. Any poor coverage or coverage hole can be identified by the network using the measurement results. Hence, network may configure frequency F1 as part of interfrequencytargetInfo and UE would log measurements for cells found in frequency F1. Hence, any coverage gap in frequency F1 would be identified by the network.

- Legacy logged MDT measurements are sufficient for identifying coverage gaps in a given frequency.

Since the network is aware of the planned slices on a given frequency, it is capable to identify that there are some places with unavailable slices in that frequency and UE needs to use legacy cell reselection criterion to find a suitable cell.

On the other hand, NSAG specific frequency priorities are cell specific parameters and are transmitted using SIB16. UE accesses SIB16 to read such information and utilize that to determine priorities during cell reselection. Since in legacy logged MDT, UE reports the current serving cell information, it is trivial for the network to identify if any slice specific cell reselection is configured and if the UE is following such configuration.

- Legacy logged MDT measurements contains serving cell information of a given UE and network can identify if any slice specific cell reselection is configured in the cell. Hence, network can estimate which cell UE should re-select to continue to have access to a specific slice and whether the target cell provides sufficient coverage or not.

The above mechanism is schematically shown in the following figures.

Figure 1: UE camps on the serving frequency, according to SIB16 of the serving cell, finds F1 as highest priority frequency and attempt to find a suitable cell in F2 to camp on. The MDT report indicates that in F1 there is a full coverage over the current serving cell of the UE, so any slice specific cell reselection should be successful.

Figure 2: UE camps on the serving frequency, according to SIB16 of the serving cell, finds F1 as highest priority frequency and attempt to find a suitable cell in F2 to camp on. However, F1 does not provide the full coverage over the serving cell of the UE. Hence any slice specific cell reselection should fail in red area. This is already deducible from the current logged MDT report.

Hence, in our view, using the legacy logged MDT measurements and information from SIB16, it is trivial for the network to create a slice coverage map and identify any gaps; additional information from the UE is not required.

The scenario identified in RAN3 can be supported using legacy logged MDT procedure, no further enhancements are needed.

### Logging slice related information in SON reports

In RAN2#129bis meeting, proposals were made to include a list of NSSAI or NSAG in other SON reports, i.e., RA report or CEF report to indicate UE was unable to get access to those listed slices. In our view, inclusion of Slice information without additional information such as UE location, measurements are not useful.

- Inclusion of slice ID without additional information such as UE location and measurement results is not beneficial to the network.

However, such inclusion is not a trivial decision since different SON reports are generated under different conditions and contains different information. For example, RA reports does not include UE location information and associated measurements, whereas CEF report includes the UE location information. Hence, RAN2 would need to spend time to analyze different options before agreeing such solution. However, considering the time left in Rel-19, it would be beneficial to postpone such analysis to Rel-20.

RAN2 to agree that, due to the need for additional analysis to include slice information in SON report and time constraints in Rel-19, postpone such discussion to Rel-20.

Furthermore, RAN2 needs to send an LS reply to RAN3 including the decision.

RAN2 to agree on the attached LS reply to RAN3.

The drafted LS is attached in 5.1.

## SON-MDT for NTN

### MDT for NTN

During last RAN2 meeting, MDT for NTN was discussed based on the LS[2] from RAN3, and there are some FFS items left:

Introduce a UE capability to support geographic area scope checking. FFS if with or without signalling. Only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT. FFS if, when the UE is in IDLE/INACTIVE, the UE logs or not if the UE cannot obtain its location.

UE capability to support geographic area scope checking

The UE is not required to check its location in RRC_IDLE/RRC_INATIVE state. Based on the general principles and requirements guiding the definition of functions for MDT from TS37.320, the network may use UE capabilities to select terminals for MDT measurements. Therefore, the information on whether the UE supports geographic area scope checking is very useful for the NW to select terminals for MDT measurements and to decide whether to configure the geographical area scope for logged MDT for an NTN UE.

Introduce a UE capability of supporting geographic area scope checking with signaling.

There is another optional UE capability already defined for the UE in RRC_IDLE/RRC_INATIVE to support location based RRM measurements of neighbour cells in NTN. The reference from TS 38.306 is copied here:

## 5.6 RRM measurement features Definitions for feature. <omitted>. Location-based measurement initiation It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support location based RRM measurements of neighbour cells in NTN quasi-Earth fixed cell as specified in TS 38.304 [21].. Location-based measurement initiation for NTN Earth-moving cell It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support location based RRM measurements of neighbour cells in NTN Earth-moving cell as specified in TS 38.304 [21]..

The UE capability of location-based measurement initiation and geographic area scope checking are both regarding the UE’s location checking in RRC_IDLE/RRC_INATIVE. There is no need to separate these two capabilities from UE implementation point of view. For that reason, a single UE capability should be used for both location-based measurement initiation and geographic area scope checking. Moreover, a separate UE capability can be introduced for NTN quasi-Earth fixed cell and Earth-moving cell.

The current UE capability of Location-based measurement initiation and Location-based measurement initiation for NTN Earth-moving cell are optional features without UE capability parameter.

Introduce a single UE capability for location-based measurement initiation and MDT geographic area scope checking.

Introduce separate UE capabilities to support geographic area scope checking for NTN quasi-Earth-fixed and Earth-moving cell.

Logged MDT collection if the UE cannot obtain its location

In the last meeting, another FFS left is that if the UE cannot obtain its location, whether the UE should perform logged MDT measurement.

If the NW knows that the UE cannot support geographic area scope checking based on the UE capability parameter, then a reasonable NW implementation is to not configure the geographic area scope for this UE. One option is not to configure this UE with logged MDT. Another option is to configure this UE with logged MDT with a legacy area scope. Hence, to enable the use of MDT with those UEs not capable of geographic area scope checking, the UE should be configured with a legacy area scope and perform MDT logging according to the legacy area scope checking rule.

On the other hand, if the NW knows that UE can support geographic area scope checking based on the UE capability parameter, a possibility is that even though this UE supports geographic area scope checking, under certain situations, e.g. when this UE is with low energy (low battery level), it may stop location checking. Hence, the NW cannot know whether the UE can obtain its location or not even though this UE has indicated that it can support geographic area scope checking.

To thoroughly exploit the functionality of MDT, the UE should continue performing logged MDT collection, even if it cannot determine its location in some situations. To align with the behaviour of the UEs not capable of geographic area scope checking, a UE that supports geographic area scope checking should also be able to be configured with a legacy area scope together with the geographical area scope. When the UE cannot obtain its location, it continues to perform MDT logging according to the legacy area scope checking rule.

If the UE cannot obtain its location, it performs legacy area checking for logged MDT.

Regarding the UE that supports geographic area scope checking but has not obtain its location under some conditions, one problem is that the NW has no idea whether the MDT report is logged within the geographic area scope or within the legacy area scope. A simple solution is to let the UE to indicate for each MDT measurement sample, i.e. each entry included in logMeasInfoList, so the reported MDT is not polluted by MDT measurements collected from different area scope configurations. This indication is only 1 bit and can be present only when the UE supports geographic area scope checking but has not obtained its location.

Include an indication in each logged MDT measurement sample, i.e. in the LogMeasReport IE, if the UE has not obtained its location even though it supports geographic area scope checking.

Logged MDT collection if the UE can obtain its location

It was agreed that if UE can obtain location information in RRC_IDLE/RRC_INACTIVE state, the UE performs geographical area checking for logged MDT. However, it has not been clearly discussed what will happen when both geographical area scope and legacy area scope are configured. We think that as long as geographical area is configured, it means that the NW wants to have the logged MDT associated to that geographical area, hence, a geographical area configuration should have a higher priority than a legacy area configuration.

If the UE is configured with a geographical area scope and it can obtain location information, the UE performs geographical area checking for logged MDT, irrespective if a legacy area scope is configured or not. Geographical area has higher priority than legacy area scope.

### MRO for location-based CHO

In the RAN3 #127bis meeting, an LS [3] from RAN3 related to the MRO enhancement for location-based CHO was sent to RAN2:

RAN3 discussed the MRO enhancement for location-based CHO in NTN for earth moving scenario. In order to optimize the distance threshold for location-based CHO, RAN3 thinks it is beneficial to support either option provided below. - Option 1: to introduce the absolute UTC time when RLF/HOF happens, based on which the network computes the distance. - Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens. RAN3 would like to check with RAN2 which option to be adopted in the UE RLF report.

The LS from RAN3 only concerns location-based CHO in NTN for earth moving scenario.

Regarding option 1, with the introduction of the absolute UTC time when RLF/HOF happens and the already existing locationInfo logged in the RLF report, the network can compute the distances between the UE and the reference locations when RLF/HOF happens. Therefore, option1 can be used for the MRO enhancement for location-based CHO with no extra UE implementation effort. However, the absolute UTC time proposed in option 1 needs 39 bits, which will increase the message size of the RLF report.

Regarding option 2, it is to report the measured distances between the UE and the reference locations, which will disclose the detailed location of the UE, therefore, similar to locationInfo in the RLF report, the measured distances information also requires user consent. Besides, when location-based CHO is initiated by the UE, there is no requirement for the UE to continue measuring the distances between the UE and the reference locations, when HOF happens, the UE needs to measure the distances at the time of HOF in addition. In other words, option 2 will add new requirement for the UE to measure the distances to the reference locations at the time of RLF/HOF.

- Both options can solve the problem, however option 1 increases the RLF report message size and option 2 requires additional UE implementation effort.

To minimize option 1’s impact on the RLF report size and the size of the UEInformationResponse message when the RLF report is included, a relative time may be a better option than the absolute UTC time. As the locationInfo in the RLF report contains the UTC time of when the location was measured, a relative time between the time when the location was measured and the time when the RLF/HOF happened can be reported with less impact on the RLF report size. As an example, assuming a 10 ms time granularity and a maximum duration of 10,000 ms (maximum value of T304), the relative time can be represented using only 10 bits.

Introduce the relative time between the time when locationInfo was measured and the time when the RLF/HOF happens, based on which the network computes the distance.

A drafted LS with this response is attached in 5.2.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Legacy logged MDT measurements are sufficient for identifying coverage gaps in a given frequency.

Observation 2	Legacy logged MDT measurements contains serving cell information of a given UE and network can identify if any slice specific cell reselection is configured in the cell. Hence, network can estimate which cell UE should re-select to continue to have acces to a specific slice and whether the target cell provides sufficient coverage or not.

Observation 3	Inclusion of slice ID without additional information such as UE location and measurement results is not beneficial to the network.

Observation 4	Both options can solve the problem, however option 1 increases the RLF report message size and option 2 requires additional UE implementation effort.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The scenario described in the LS may occur only if there is a coverage hole in the frequencies corresponding to the highest ranked NSAG.

Proposal 2	The scenario identified in RAN3 can be supported using legacy logged MDT procedure, no further enhancements are needed.

Proposal 3	RAN2 to agree that, due to the need for additional analysis to include slice information in SON report and time constraints in Rel-19, postpone such discussion to Rel-20.

Proposal 4	RAN2 to agree on the attached LS reply to RAN3.

Proposal 5	Introduce a UE capability of supporting geographic area scope checking with signaling.

Proposal 6	Introduce a single UE capability for location-based measurement initiation and MDT geographic area scope checking.

Proposal 7	Introduce separate UE capabilities to support geographic area scope checking for NTN quasi-Earth-fixed and Earth-moving cell.

Proposal 8	If the UE cannot obtain its location, it performs legacy area checking for logged MDT.

Proposal 9	Include an indication in each logged MDT measurement sample, i.e. in the LogMeasReport IE, if the UE has not obtained its location even though it supports geographic area scope checking.

Proposal 10	If the UE is configured with a geographical area scope and it can obtain location information, the UE performs geographical area checking for logged MDT, irrespective if a legacy area scope is configured or not. Geographical area has higher priority than legacy area scope.

Proposal 11	Introduce the relative time between the time when locationInfo was measured and the time when the RLF/HOF happens, based on which the network computes the distance.

# References

RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, RAN#102, Edinburgh, Scotland, December 2023.

R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT.

R3-252382, LS on SONMDT for NTN.

# Annex

## Draft LS reply to RAN3

3GPP TSG-RAN WG2 Meeting #130	R2-250xxxx

St Julian’s, Malta, 19-23 May 2025

Title:	Reply LS on SON for network slicing

Response to:	R2-2501734(R3-250914)

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	TSG RAN WG2

To:	TSG RAN WG3

Cc:	-

Contact Person:

#### Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

In the LS R2-2501734/R3-250914, it mentions:

RAN3 has discussed how to optimize slice based cell reselection via e.g. logged MDT and identified the following scenario:

•	A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell.

RAN3 would like to ask RAN2 to take the above scenario into account, and to discuss whether and how it can be addressed and whether any standard changes are needed.

RAN2 made the following agreements:

RAN2 has analysed the scenario and has agreed that existing logged MDT measurement is sufficient for the scenario mentioned by RAN3.

Moreover, RAN2 thinks any additional enhancements to other SON reports from UE requires careful consideration and is not feasible to decide in Rel-19 given the time constraints.

2. Actions:

To RAN WG3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take account of the feedback on logged MDT and postpone further investigation of the SON reports to Rel-20.

3. Date of Next TSG-RAN WG2 Meeting:

3GPP RAN2#131	from 2025-08-26	to 2025-08-29		India

## Draft LS reply to RAN3

3GPP TSG-RAN WG2 #130	R2-25xxxxx

St Julian’s, Malta, 19-23 May 2025

Title:	[Draft] Reply LS on RAN3 agreements on SON/MDT for NTN

Response to:	R3-252382

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	TSG RAN WG2

To:	TSG RAN WG3

Cc:	N/A

Contact Person:

#### Name:

E-mail Address:

Attachments:	None

# 1	Overall description

RAN2 would like to thank RAN3 for the LS regarding MRO for location-based CHO in NTN.

RAN2 confirms that the UTC time when RLF/HOF happens can be adopted in the RLF report, based on which the network can compute the distance. The UTC time of the RLF/HOF is represented in the RLF report in the form of the relative time between the absolute time reported in locationInfo and the time of the RLF/HOF .

# 2	Actions

To RAN3

ACTION: 	RAN2 kindly requests RAN3 to take the above information into consideration.

# 3	Dates of next TSG WG2 meetings

TSG-RAN WG2 Meeting #131	25 - 29 Aug 2025	Bengaluru, India