3GPP TSG-WG2 Meeting # 130	R2-2504302

# Malta, MT,  19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0555 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for GNSS periodic AD request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Reason 1: Currently, it is possible to request periodically GNSS-Integrity-ServiceAlert which contains DNU flags as below: GNSS-Integrity-ServiceAlert-r17 ::= SEQUENCE { ionosphereDoNotUse-r17 BOOLEAN, troposphereDoNotUse-r17 BOOLEAN, ... The interpretation of DNU flags for Integrity service Alert is also dependent on the IE GNSS-RealTimeIntegrity and thus when periodic request for Integrity Service Alert is requested, it should be possible to also request GNSS-RealTimeIntegrity. NOTE 1: If GNSS integrity assistance data are provided (i.e., any of GNSS-Integrity-ServiceParameters, GNSS-Integrity-ServiceAlert, ORBIT-IntegrityParameters, SSR-IntegrityOrbitBounds, CLOCK-IntegrityParameters, SSR-IntegrityClockBounds, SSR-IntegrityCodeBiasBounds, SSR-IntegrityPhaseBiasBounds, STEC-IntegrityParameters, STEC-IntegrityErrorBounds, SSR-GriddedCorrectionIntegrityParameters, TropoDelayIntegrityErrorBounds) the following interpretation of the IE GNSS-RealTimeIntegrity applies: - Absence of the IE GNSS-RealTimeIntegrity indicates DNU=FALSE according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40] for all GNSS satellites for which integrity assistance data are provided. - Presence of the IE GNSS-RealTimeIntegrity for a GNSS satellite and signal combination indicates DNU=TRUE for this GNSS satellite and signal combination according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40]. Reason 2: GNSS-RealTimeIntegrity is only provided when there is bad signal. However as part of periodic assisatcne data, there can be ambiguity if NW does not provide periodically GNSS-RealTimeIntegrity; i.e from UE perspective it is ambiguous whether the bad signal situation has not changed or there is no more bad signal. Thus, it is essential that GNSS-RealTimeIntegrity is also part of periodic assistane data transfer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IE GNSS-RealTimeIntegrity is added for periodic request. Impacted functionality: GNSS Periodic AD Request Inter-operability: If the UE implements the feature and the NW does not; - NW may not provide the IE GNSS-RealTimeIntegrity periodically If the NW implements the feature and the UE does not; - UE cannot request for periodic transfer of the IE GNSS-RealTimeIntegrity which may hinder Integrity calculations/ interpretations especially when SV bad signal situation toggles. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear interpretation if GNSS-RealTimeIntegrity is not part of periodic data request/transfer. Risk of failing Integrity/Validation checks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Changes

#### 6.5.2.3	GNSS Assistance Data Request

#### –	A-GNSS-RequestAssistanceData

<Skipped Unmodified Changes>

#### –	GNSS-PeriodicAssistDataReq

The IE GNSS-PeriodicAssistDataReq is used by the target device to request periodic assistance data delivery from a location server.

-- ASN1START

GNSS-PeriodicAssistDataReq-r15 ::= SEQUENCE {

gnss-RTK-PeriodicObservationsReq-r15	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pOSR

glo-RTK-PeriodicBiasInformationReq-r15	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pCPB

gnss-RTK-MAC-PeriodicCorrectionDifferencesReq-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pMAC

gnss-RTK-PeriodicResidualsReq-r15		GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pRes

gnss-RTK-FKP-PeriodicGradientsReq-r15	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pFKP

gnss-SSR-PeriodicOrbitCorrectionsReq-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pOC

gnss-SSR-PeriodicClockCorrectionsReq-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pCC

gnss-SSR-PeriodicCodeBiasReq-r15		GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pCB

...,

[[

gnss-SSR-PeriodicURA-Req-r16			GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pURA

gnss-SSR-PeriodicPhaseBiasReq-r16		GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pPB

gnss-SSR-PeriodicSTEC-CorrectionReq-r16	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pSTEC

gnss-SSR-PeriodicGriddedCorrectionReq-r16

GNSS-PeriodicControlParam-r15	OPTIONAL  -- Cond pGrid

]],

[[

gnss-Integrity-PeriodicServiceAlertReq-r17

GNSS-PeriodicControlParam-r15	OPTIONAL  -- Cond pDNU

]],

[[

gnss-SSR-PeriodicOrbitCorrectionsSet2Req-r17

GNSS-PeriodicControlParam-r15	OPTIONAL,		-- Cond pOC2

gnss-SSR-PeriodicClockCorrectionsSet2Req-r17

GNSS-PeriodicControlParam-r15	OPTIONAL,		-- Cond pCC2

gnss-SSR-PeriodicURA-Set2Req-r17

GNSS-PeriodicControlParam-r15	OPTIONAL		-- Cond pURA2

]],

[[

gnss-PeriodicRealTimeIntegrityReq-r17

GNSS-PeriodicControlParam-r15	 OPTIONAL -- Cond pRTI

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| pOSR | The field is mandatory present if the target device requests periodic GNSS-RTK-Observations; otherwise it is not present. |
| pCPB | The field is mandatory present if the target device requests periodic GLO-RTK-BiasInformation; otherwise it is not present. |
| pMAC | The field is mandatory present if the target device requests periodic GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| pRes | The field is mandatory present if the target device requests periodic GNSS-RTK-Residuals; otherwise it is not present. |
| pFKP | The field is mandatory present if the target device requests periodic GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| pOC | The field is mandatory present if the target device requests periodic GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| pCC | The field is mandatory present if the target device requests periodic GNSS-SSR-ClockCorrections; otherwise it is not present. |
| pCB | The field is mandatory present if the target device requests periodic GNSS-SSR-CodeBias; otherwise it is not present. |
| pURA | The field is mandatory present if the target device requests periodic GNSS-SSR-URA; otherwise it is not present. |
| pPB | The field is mandatory present if the target device requests periodic GNSS-SSR-PhaseBias; otherwise it is not present. |
| pSTEC | The field is mandatory present if the target device requests periodic GNSS-SSR-STEC-Correction; otherwise it is not present. |
| pGrid | The field is mandatory present if the target device requests periodic GNSS-SSR-GriddedCorrection; otherwise it is not present. |
| pDNU | The field is mandatory present if the target device requests periodic GNSS-Integrity-ServiceAlert; otherwise it is not present. |
| pOC2 | The field is mandatory present if the target device requests periodic GNSS-SSR-OrbitCorrectionsSet2; otherwise it is not present. |
| pCC2 | The field is mandatory present if the target device requests periodic GNSS-SSR-ClockCorrectionsSet2; otherwise it is not present. |
| pURA2 | The field is mandatory present if the target device requests periodic GNSS-SSR-URA-Set2; otherwise it is not present. |
| pRTI | The field is mandatory present if the target device requests periodic GNSS-RealTimeIntegrity; otherwise it is not present. |

End of Changes