3GPP TSG-RAN WG2 Meeting #130	R2-2504306

# Malta, MT,  May 19th - 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0557 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE request for equalIntegerAmbiguityLevel assistance data [GNSS-EqualIntegerAmbiguity] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, CATT, Samsung, ZTE Corporation, Nokia, AT&T, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_pos-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-16 added assistance data support where Location server can indciate if two RTK Reference stations have the same equal interger ambiguity level. The UE should be able to request the RTK Reference station parameters assistance from the location server (seen more as an on demand/need basis). The assistance request for GNSS RTK reference station with equal Integer ambiguity level though is currently missing. The capability for UE to request the equalIntegerAmbiguityLevel info is beneficial when UE changes RTK reference station, especially since the network can only provide 16 Reference stations in the referenceStationList field. The UE should be able to request when NW should provision this assistance data similar to other fields of GNSS-RTK-ReferenceStation. The UE being able to indicate if it needs the equalIntegerAmbiguityLevel information or not may also help the Location server save the signaling bits if the assistance is provided only when requested by the UE. This field is beneficial when UE changes RTK reference station and thus UE can request when NW should provision this assistance data similar to other fields of GNSS-RTK-ReferenceStation which the UE can request. If the UE makes an assumption about the integer ambiguity that is incorrect, there is a risk of worsen performance during the reference station switch time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The request from UE to request for infiormation about equal Integer ambiguity is added. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: GNSS RTK Reference Station Information for Integer Ambiguity Level Inter-operability: - If NW implements the feature and UE does not - The UE will not be able to request Integer ambiguity level information and the UE may have to solely rely on network provided Integer ambiguity level information only. - If UE implements the feature and NW does not - The UE request for Integer ambiguity level information may fail as the NW cannot recognize the request from UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may make an incorrect assumption about the integer ambiguity level of a Reference station if the Reference station is not in the list of Reference stations provided in the assistance data. Signaling resource wastage, basic functionality from UE to be able to pull information from NW will be missing, feature incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Changes

### 6.5.2.4	GNSS Assistance Data Request Elements

### –	GNSS-ReferenceTimeReq

### <Skipped Unmodified Changes>

#### –	GNSS-RTK-ReferenceStationInfoReq

The IE GNSS-RTK-ReferenceStationInfoReq is used by the target device to request the GNSS-RTK-ReferenceStationInfo assistance from the location server.

-- ASN1START

GNSS-RTK-ReferenceStationInfoReq-r15 ::= SEQUENCE {

antennaDescriptionReq-r15			BOOLEAN,

antennaHeightReq-r15				BOOLEAN,

physicalReferenceStationReq-r15		BOOLEAN,

stationID-r15						GNSS-ReferenceStationID-r15		OPTIONAL,

...,

[[

equalIntegerAmbiguityLevelReq-r19	BOOLEAN

]]

}

-- ASN1STOP

GNSS-RTK-ReferenceStationInfoReq field descriptions

antennaDescriptionReq This field specifies whether or not the location server is requested to include the field AntennaDescription in the GNSS-RTK-ReferenceStationInfo IE. TRUE means requested.

antennaHeightReq This field specifies whether or not the location server is requested to include the field antennaHeight in the GNSS--RTK-ReferenceStationInfo IE. TRUE means requested.

equalIntegerAmbiguityLevelReq This field specifies whether or not the location server is requested to include the field equalIntegerAmbiguityLevel in the GNSS-RTK-ReferenceStationInfo IE. TRUE means requested.

physicalReferenceStationReq This field specifies whether or not the location server is requested to include the field physical-reference-station-info in the GNSS--RTK-ReferenceStationInfo IE. TRUE means requested.

stationID This field specifies the Station ID for which the GNSS-RTK-ReferenceStationInfo is requested.

End of Changes