3GPP TSG-RAN2 Meeting #130	R2-2504334

# St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:		8.17

Work Item:			IoT_NTN_TDD

Source:	Nordic Semiconductor ASA

Title:					On SI scheduling, postponing impacts, and early implementation of the IoT-NTN TDD mode

Document for:		Discussion and Decision

# Introduction

RAN2#129 started to discuss IoT-NTN TDD mode and had the following agreements:

Agreements:

- RAN2 will continue studying paging aspects based, on RAN1 progress

- RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

- RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

- RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated.

- RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

- RAN2 can continue the discussion also on RAR window

- Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

RAN2#129bis had the following agreements:

Agreements:

1.	Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

2.	In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

This paper discusses early implementation in IoT-NTN TDD mode, and SI scheduling principles for which there are no recorded RAN2 agreements yet for IoT-NTN TDD mode.

# SI scheduling

RAN1#120 had the following conclusion:

R1-2501614 Feature lead summary #3 on IoT-NTN TDD mode Moderator (Qualcomm Incorporated) Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

The [1][2] presented and discussed SIB1-NB scheduling in detail and proposed that in general there is no need to update the SIB1-NB scheduling. Used of additionalTransmissionSIB1-r15 is always possible (with a cost of reduced available “D NB-IoT subframes” for other purposes), which would help the acquisition of the SIB1-NB even further when 16 repetitions are configured. The SIB1-NB subframes that are scheduled on “non-D NB-IOT subframes” can plain be dropped while the UE can still receive SIB1-NB in maximum 2560ms time and typically based on our simulations only after few “D NB-IoT subframes” containing a SIB1-NB repetition.

Proposal-1: When operating in IoT-NTN TDD mode the scheduling mechanism of SIB1-NB remains unchanged.

Proposal-2: The SIB1-NB subframes that fall into “non-D NB-IOT subframes” are plain dropped.

A SIB2-NB and a SIB31-NB used in NTN context both are too large to fit into TBS of 120bits or less, thus a minimum of 208bits TBSes must be used. This implies 8 subframes are needed for transmission of SIB2-/SIB31-NB. A 90ms TDD frame has maximum 6 PSDCH and minimum 3 PDSCH “D NB-IoT subframes” available for other SI. With additionalTransmissionSIB1-r15 there are maximum 9 “D NB-IoT subframes” and without additionalTransmissionSIB1 10 “D NB-IoT subframes” PDSCH available in two consecutive TDD frames. This implies at least two 90ms TDD frames (and 160ms in a good case but to be on the safe side a longer si-WindowLength) are always needed to transfer another SI such as SIB2-NB. Other SI subframes that do not fit into one set of available “D NB-IoT subframes” or fall into “non-D NB-IOT subframes” would need to be postponed into the next 90ms TDD frame. The scheduling of other SI should follow the existing procedures but allow postponing the SI subframes that fall on “non-D NB-IOT subframes” next available subframe even if located on the next 90ms TDD frame within the SI-window.

Observation-1: A transmission of one “other SI” message always takes two 90ms TDD frames.

Observation-2: Postponing of other SI subframes to the next 90ms TDD frame is required.

Proposal-3: The scheduling procedure of other SI messages remains unchanged.

Proposal-4: Postponing of other SI subframes to the next 90ms TDD frame is supported.

To avoid further complicating the IoT-NTN TDD mode solution the network configuration should be such that SI repetitions within the SI window never overlap, and in a case SI transmission would overflow the SI-window the excess SI subframes are plain discarded.

Proposal-5: A network configuration shall ensure SI repetitions within a SI-window never overlap.

Proposal-6: Other SI transmission that overflows a configured SI-window shall be truncated.

# Early implementation of IoT-NTN TDD Mode

The IoT-NTN TDD mode can be seen and implemented as distinct feature on a basis of Rel-17 NB-IOT IoT-NTN. Its UE and network capability is implicitly associated with the used TDD bands (currently only B249) that a non-“IoT-NTN TDD mode” capable UE won’t ever attempt to search for nor camp on a cell on that band. Therefore, there is no need for the UE to explicitly signal its capability and the network to explicitly signal support for the IoT-NTN TDD mode.

Observation-3: The UE capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Observation-4: The network capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Although new bands are introduced in a specific 3GPP release, it is typical for UEs to add support for bands as soon as they become available even if the UE ASN.1 baseline is still on the earlier releases. The same practice would apply in the IoT-NTN TDD mode case. Furthermore, there is no UE capability ambiguity on the network side even if an IoT-NTN TDD mode compliant UE with advertised Rel-17 ASN.1 baseline signals support for Rel-19 an IoT-NTN TDD mode specific band (B249 in this case), since the entire feature is associated to a specific band.

Observation-5: There is no UE IoT-NTN TDD mode capability or band support ambiguity from the network point of view, since supporting UEs are only being served on specific IoT-NTN TDD mode bands.

The baseline functionality of the IoT-NTN TDD mode has no Rel-18 features that a compliant UE or a network would mandatorily or even optionally need to implement.

Observation-6: The Rel-19 IoT-NTN TDD mode can be implemented on top of Rel-17.

Proposal-7: Document in 3GPP TS36.331 Annex G that Rel-19 IoT-NTN TDD mode is early implementable feature starting from Rel-17 for a UE supporting a IoT-NTN TDD mode band.

# Conclusions

In this paper, we discussed IoT-NTN TDD mode SI scheduling impacts and H-SFN change impact with IDLE mode eDRX. Our observations and proposals are listed below:

SI Scheduling:

Observation-1: A transmission of one “other SI” message always takes two 90ms TDD frames.

Observation-2: Postponing of other SI subframes to the next 90ms TDD frame is required.

Proposal-1: When operating in IoT-NTN TDD mode the scheduling mechanism of SIB1-NB remains unchanged.

Proposal-2: The SIB1-NB subframes that fall into “non-D NB-IOT subframes” are plain dropped.

Proposal-3: The scheduling procedure of other SI messages remains unchanged.

Proposal-4: Postponing of other SI subframes to the next 90ms TDD frame is supported.

Proposal-5: A network configuration shall ensure SI repetitions within a SI-window never overlap.

Proposal-6: Other SI transmission that overflows a configured SI-window shall be truncated.

Early implementation of the IoT-NTN TDD mode:

Observation-3: The UE capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Observation-4: The network capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Observation-5: The Rel-19 IoT-NTN TDD mode can be implemented on top of Rel-17.

Observation-5: There is no UE IoT-NTN TDD mode capability or band support ambiguity from the network point of view, since supporting UEs are only being served on specific IoT-NTN TDD mode bands.

Observation-6: The Rel-19 IoT-NTN TDD mode can be implemented on top of Rel-17.

Proposal-7: Document in 3GPP TS36.331 Annex G that Rel-19 IoT-NTN TDD mode is early implementable feature starting from Rel-17.

# References

- R2-2500390	Discussion on IoT-NTN TDD mode	Iridium Satellite LLC	discussion	Rel-19

- R2-2501989	Discussion on IoT-NTN TDD mode	Iridium Satellite LLC	discussion	Rel-19