3GPP TSG-RAN WG2 Meeting #130	R2-2504342

# St. Julians, Malta, 19th – 23rd May 2025

Agenda Item:	8.7.5

Source:	Ericsson, Qualcomm Incorporated, ZTE Corporation, MediaTek Inc., Xiaomi

Title:	Discussion on UE Capabilities for Unnecessary Retransmission Avoidance

Document for:	Discussion, Decision

# 1	Introduction

On the topic of avoiding unnecessary retransmissions, a combined Tx-Rx solution was agreed. As the name suggest, this solution consists of aspects on both the Tx-side and Rx-side. But from an operating standpoint, these aspects are independent of each other with the Rx-side aspect being the essential part of the solution. This issue was also captured by the rapp in the list of open issues, as follows:

Open issue 2: whether to have separate or joint UE capability for UE Tx and Rx operations for unnecessary retransmission avoidance.

In this contribution, we discuss the corresponding UE capabilities and how these capabilities affect the co-existence of the features/aspects of a solution.

# 2	Discussion

Given all the solutions for RLC AM enhancements are clearly L2 functionality with no RAN1 impact, it is safe to assume that related capabilities will be per-UE with signalling.

- All the solutions for RLC AM enhancements should have per-UE capabilities with signalling.

The combined Tx-Rx solution however involves two aspects, where the Tx side stops transmissions of an outdated SDU, and the Rx-side abandons the outdated SDUs based on a local timer. Although the solution is based on both aspects, from an operating standpoint they are independent. In addition, the essential part of the solution is the Rx-side aspect used to abandon outdated SDUs.

- From an operation standpoint, the Tx-side and Rx-side aspects are independent with the Rx-side aspect being more essential.

Thus, bundling the two aspects into a single per-UE capability results in UE vendors having less flexibility in implementation and forces them to implement both aspects thereby delaying time to market (i.e., longer implementation and interoperability testing timeline). In general, aspects independent of each other as a matter of principle should not be bundled.

- Independent aspects of a solution should not be bundled into a single UE capability.

Hence, we propose that RAN2 defines two (optional) per-UE capabilities to cover the Tx-side and Rx-side aspects with the Tx-side capability conditional on the Rx-side capability. This allows vendors to stagger their implementation of this feature resulting in faster time to market by focusing on implementation of the essential aspect.

Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report.

Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. A UE supporting this feature shall also indicate the support of Rx-side aspect.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	All the solutions for RLC AM enhancements should have per-UE capabilities with signalling.

Observation 2	From an operation standpoint, the Tx-side and Rx-side aspects are independent with the Rx-side aspect being more essential.

Observation 3	Independent aspects of a solution should not be bundled into a single UE capability.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report.

Proposal 2	Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. A UE supporting this feature shall also indicate the support of Rx-side aspect.