3GPP TSG-RAN WG2 #130	R2-2504345

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.19.1

Source: 	Ericsson

Title:  	Scheduling Request (SR) to Random Access (RA) Fallback Enhancements

Document for:	Discussion and Decision

# Introduction

In situations of high load or congestion, when there are not enough resources available, the gNB might not be able to respond (with a grant) to all the scheduling requests (SRs) from the UEs in the cell. Although this high load or congestion might be transient, in the current SR procedure, the UEs would revert to the random access (RA) procedure, thereby releasing the corresponding PUCCH SR resources. This could result in traffic delays and latency spikes, especially for low latency services like time critical communication (TCC).

In this contribution, we discuss the issue with the current SR procedure which leads to early or unnecessary fallback to the RA procedure when there is temporary congestion or high loading at the network.

# Discussion

In the current SR procedure, if the SR_COUNTER exceeds the sr-TransMax i.e., the number of (failed) SR attempts is greater or equal to the maximum number of transmission attempts, then (as shown below [1]), the corresponding PUCCH SR resources are released, and the UE falls back to the contention-based RA procedure. In essence, the UE assumes that it has lost uplink (UL) synchronization with the network.

4> if SR_COUNTER < sr-TransMax: 5> instruct the physical layer to signal the SR on one valid PUCCH resource for SR; 5> if LBT failure indication is not received from lower layers: 6> increment SR_COUNTER by 1; 6> start the sr-ProhibitTimer. 5> else if lbt-FailureRecoveryConfig is not configured: 6> increment SR_COUNTER by 1. 4> else: 5> notify RRC to release PUCCH for all Serving Cells; 5> notify RRC to release SRS for all Serving Cells; 5> clear any configured downlink assignments and uplink grants; 5> clear any PUSCH resources for semi-persistent CSI reporting; 5> if rach-LessHO is not configured and if there is no ongoing RACH-less LTM cell switch: 6> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel all pending SRs.

In general, there are only a fixed number of UEs that can be scheduled in each scheduling instance, i.e., a fixed number of SRs can be processed in a scheduling instance.

Only a limited number of SRs can be processed in a scheduling instance.

However, there could be several reasons for a failed SR attempt, as listed below:

- The network is unable to decode the SR due to poor radio conditions.

- The UE has indeed lost UL synchronization with the network.

- gNB is highly loaded and cannot process/decode all the received SRs.

Considering the scenario in (3), in highly loaded scenarios or during congestion, it is likely that all UEs cannot be immediately scheduled and the period before they are scheduled is longer than the SR’s configured prohibit timer. This results in multiple SRs being triggered and worsening the loading situation (SR processing) in the gNB. In the meanwhile, UE’s repeated SR attempts keep incrementing the SR_COUNTER and finally exceeds sr-TransMax resulting in a RA procedure fallback. However, high loading or congestion at the gNB might be transient but based on the current SR procedure, the UE could the fallback to the RA procedure too early and unnecessarily.

Continuous SR attempts from UEs to a highly loaded or congested gNB results in an early or unnecessary fallback to the RA procedure. In a cyclic fashion, subsequent RA attempts would increase the load on the RACH and in turn gNB load/congestion.

Further, a fallback to the RA procedure may result in repeated requests until the network reconfigures the UEs with new SR resources. In such a highly loaded or congested system, this could happen for many UEs at the same time, reducing the overall performance and introducing time delays/latency spikes for the impacted UEs especially affecting low latency service such as TCC.

Reconfiguration of PUCCH SR resources for several UEs in a high loaded system will reduce the overall performance and introduce time delays/latency spikes.

A straightforward solution would be to configure a longer SR prohibit timer values to stop the UE from making continuous SR attempts and allow for the gNB to recover from the high load/congestion. However, always configuring a longer prohibit timer even when the gNB is not congested or experiencing high load results in unnecessary delays.

Always configuring a longer SR prohibit timer leads to unnecessary delays even when the gNB is not congested.

Especially, for low latency services such as TCC, it is important to configure a short SR prohibit timer to avoid such unnecessary delays. However, during congestion, a short SR prohibit timer will further increase the risk of reaching maximum number of SR attempts.

When the gNB is not congested, low latency services like TCC should be configured with a shorter prohibit timer to avoid unnecessary delays. This of course has a high risk of reaching the maximum SR attempts if the gNB experiences congestion.

As a result, we believe this issue of early/unnecessary fallback to the RA procedure with the current SR mechanism should be addressed especially to keep the low delays for delay-critical services such as TCC. Hence, we propose:

Address the issue in the current SR mechanism of early/unnecessary fallback to the RA procedure.

One solution would be to delay the fallback to the RA procedure, and this can be achieved by configuring an additional prohibit timer which can be configured to a value longer than the normal prohibit timer. Such that after a certain number of configured (failed) SR attempts, the UE can switch to this additional prohibit timer. By applying the additional prohibit timer and reducing the frequency of SR attempts, it allows the gNB to recover from congestion/high loading whilst allowing the UE to maintain its PUCCH SR resources. Thus, preventing early/unnecessary fallback to the RA procedure.

Delay the fallback to the RA procedure with an additional (longer) prohibit timer and a corresponding threshold (number of failed SR attempts) as trigger to use this timer, thereby reducing the frequency of SR attempts. This enables the gNB to recover from congestion/high loading whilst allowing the UE to maintain the PUCCH SR resources i.e., it prevents early/unnecessary fallback to the RA procedure.

As mentioned above, the condition for when the UE would switch to applying the additional prohibit timer can be based on a threshold of the number of failed SR attempts. The gNB can configure this value based on statistics obtained from the field such that under normal circumstances i.e., no congestion, the UE is unlikely to reach this threshold. Hence, gNB configuration can ensure the additional prohibit timer is applied under the correct circumstances.

Network configuration of the threshold of number of failed SR attempts can ensure that the second prohibit timer is applied only at congestion/high loading in the gNB.

Thus, we propose the following solution to address the issue of early/unnecessary fallback to the RA procedure in the current SR mechanism:

gNB can configure an additional prohibit timer along with a threshold for the number of failed SR attempts to apply this timer.

Once the UE has received a grant for scheduling UL/DL data, the UE can switch back to the configured sr-ProhibitTimer for the next SR attempts.

UE can revert to the configured sr-ProhibitTimer for the next SR attempts once the UE has received a grant for scheduling UL/DL data.

The following is the TP for the proposed change in the TS38.321, MAC specification:

4> if SR_COUNTER < sr-TransMax: 5> instruct the physical layer to signal the SR on one valid PUCCH resource for SR; 5> if LBT failure indication is not received from lower layers: 6> increment SR_COUNTER by 1; 6> If SR_COUNTER > sr-SwitchThres 7> start the sr-ProhibitTimer-additional 6> else: 7> start the sr-ProhibitTimer. 5> else if lbt-FailureRecoveryConfig is not configured: 6> increment SR_COUNTER by 1. 4> else: 5> notify RRC to release PUCCH for all Serving Cells; 5> notify RRC to release SRS for all Serving Cells; 5> clear any configured downlink assignments and uplink grants; 5> clear any PUSCH resources for semi-persistent CSI reporting; 5> if rach-LessHO is not configured and if there is no ongoing RACH-less LTM cell switch: 6> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel all pending SRs.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Only a limited number of SRs can be processed in a scheduling instance.

Observation 2	Continuous SR attempts from UEs to a highly loaded or congested gNB results in an early or unnecessary fallback to the RA procedure. In a cyclic fashion, subsequent RA attempts would increase the load on the RACH and in turn gNB load/congestion.

Observation 3	Reconfiguration of PUCCH SR resources for several UEs in a high loaded system will reduce the overall performance and introduce time delays/latency spikes.

Observation 4	Always configuring a longer SR prohibit timer leads to unnecessary delays even when the gNB is not congested.

Observation 5	When the gNB is not congested, low latency services like TCC should be configured with a shorter prohibit timer to avoid unnecessary delays. This of course has a high risk of reaching the maximum SR attempts if the gNB experiences congestion.

Observation 6	Delay the fallback to the RA procedure with an additional (longer) prohibit timer and a corresponding threshold (number of failed SR attempts) as trigger to use this timer, thereby reducing the frequency of SR attempts. This enables the gNB to recover from congestion/high loading whilst allowing the UE to maintain the PUCCH SR resources i.e., it prevents early/unnecessary fallback to the RA procedure.

Observation 7	Network configuration of the threshold of number of failed SR attempts can ensure that the second prohibit timer is applied only at congestion/high loading in the gNB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Address the issue in the current SR mechanism of early/unnecessary fallback to the RA procedure.

Proposal 2	gNB can configure an additional prohibit timer along with a threshold for the number of failed SR attempts to apply this timer.

Proposal 3	UE can revert to the configured sr-ProhibitTimer for the next SR attempts once the UE has received a grant for scheduling UL/DL data.

# References

3GPP TS 38.321, V18.4.0 (2024-12)