3GPP TSG-RAN WG2 Meeting #130       	                      R2-2504376

Malta, MT, 19th-23rd May 2025

Agenda item:	8.1.2.2

Source:	CMCC

Title:	Discussion on LCM for UE-sided model for BM

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed LCM for UE-sided model for BM and achieved the following agreements.

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior Agreements on applicability reporting 1 Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec 2 Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. 3 No prohibit timer is introduced Agreements on data collection configuration 1 The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. 2 Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

This contribution will discuss the remaining issues on UE-sided model LCM for beam management use case.

- Discussion

- 2.1 Applicable functionality reporting

- Open issue RRC-1: Cause of inapplicability

- In last meeting, RAN2 agreed that UE further indicates a simple cause value of inapplicability, FFS how to define this simple cause related to model availability and how we capture it in the spec. During the discussion, some companies proposed to differentiate temporary or permanent inapplicability. In our understanding, if the model is permanently unavailable, the UE should not report that the functionality is supported in UE capability reporting. Thus, the model can only be temporarily unavailable. Therefore, a simple cause indicating whether model is available or not is enough.

- Proposal 1: Introduce a simple cause indicating whether model is available or not.

- Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

- RAN2 agreed that if option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- The follow-up issue is that if the non-applicable functionality becomes applicable and the inference configuration for the functionality is provided in step 3, whether the UE can autonomously activate the applicable functionalities upon reporting applicable functionalities via UEAssistanceInformation in step 6 for periodic CSI reporting.

- For example, the network provides three CSI-ReportConfig for functionality#1,#2,#3 for periodic CSI reporting in step 3, the UE reports applicability (#1) and inapplicability (#2 and #3) in step 4 and activates functionality#1 autonomously. After a while, functionality#2 becomes applicable, and UE reports it via UAI in step 6. Whether functionality#2 can be activated autonomously upon the reporting via UAI needs to be clarified.

- Fig 1. Basic flow for option A

- From the perspective of operator, we don’t expect the UE autonomously activate the applicable functionalities upon the subsequent applicable functionalities reporting via UEAssistanceInformation message for periodic CSI reporting.

- On the one hand, UAI is just for information for network in legacy mechanism, and there is no UE autonomous behaviour after transmitting UAI message.

- On the other hand, for non-applicable functionality reported in step 4, the network may release the reserved resources after step 4 due to efficiency. That means the inference configuration received in step 3 may be unavailable when the functionality becomes applicable from non-applicable. We think the reasonable solution is that after the UE reports updated applicability functionality via UAI, the network can provide the inference configuration via RRCReconfiguration message.

- Proposal 2: If option A is configured in Step 3, when the non-applicable functionality becomes applicable, the UE cannot autonomously activate the applicable functionalities for periodic CSI reporting upon reporting functionality applicability update via UAI.

- For option B, the CSI-ReportConifg (i.e. inference configuration) included in RRCReconfiguration is provided based on the applicability/inapplicability reporting in step 4, a smart network cannot provide the inference configuration for non-applicable functionality. Therefore, there will be no issue similar to option A mentioned above.

- 2.2 Data collection

- Open issue RRC-3: UE data collection request

- In last meeting, RAN2 agreed that the UE can send UAI to indicate its preferred data collection configuration from a list of candidate configurations provided by NW and details of signaling are FFS. In our understanding, the candidate configuration can be provided via RRCReconfiguration message based on UE capability, and may include such as RS type, resource, mapping for set A/set B.

- Proposal 3: The candidate configuration can be provided via RRCReconfiguration message based on UE capability, and may include such as RS type and resource for set A/set B.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

- In previous meeting, there was a FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

- In our understanding, although RAN2 has no agreements on the details of data collection configuration, it is provided to the UE by RRCReconfiguration message since the UE stays in RRC_CONNECTED state. As specified in TS 38.331 [2], if the UE cannot comply with RRCReconfiguration, the UE shall continue using the configuration used prior to the reception of RRCReconfiguration message or perform the actions upon going to RRC_IDLE or initiate the connection re-establishment procedure.

- 5.3.5.8 Reconfiguration Failure - 5.3.5.8.2 Inability to comply with RRCReconfiguration The UE shall: - <omitted> 1> else if RRCReconfiguration is received via NR (i.e., NR standalone, NE-DC, or NR-DC): - <omitted> 2> else if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over the SRB1 or if the upper layers indicate that the nas-Container is invalid: NOTE 0a: The compliance also covers the SCG configuration carried within octet strings e.g. field mrdc SecondaryCellGroupConfig. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded SCG configuration or with the combination of (parts of) the MCG and SCG configurations. NOTE 0b: The compliance also covers the V2X sidelink configuration carried within an octet string, e.g. field sl ConfigDedicatedEUTRA. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded V2X sidelink configuration. 3> if the RRCReconfiguration message was received as part of ConditionalReconfiguration; or, 3> if the RRCReconfiguration message was received as part of ltm-Config: 4> continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected; 3> else: 4> continue using the configuration used prior to the reception of RRCReconfiguration message; 3> if AS security has not been activated: 4> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other' 3> else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, have not been setup: 4> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'; 3> else: 4> initiate the connection re-establishment procedure as specified in 5.3.7, upon which the reconfiguration procedure ends; - <omitted>

- For data collection for model training, we think it is a separate procedure from other UE behaviors in connected mode. If the UE cannot perform data collection based on received configuration, there is no need to initiate the connection re-establishment procedure or perform the actions upon going to RRC_IDLE, and the other UE behaviors should not be affected and should be performed as usual.

- Proposal 4: When the UE can’t perform data collection for model training based on received configuration, other UE behaviors in connected mode should not be affected.

- However, we think the network needs to know when the UE cannot perform data collection, which is similar as the discussion in NW-side data collection that the UE cannot perform data collection due to low power or full buffer. Based on the UE reporting, the network can reconfigure the UE to release the data collection configuration. Therefore, an indication is needed from UE to network to indicate that the UE cannot perform data collection based on received configuration. Furthermore, the indication can be included in RRCReconfigurationComplete message. Another option is to send the indication via UAI, but it is optional for the UE whether and when to send UAI, which cannot let NW know in a timely manner.

- Proposal 5: An indication is introduced in RRCReconfigurationComplete message when UE cannot perform data collection based on received configuration.

- 2.3 Performance monitoring

- For performance monitoring for beam management, RAN1 discussed different monitoring type, and had some agreements on Type 1. Given that only one WG meeting left for RAN1 to conclude this WI, we think there is no time to address Type 2. Therefore, RAN2 should focus on Type 1 and discuss whether there is any impact on RAN2.

- Type 1 performance monitoring:

  - Option 1 (NW-side performance monitoring):

    - UE sends a report to NW (for the calculation of performance metric at NW)

      - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report

      - FFS on other contents

    - The report is at least configured/triggered by NW

  - Option 2 (UE-assisted performance monitoring):

    - UE calculates performance metric(s)

- FFS Type 2 performance monitoring

- Observation 1: There is no time to address Type 2 in RAN1 considering only one WG meeting left to conclude this WI, so RAN2 should focus on Type 1 and discuss whether there is any impact on RAN2.

- For Type 1 Option 1, UE sends a report to NW which includes at least measurement results (e.g. L1-RSRP), and the report is configured/triggered by NW. In our understanding, the L1 measurement results will be reported via L1 signaling which is similar to inference.

- For Type 1 Option 2, RAN1#119 meeting agreed that dedicated resource set(s) and report configuration for monitoring are configured in a dedicated CSI report configuration, which means the legacy CSI framework will be reused for monitoring configuration and reporting.

RAN1#119 Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

- Observation 2: For performance monitoring Type 1 Option 1 and Option 2, the legacy CSI framework is reused for monitoring configuration and reporting.

- Based on the above analysis, we think there is no further RAN2 impact except ASN.1 impact for now.

- Proposal 6: For performance monitoring Type 1 Option 1 and Option 2, there is no additional RAN2 impact apart from ASN.1 impact, based on the current progress in RAN1.

# 3	Conclusion

Here are the observations and proposals for LCM for UE-sided model for beam management.

- Open issue RRC-1: Cause of inapplicability

- Proposal 1: Introduce a simple cause indicating whether model is available or not.

- Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

- Proposal 2: If option A is configured in Step 3, when the non-applicable functionality becomes applicable, the UE cannot autonomously activate the applicable functionalities for periodic CSI reporting upon reporting functionality applicability update via UAI.

- Open issue RRC-3: UE data collection request

- Proposal 3: The candidate configuration can be provided via RRCReconfiguration message based on UE capability, and may include such as RS type and resource for set A/set B.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

- Proposal 4: When the UE can’t perform data collection for model training based on received configuration, other UE behaviors in connected mode should not be affected.

- Proposal 5: An indication is introduced in RRCReconfigurationComplete message when UE can’t perform data collection based on received configuration.

Performance monitoring

- Observation 1: There is no time to address Type 2 in RAN1 considering only one WG meeting left to conclude this WI, so RAN2 should focus on Type 1 and discuss whether there is any impact on RAN2.

- Observation 2: For performance monitoring Type 1 Option 1 and Option 2, the legacy CSI framework is reused for monitoring configuration and reporting.

- Proposal 6: For performance monitoring Type 1 Option 1 and Option 2, there is no additional RAN2 impact apart from ASN.1 impact, based on the current progress in RAN1.

# 4	Reference

- Draft_RAN2_129bis_Meeting_Report_v3

- TS 38.331 NR; Radio Resource Control (RRC) Protocol Specification