3GPP TSG-RAN WG2 Meeting #130	R2-2504389

St.Julians, Malta,  May 19th – 23rd , 2025

# Agenda item:	8.6.2

# Source:	CMCC

# Title:	Discussion on inter-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#129bis meeting, RAN2 reached the following agreements on inter-CU LTM [1]:

Agreements on inter-CU LTM

- A list of sk-counters is linked to a Rel-19 ID configured by the SN.

- At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

- How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

- The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

- Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

- An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

In this paper, we further discuss the open issues of inter-CU LTM.

- Discussion

Inter-CU LTM Fast recovery

In last meeting, RAN2 agreed the security key information updating mechanism for inter-CU LTM case, that NCC value is indicated in LTM cell switch command. And based on this, RAN2 discussed whether and how to support fast recovery mechanism in inter-CU LTM scenario. However, only fast recovery to the candidate cell with the same Rel-19 ID with the source cell for RLF case or to the candidate cell with the same Rel-19 ID with the target cell for reconfiguration with sync failure case is supported, while whether fast failure recovery with different Rel-19 IDs for reconfiguration with sync failure case is allowed is still FFS.

For the RLF case, UE has the serving/source cell configuration, therefore, if the reselected cell has the same Rel-19 ID with the serving cell, no security updating is needed, fast recovery can be supported.

For the reconfiguration with sync failure case, which means T304 expires, the UE should revert back to the serving cell’s configuration, also including the NCC value. As to the NCC value indicated to candidate cells, in our opinion, the inter-CU LTM procedure is similar to CHO, therefore the NCC value should be forward by the source gNB in  { KNG-RAN*, NCC} pair to the target gNB in LTM preparation phase, and KNG-RAN* is derived based on source cell’s unused NH value. What’s more, there’s only one unused stored {NH, NCC} pair in the source cell, hence, the NCC value in different target cell is the same.

Observation 1: The same NCC value is indicated to candidate cells, since there’s only one unused stored {NH, NCC} pair in the source cell.

In CHO, fast recovery can be perform to any candidate belonged to the different gNBs from the target cell. Now that fast recovery is supported in the case that the selected candidate cell has the same Rel-19 ID as target, and the UE revserts back to its source cell’s configuration, we think fast recovery to candidate cell with different Rel-19 ID with target cell, can also be supported. The UE can identify whether security updating is needed based on Rel-19 IDs of reselected candidate cell and the source cell, and derive the key based on the NCC value related NH and other needed information.

Proposal 1: Fast failure recovery with different Rel-19 IDs can be supported.

What’s more, there’s the other open issue related to fast recovery in the email discussion of RRC Running CR[2]:

## 2.6 RRC-6 Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration. Rapporteur comment: It is still not crystal clear how the fast recovery procedure would work, and especially when the UE should revert back to the previous PCell configuration. We suggest to discuss this based on companies’ contribution.

The Editor’s Note is about UE’s behavior when T304 expires. Generally, if T304 expires, the UE should revert back to the Pcell configuration. And in Rel-18 LTM discussion, since it’s intra-CU case and PDCP is not re-established, therefore, the UE reverts back to configuration used in the source Pcell except for the PDCP state variable.

In last meeting ,inter-CU fast recovery mechanism is supported if UE’s reselect cell has the same Rel-19 ID with the source cell or the target cell. And descriptions related to the failure of fast recovery to the cell with the same Rel-19 ID of target cell is proposed by RRC spec editor to specify the time point of reverting configuration. Except for LTM, fast recovery mechanism can also be configured for CHO candidates, but no specific UE behavior  for CHO recovery failure is specified. When it comes to inter-CU case that UE should perform PDCP re-establishment, we think the UE can just revert back to the PCell’s configuration when T304 expires.

Proposal 2: For LTM fast recovery to the candidate cell requiring security key updating, UE should revert back to the PCell’s configuration if T304 expires.

CSI-RS resources for  RACH-based LTM

In Rel-18, only SSB based L1 measurement is supported for LTM, and only SSB based CFRA resources can be indicated in LTM cell switch command. However, in legacy handover, CSI-RS resource can be configured in rach-configdedicated. Considering that CSI-RS based measurement is supported in Rel-19 LTM, from RAN2 perspective, CSI-RS resource can also be considered to be indicated in LTM cell switch command, and it can be supported for both intra-CU LTM and inter-CU LTM. Considering the potential impact to RAN1, we can revisit it when there’s progress in RAN1.

Proposal 3: From RAN2 perspective, CSI-RS resources can be indicated in LTM cell switch command for RACH-based intra-CU/inter-CU LTM.

Inter-CU SCG LTM execution

In Rel-18, only intra-CU LTM is supported, and UE include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate in RRCReconfigurationComplete message and sends the message to the node providing LTM configuration, i.e., MN or SN only. When it comes to inter-CU SCG LTM, one open issue is if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message. Considering that in inter-CU SCG LTM, it’s MN to provide inter-CU LTM configuration including the LTM-CandidateId, therefore, we think appliedLTM-CandidateId is only needed to be included in MCG RRCReconfigurationComplete message. And the MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message as captured in TS 37.340 Running CR.

Observation 3: The content of appliedLTM-CandidateId is LTM-CandidateId, which is configured by MN in inter-CU SCG LTM.

Proposal 4: appliedLTM-CandidateId is only included in MCG RRCReconfigurationComplete message.

Coexistence of inter-CU SCG LTM and L3 SN handling

It was agreed in RAN2#127bis meeting that L3 mobility can be configured to the UE with inter-CU LTM configured. Following are the related agreements with regard to the coexistence of  L3 mobility and inter-CU MCG LTM [3]:

9. L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied.

Therefore, when it comes to SCG LTM, the coexistence of inter-CU SCG LTM and L3 SCG handling like SN change/PsCell change/(S-)CPAC should also be discussed. And we think the similar principle can be applied, including:

- Network triggered SN change/PsCell change and (S-)CPAC can be configured to UE, when (SN initiated) inter-CU SCG LTM is configured.

  - When performing network triggered SN change/PsCell change or (S-)CPAC, the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

  - The RRCReconfiguration message to execute an SN change/PsCell change/(S-)CPAC procedure may reconfigure inter-CU LTM configurations.

  - For the execution order between (S-)CPAC and inter-CU SCG LTM, similar principle for CHO and inter-CU MCG LTM is reused.

Proposal 5: Network triggered SN change/PsCell change and (S-)CPAC can be configured to UE, when (SN initiated) inter-CU SCG LTM is configured.

-	When performing network triggered SN change/PsCell change or (S-)CPAC, the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

-	The RRCReconfiguration message to execute an SN change/PsCell change/(S-)CPAC procedure may reconfigure inter-CU LTM configurations.

-	For the execution order between (S-)CPAC and inter-CU SCG LTM, similar principle for CHO and inter-CU MCG LTM is reused.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation: The same NCC value is shared among different CUs/gNBs in CHO.

Observation 2: CHO candidate cells are invisible to each other, while for LTM, candidate cells are visible to each other, which may result in high probability of security exposure.

Observation 3: The content of appliedLTM-CandidateId is LTM-CandidateId, which is configured by MN in inter-CU SCG LTM.

Proposals:

Proposal 1: Fast failure recovery with different Rel-19 IDs can be supported.

Proposal 2: For LTM fast recovery to the candidate cell requiring security key updating, UE should revert back to the PCell’s configuration if T304 expires.

Proposal 3: CSI-RS resources can be indicated in LTM cell switch command for RACH-based intra-CU/inter-CU LTM.

Proposal 4: appliedLTM-CandidateId is only included in MCG RRCReconfigurationComplete message.

Proposal 5: Network triggered SN change/PsCell change and (S-)CPAC can be configured to UE, when (SN initiated) inter-CU SCG LTM is configured.

-	When performing network triggered SN change/PsCell change or (S-)CPAC, the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

-	The RRCReconfiguration message to execute an SN change/PsCell change/(S-)CPAC procedure may reconfigure inter-CU LTM configurations.

-	For the execution order between (S-)CPAC and inter-CU SCG LTM, similar principle for CHO and inter-CU MCG LTM is reused.

- References

- Report of 3GPP TSG RAN WG2 meeting #129bis;

- List of RRC FFSs for mobility

- Report of 3GPP TSG RAN WG2 meeting #127bis;