3GPP TSG RAN WG2 Meeting #130	R2-2504394

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.9.4

Source:	CMCC

Title:	Support of PWS messages for NB-IoT

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of PWS for NB-IoT and some agreements were achieved as follows [1]:

Agreements:

1.	In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

In this contribution, we would like to provide some further considerations on the remaining issues for PWS for NB-IoT.

Discussion

ETWS

In NR NTN, it is agreed to include warningAreaCoordinates in SIB6 and warningAreaCoordinatesSegment in SIB7 to indicate Warning Area Coordinates IE for ETWS. And in RAN2 #129 meeting, we agreed to provide warning area coordinates for ETWS for both NR NTN and NB-IoT NTN without segmentation for primary notification. To keep alignment between NR NTN and IoT-NTN, warningAreaCoordinates and warningAreaCoordinatesSegment should be also included in SIB10-NB and SIB11-NB respectively.

Observation 1: In NR NTN, it is agreed to include warningAreaCoordinates in SIB6 and warningAreaCoordinatesSegment in SIB7 to indicate Warning Area Coordinates IE for ETWS.

Proposal 1: It is proposed that warningAreaCoordinates and warningAreaCoordinatesSegment should be also included in SIB10-NB and SIB11-NB respectively.

SIB1-NB acquisition

In RAN2 #128 meeting, we have agreed to extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. For whether support SIB1-NB acquisition is FFS.

Per the existing LTE mechanism, the UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, however, if etws-Indication or cmas-Indication is included in the Paging message, it would trigger the SystemInformationBlockType1 re-acquisition immediately, i.e., without waiting until the next system information modification period boundary. Then UE read the schedulingInfoList to acquire the System Information Block carrying specific PWS message(SystemInformationBlockType10 and SystemInformationBlockType11 for ETWS, SystemInformationBlockType12 for CMAS) if present.

- 5.2.1.4 Indication of ETWS notification <<omitted>> NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the etws-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType10 and SystemInformationBlockType11. The UE may or may not receive a Paging message including the etws-Indication and/or systemInfoModification when ETWS is no longer scheduled. <<omitted>> - 5.2.1.5 Indication of CMAS notification <<omitted>> NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the cmas-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType12. The UE may or may not receive a Paging message including the cmas-Indication and/or systemInfoModification when SystemInformationBlockType12 is no longer scheduled. <<omitted>>

- 5.3.2.3 Reception of the Paging message by the UE <<omitted>> if the etws-Indication is included and the UE is ETWS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary; 2> if the schedulingInfoList indicates that SystemInformationBlockType10 is present: 3> acquire SystemInformationBlockType10; NOTE 2: If the UE is in CE, it is up to UE implementation when to start acquiring SystemInformationBlockType10. 2> if the schedulingInfoList indicates that SystemInformationBlockType11 is present: 3> acquire SystemInformationBlockType11; 1> if the cmas-Indication is included and the UE is CMAS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5; 2> if the schedulingInfoList indicates that SystemInformationBlockType12 is present: 3> acquire SystemInformationBlockType12; <<omitted>>

Some companies provide enhancements solutions that network broadcasts early PWS scheduling message even though there is no PWS notification considering the latency. However, such enhancements may introduce some additional signaling overhead. Therefore, we could just extend the existing RRC procedure to NB-IoT NTN for SIB1-NB acquisition, and no further enhancement is needed.

Proposal 2: We could just extend the existing mechanism to NB-IoT NTN for SIB1-NB acquisition, and no further enhancement is needed.

PWS reception during mobility

There is another open issue that whether allow UE to receive and assemble PWS segments from different cells during mobility. Per the existing specification, for UE is ETWS/CMAS capable, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment, it should discard any previously buffered warningMessageSegment and clear, if any, the current values of messageIdentifier and serialNumber. However, considering that for NTN system, satellites are always moving rapidly, which would lead to that UE may be not able to receive the whole PWS message from the serving cell. And from UE side, it is expected to obtain the whole PWS message as soon as possible. Therefore it is beneficial to enable UE to receive and assemble PWS segments from different cells.

Observation 2: Different from TN, for NTN system, satellites are always moving rapidly, which would lead to that UE may be not able to receive the whole PWS message from the serving cell.

Proposal 3: It is proposed that allow UE to receive and assemble PWS segments from different cells during mobility.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on broadcast of PWS messages for NB-IoT and have the following observations and proposals:

Observation 1: In NR NTN, it is agreed to include warningAreaCoordinates in SIB6 and warningAreaCoordinatesSegment in SIB7 to indicate Warning Area Coordinates IE for ETWS.

Proposal 1: It is proposed that warningAreaCoordinates and warningAreaCoordinatesSegment should be also included in SIB10-NB and SIB11-NB respectively.

Proposal 2: We could just extend the existing mechanism to NB-IoT NTN for SIB1-NB acquisition, and no further enhancement is needed.

Observation 2: Different from TN, for NTN system, satellites are always moving rapidly, which would lead to that UE may be not able to receive the whole PWS message from the serving cell.

Proposal 3: It is proposed that allow UE to receive and assemble PWS segments from different cells during mobility.

Reference

- R2-129bis NR-NTN-IoT-NTN (Sergio)_ EOM

- RP-250472 WID Rev R19 IOT-NTN cl

- 3GPP TS 36.300 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18)

- 3GPP TS 36.331 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)