3GPP TSG-WG2 Meeting #130                                                                     R2-2504400

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda item:	8.8.2

Title:	Analysis on DL coverage enhancements due to extended SSB periodicity

Source:	CMCC

Document for:	Discussion

- Introduction

In previous RAN2 129bis meetings, the following statements on DL coverage enhancements have been agreed with some remaining issues:

- Agreements:

- From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

- We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

- RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

- We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

- Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In this contribution, we will further provide our views on the remaining issues of DL coverage enhancements, including: SMTC enhancements, Access control for DL CE and Link level coverage enhancements.

- Discussion

2.1 SMTC enhancements

2.2.1 Signaling process for SMTC configuration

RAN2 observes that if the cells active simultaneously are scattered as shown in Figure 1, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

Figure 1 the cells active simultaneously are scattered

- In addition, RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame. RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) as shown in Figure 2. Based on the coverage scenarios of multiple SSB beams in different spatial directions and the application of “beam hopping”, UE needs to know the transmitted SSB index and other assistance information in order to improve measurement accuracy and efficiency.

Figure 2 beam hopping applies equally to all the satellite beams of a given cell

To solve the above problems, RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode and support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. RAN2 considers to introduce signaling support for different SMTC periodicity and offset in the same frequency layer, for both idle , inactive and connected mode.

According to the spec [4], the existing SMTC mechanism  is as follows:

The UE shall setup the first SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicityAndOffset parameter (providing Periodicity and Offset value for the following condition) in the SSB-MTC configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = (FLOOR (Offset/10));

if the Periodicity is larger than sf5:

subframe = Offset mod 10;

else:

subframe = Offset or (Offset +5);

with T = CEIL(Periodicity/10).

If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received offset parameter in each SSB-MTC4 configuration and use the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

–	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements and/or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

MeasObjectNR ::=                SEQUENCE {

ssbFrequency                      ARFCN-ValueNR             OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing              SubcarrierSpacing         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                             SSB-MTC                   OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                             SSB-MTC2                  OPTIONAL,   -- Cond IntraFreqConnected

...

smtc4list-r17                     SSB-MTC4List-r17          OPTIONAL,   -- Need R

...

}

...

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

–	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

...

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

Observation 1: Each SSB-MTC4 element of the smtc4list in the same MeasObjectNR uses the same duration and periodicity parameter (derived from parameter periodicityAndOffset) from the smtc1 configuration, not supporting two  periodicities.

Observation 2:  The IE smtc1 and smtc4list-r17 in the same MeasObjectNR can configure at most 4 SMTC occasions.

For supporting different SSB periodicity among neighbour cells in the same frequency layer, the SMTC configuration needs to be modified to allow multiple periodicity configurations with different offset. Based on the 129bis agreement, legacy UE will not be barred from DL CE cell, so NW need to configure SMTC for both legacy UE and R19 DL CE capable UE. From the perspective of minimizing the impact on legacy UE and providing more flexibility, it is preferable to introduce new SMTC information element (SMTC5) with backwards compatible manner.

Proposal 1: RAN2 introduce new SMTC information element (smtc5list-r19) in MeasObjectNR for R19 DL CE capable UE.

Proposal 2: RAN2 assume maximum number of SMTCs that need to be supported is 6, wherein the maximum sequence size of smtc5list-r19 is 5 for supporting 6 SMTCs per frequency for idle/inactive UEs.

MeasObjectNR information element

MeasObjectNR ::=                SEQUENCE {

ssbFrequency                      ARFCN-ValueNR             OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing              SubcarrierSpacing         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                             SSB-MTC                   OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                             SSB-MTC2                  OPTIONAL,   -- Cond IntraFreqConnected

...

smtc4list-r17                     SSB-MTC4List-r17          OPTIONAL,   -- Need R

...,

smtc5list-r19                     SSB-MTC5List-r19          OPTIONAL,   -- Need R

}

...

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

SSB-MTC5List-r19::=                 SEQUENCE (SIZE(1..maxNrofSMTC5)) OF SSB-MTC5-r19

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

...

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

SSB-MTC5List-r19 ::=   SEQUENCE (SIZE (1..maxNrofSMTC5)) OF SSB-MTC5-r19

SSB-MTC5List-r19 ::= SEQUENCE {

physCellId                 PhysCellId,

assitantInfo               FFS, (It will be depends on the final decision on the selection from SSB Index, location information, etc.)

periodicity                ENUMERATED { sf5, sf10, sf20, sf40, sf80, sf160},

offsetList                 SEQUENCE (SIZE (1..maxNrofOffset)) OF INTEGER (0..159)

}

In the scenario shown in figure 2, due to the coverage by multiple SSB beams in different spatial directions and applying “beam hopping” to all the satellite beams of a given cell, UE needs to know the association between STMC and SSB beam  in order to improve measurement accuracy and efficiency.

According to the spec [4], UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell when field deriveSSB-IndexFromCell  is set to true. For example:

6.3.1	System information blocks

–	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

intraFreqCellReselectionInfo        SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

...

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

ss-RSSI-Measurement                 SS-RSSI-Measurement                             OPTIONAL,       -- Need R

ssb-ToMeasure                       SSB-ToMeasure                                   OPTIONAL,       -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

...

deriveSSB-IndexFromCell

This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in TS 38.133 [14].

6.3.2	Radio resource control information elements

–	MeasIdleConfig

The IE MeasIdleConfig is used to convey information to UE about measurements requested to be done while in RRC_IDLE or RRC_INACTIVE.

MeasIdleConfig information element

-- ASN1START

-- TAG-MEASIDLECONFIG-START

MeasIdleConfigSIB-r16 ::= SEQUENCE {

measIdleCarrierListNR-r16       SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16          OPTIONAL,     -- Need S

...

MeasIdleCarrierNR-r16 ::=        SEQUENCE {

carrierFreq-r16                  ARFCN-ValueNR,

ssbSubcarrierSpacing-r16         SubcarrierSpacing,

frequencyBandList                MultiFrequencyBandListNR                                             OPTIONAL,  -- Need R

measCellListNR-r16               CellListNR-r16                                                       OPTIONAL,  -- Need R

ssb-MeasConfig-r16               SEQUENCE {

nrofSS-BlocksToAverage-r16          INTEGER (2..maxNrofSS-BlocksToAverage)                            OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation-r16 ThresholdNR                                                       OPTIONAL,   -- Need S

smtc-r16                            SSB-MTC                                                           OPTIONAL,   -- Need S

ssb-ToMeasure-r16                   SSB-ToMeasure                                                     OPTIONAL,   -- Need S

deriveSSB-IndexFromCell-r16         BOOLEAN,

ss-RSSI-Measurement-r16             SS-RSSI-Measurement       OPTIONAL    -- Need S

}                                                                 OPTIONAL,   -- Need S

beamMeasConfigIdle-r16           BeamMeasConfigIdle-NR-r16        OPTIONAL,   -- Need R

...

}

deriveSSB-IndexFromCell

This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the neighbor frequency as specified in TS 38.133 [14].

The same IE can also be found in  SSB-ConfigMobility of MeasObjectNR.

Observation 3:  UE needs to know the association between STMC and SSB beam  in order to improve measurement accuracy and efficiency. UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell when field deriveSSB-IndexFromCell is set to true. Nevertheless, UE without the capability need to be informed the SSB index corresponding to each SMTC configuration.

Proposal 3: For informing UE the association between STMC and SSB beam, SSB index of neighbour cells needs be included in assitantInfo of the new SMTC information element (smtc5list-r19).

2.2.2 Network assistance information for idle/inactive mode UE

There is a baseline that a%3CmxGraphModel%3E%3Croot%3E%3CmxCell%20id%3D%220%22%2F%3E%3CmxCell%20id%3D%221%22%20parent%3D%220%22%2F%3E%3CmxCell%20id%3D%222%22%20value%3D%22%26lt%3Bfont%20style%3D%26quot%3Bfont-size%3A%2014px%3B%26quot%3B%26gt%3BSending%20NW%20assistance%20information%26lt%3B%2Ffont%26gt%3B%22%20style%3D%22text%3BstrokeColor%3Dnone%3Balign%3Dcenter%3BfillColor%3Dnone%3Bhtml%3D1%3BverticalAlign%3Dmiddle%3BwhiteSpace%3Dwrap%3Brounded%3D0%3B%22%20vertex%3D%221%22%20parent%3D%221%22%3E%3CmxGeometry%20x%3D%22-240%22%20y%3D%221637%22%20width%3D%22180%22%20height%3D%2230%22%20as%3D%22geometry%22%2F%3E%3C%2FmxCell%3E%3C%2Froot%3E%3C%2FmxGraphModel%3Ethert any time the UE will not use more SMTCs in parallel than in previous releases. Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

If  UE can derive the index of SS block transmitted by neighbour cells or SSB index of neighbour cells is included in SMTC configuration, UE can perform SMTC selection based on the SSB index and its own location. The NW assistance information about the SSB beam location can be the SSB index of neighbour cells, or the neighbour cells’ reference location (rough location or for the one SSB beam fully coverage cell), or the preset azimuth/tilt Angle corresponding to the antenna array that emits the SSB beam, etc. Regardless of what kinds of information is used, it needs to be included in  assitantInfo of the new SMTC information element (smtc5list-r19) of IE MeasObjectNR corresponding to other SMTC parameter.

Observation 4: Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode.

Proposal 4: The NW assistance information about the SSB beam location can be the SSB index of neighbour cells, or the neighbour cells’ reference location (rough location or for the one SSB beam fully coverage cell), or the preset azimuth/tilt Angle corresponding to the antenna array that emits the SSB beam, etc.

Proposal 5: NW assistance information needs to be included in  assitantInfo of the new SMTC information element (smtc5list-r19) of MeasObjectNR corresponding to other SMTC parameter.

2.2.3 UE assistance information for Network

Figure 3 NW performs SMTC selection

For UE in connected node, the network side can dynamically configure SMTC for a given UE based on its assistance information. The UE assistance information may be its own location, the signal quality of the measured PCI/SSB beam, etc.

According to the spec [4], UE can report its coarse UE location and measurement result to NW through MeasResults report or provide its coarse UE location to NW via UEInformationResponse upon NW's request. NW can perform SMTC selection for the given UE based on the UE assistance information and the deployment of the beam hopping pattern of the neighbour cell surrounding the UE.

Observation 5: It will be facilitate the accurate SMTC configuration by network for a given UE, based on the  assistance information reported from the UE. And UE can report the UE assistance information through existing signaling process.

Proposal 6: Network dynamically configures SMTC for a given UE based on its assistance information.

2.2 Access control for DL CE

In previous RAN2#127bis meetings, RAN2 has agreed to consider possible enhancements to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE with the intention that save UE power when UE incapable of DL CE is not able to reselect or perform measurements on neighbor cells with DL CE.

Agreements: (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

Figure 4 Cell re-selection scenario for idle/inactive mode UE not supporting DL CE

There are two cases in cell re-selection scenario:

- Case 1:Neighboring cells supporting DL CE and normal cells are deployed in different carriers

If neighboring cells supporting DL CE and normal cells are not be deployed in co-channel manner, i.e., DL CE cells and normal cells are deployed in different carriers, existing frequency priority can be used to down-prioritize cells operating with DL CE for UEs not supporting DL CE. This means that the UEs not supporting DL CE can realize de-prioritizing re-selection to cells operating with DL CE via legacy frequency priority.

- Case 2: Neighboring cells supporting DL CE and normal cells are deployed in co-channel manner

If neighboring cells supporting DL CE and normal cells are deployed in co-channel manner, it seems that a new cell-specific priority may be introduced to down-prioritize cells operating with DL CE for UEs not supporting DL CE. However, inter-cell interference cannot be avoided if the radio condition of high-priority cell is worse than that of low-priority cell. Hence, to avoid the complexity and inter-cell interference of the introducing the cell-specific priority, keeping the frequency-specific priority is preferred. Conversely, the existing IntraFreqExcludedCellList IE can be utilized to enable the UEs not supporting DL CE to avoid the performance of the detection for PSS/SSS of cell with DL CE to reduce the amount of coherent detection and UE power.

Observation 6: There are two cases in cell re-selection scenario:

- Case 1: Neighboring cells supporting DL CE and normal cells are deployed in different carriers

- Case 2: Neighboring cells supporting DL CE and normal cells are deployed in co-channel manner

Proposal 7: For case 1, the UEs not supporting DL CE can realize de-prioritizing re-selection to cells operating with DL CE via legacy frequency priority.

Proposal 8: For case 2, to avoid the complexity and inter-cell interference of the introducing the cell-specific priority, keeping the frequency-specific priority is preferred. Conversely, the existing IntraFreqExcludedCellList IE can be utilized to enable the UEs not supporting DL CE to avoid the performance of the detection for PSS/SSS of cell with DL CE to reduce the amount of unnecessary coherent detection and UE power.

2.3 Link level coverage enhancements

As agreements of RAN2#129bis meetings, RAN2 will wait for RAN1 conclusion on link level enhancements.

- Agreements:

- We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

In RAN1#120bis meeting, link level enhancement for NR-NTN downlink coverage enhancements  had been discussed,  the following agreements have been achieved:

Agreement: When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination. Agreement: For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition. Agreement: For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS. Agreement: For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots: - The same resource allocation is assumed for all repetitions - The supported repetition factors are: 2 and 4 - The network configures a single value between 2 and 4 at a given time - The RV cycling is used for each repetition - If N=4: Starting RV; RV applicable to the nth repetition/transmission. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3.

In previous RAN1#120 meetings, related agreements are as follow:

Agreement: Support only inter-slot repetition for Type0 PDCCH CSS. Agreement: For Msg4 PDSCH repetition support, RAN1 to consider: - Option 1: UE specific repetition indication via DCI - Option 2: Msg4 repetition is configured by SIB1 - Option 3: Msg4 PDSCH repetition is implicitly determined by SIB1 PDSCH repetition Agreement: At least for enabling PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, RAN1 to consider the following options - Option 1: Using the spare 1 bit in MIB - Option 2: Using reserved bit(s) in PBCH payload - Option 3: Using codepoint(s) in PBCH payload - Option 4: UE blind decoding without signaling from the network during initial access

The repetition scenarios discussed in RAN1 are focused on the following three aspects: PDCCH CSS type-0 repetition, SIB1 PDSCH repetition and Msg4 PDSCH repetition, respectively using cell level or UE level resources.

For above  repetition scenarios, repetition trigger condition (e.g. threshold for signal quality) and  Specific thresholds may be discussed by RAN1 since the values of the threshold need to be  simulated and evaluated by RAN1.

Observation 7: For above repetition scenarios, repetition trigger condition (e.g. threshold for signal quality) and  Specific thresholds may be discussed by RAN1 since the values of the threshold need to be  simulated and evaluated by RAN1.

- For Msg4 PDSCH repetition

For Msg4 PDSCH repetition, the network should know whether UE is capable of Msg4 PDSCH repetition before performing repetition. Similar to PUCCH enhancements for Msg4 HARQ-ACK in R18, Msg3 can be used to indicate the capability/request for Msg4 PDSCH repetition.

Proposal 9: RAN2 assume Msg3 can be used to indicate the capability/request for Msg4 PDSCH repetition.

To enable Msg3 based capability/request indication, LCID range by 64 need to be extended. According to the spec [5], codepoints 8 to 63 are reserved. These reserved codepoints can be used to indicate UE capability/request for Msg4 PDSCH repetition.

6.2	Formats and parameters

6.2.1	MAC subheader for DL-SCH and UL-SCH

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 to 63 | (216 + 328) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

Observation 8: These reserved codepoints 8 to 63 can be used to indicate UE capability/request for Msg4 PDSCH repetition.

In Xiaomi’s proposal [6], based on the simulation results in TR38.830, the Msg4 PDSCH channel is always better than PUCCH with HARQ-ACK Msg4 channel. And if UE requires to send Msg4 PDSCH repetition request, it always needs to send PUCCH repetition of Msg4 HARQ-ACK request.

However, from our perspective, if the downlink transmission power is set higher than that in the simulation, the simulation result and corresponding conclusion may not correct. Hence, we proposed as follows:

Proposal 10:  RAN2 discuss two options for supporting Msg4 PDSCH repetition:

- Option1: Adding the following codepoints in Msg3 to indicate UE capability/request for Msg4 PDSCH repetition:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE

- Option 2: Adding the  following codepoints in Msg3 to indicate UE capability/request for Msg4 PDSCH repetition and sending two MAC CE simultaneously when Msg4 HARQ-ACK & Msg4 PDSCH repetition are supported:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- Conclusion

Observation 1: Each SSB-MTC4 element of the smtc4list in the same MeasObjectNR uses the same duration and periodicity parameter (derived from parameter periodicityAndOffset) from the smtc1 configuration, not supporting two  periodicities.

Observation 2:  The IE smtc1 and smtc4list-r17 in the same MeasObjectNR can configure at most 4 SMTC occasions.

Proposal 1: RAN2 introduce new SMTC information element (smtc5list-r19) in MeasObjectNR for R19 DL CE capable UE.

Proposal 2: RAN2 assume maximum number of SMTCs that need to be supported is 6, wherein the maximum sequence size of smtc5list-r19 is 5 for supporting 6 SMTCs per frequency for idle/inactive UEs.

Observation 3:  UE needs to know the association between STMC and SSB beam  in order to improve measurement accuracy and efficiency. UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell when field deriveSSB-IndexFromCell is set to true. Nevertheless, UE without the capability need to be informed the SSB index corresponding to each SMTC configuration.

Proposal 3: For informing UE the association between STMC and SSB beam, SSB index of neighbour cells needs be included in assitantInfo of the new SMTC information element (smtc5list-r19).

Observation 4: Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode.

Proposal 4: The NW assistance information about the SSB beam location can be the SSB index of neighbour cells, or the neighbour cells’ reference location (rough location or for the one SSB beam fully coverage cell), or the preset azimuth/tilt Angle corresponding to the antenna array that emits the SSB beam, etc.

Proposal 5: NW assistance information needs to be included in  assitantInfo of the new SMTC information element (smtc5list-r19) of MeasObjectNR corresponding to other SMTC parameter.

Observation 5: It will be facilitate the accurate SMTC configuration by network for a given UE, based on the  assistance information reported from the UE. And UE can report the UE assistance information through existing signaling process.

Proposal 6: Network dynamically configures SMTC for a given UE based on its assistance information.

Observation 6: There are two cases in cell re-selection scenario:

- Case 1: Neighboring cells supporting DL CE and normal cells are deployed in different carriers

- Case 2: Neighboring cells supporting DL CE and normal cells are deployed in co-channel manner

Proposal 7: For case 1, the UEs not supporting DL CE can realize de-prioritizing re-selection to cells operating with DL CE via legacy frequency priority.

Proposal 8: For case 2, to avoid the complexity and inter-cell interference of the introducing the cell-specific priority, keeping the frequency-specific priority is preferred. Conversely, the existing IntraFreqExcludedCellList IE can be utilized to enable the UEs not supporting DL CE to avoid the performance of the detection for PSS/SSS of cell with DL CE to reduce the amount of unnecessary coherent detection and UE power.

Observation 7: For above repetition scenarios, repetition trigger condition (e.g. threshold for signal quality) and  Specific thresholds may be discussed by RAN1 since the values of the threshold need to be  simulated and evaluated by RAN1.

Proposal 9: RAN2 assume Msg3 can be used to indicate the capability/request for Msg4 PDSCH repetition.

Observation 8: These reserved codepoints 8 to 63 can be used to indicate UE capability/request for Msg4 PDSCH repetition.

- Proposal 10:  RAN2 discuss two options for supporting Msg4 PDSCH repetition:

- Option1: Adding the following codepoints in Msg3 to indicate UE capability/request for Msg4 PDSCH repetition:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE

- Option 2: Adding the  following codepoints in Msg3 to indicate UE capability/request for Msg4 PDSCH repetition and sending two MAC CE simultaneously when Msg4 HARQ-ACK & Msg4 PDSCH repetition are supported:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- References

- Chairman's Notes RAN2#129bis

- Chairman's Notes RAN2#129

- Chairman's Notes RAN2#128

- TS 38.331, Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification (Release 18)

- TS 38.321, Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification (Release 18)

- R2-2501798, Discussion on DL coverage enhancement, Xiaomi.

- Chairman's Notes RAN2#127bis