3GPP TSG-RAN WG2 Meeting #130	R2-2504410

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.7.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	LCP enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#129bis:

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. - It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. - For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. - Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. - All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

# 2	Discussion

## 2.1	Handling of negative Bj for LCHs with upgraded priority

For the open issue on Bj enhancement, it is worth noting that the two round LCP procedure was introduced to ensure fairness as well as avoid starvation. PBR and token bucket were introduced to guarantee the GBR bearer can meet its data rate before allocating resources for best effort bearers.

It will only work with the assumption of the NW would configure the PBR and token bucket duration reasonably even if with some scheduling delay. Nothing new comes with XR traffic as it needs to meet the data rate and PDB QoS requirements. Thus, it should be enough to rely on sensible NW implementation to configure PBR and token size according to the traffic characteristic, and QoS requirements on data rate and PDB since the delay critical data in the buffer, caused by scheduling delay, would either be discarded, if not scheduled on time, or cleared with accumulated token bucket.

Observation 1: sensible NW implementation would configure PBR and token bucket size properly to ensure it will meet the QoS requirements without resulting in data stuck in the buffer.

Besides, it should be noticed that in current specifications Bj is already allowed to go negative during the first round of the LCP procedure. This is primarily to enable the UE to avoid unnecessary segmentation, but in theory there is no constraint on how much negative Bj can go during the first round of the LCP procedure. Of course, if Bj is decreased to a very large negative value (which in practice means the PBR bucket is exceeded), the corresponding LCH may be precluded from being allocated resources in the first round of the LCP procedure the next time the UE is granted resources. Again, this can be avoided by proper configuration of the PBR and token bucket duration.

Observation 2: allowing Bj to go negative during first round of the LCP procedure is already allowed in current MAC specifications to avoid unnecessary segmentation.

Additionally, the enhanced DSR (Rel-19) provides the gNB with detailed knowledge of delay-critical data and associated remaining time in the UE buffer, hence the gNB can schedule a grant so that delay-critical data can be allocated in either the first or second step of the LCP procedure.

Observation 3: with enhanced DSR and proper configuration of PBR and bucket size, NW has all the flexibility needed to ensure it can allocate enough resources for the transmission of delay-critical data in either rounds of the LCP procedure.

Proposal 1: no enhancement needed for Bj handling for LCH priority adjustment.

## 2.2	Impact of additional priority on SR priority determination

During the running MAC CR review the following was proposed for SR priority determination:

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this determination, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3). The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR. For the MAC entity configured with lch-basedPrioritization, if the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI as specified in clause 11.2A of TS 38.213 [6] or cancelled by a high PHY-priority PUCCH transmission as specified in clause 9 of TS 38.213 [6], this configured uplink grant is considered as a de-prioritized uplink grant. If this de-prioritized uplink grant is configured with autonomousTx, the configuredGrantTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running. If this de-prioritized uplink grant is configured with autonomousTx, the cg-RetransmissionTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running. For the MAC entity configured with lch-basedPrioritization, the MAC entity shall for a pending SR: 1> if the SR is triggered by a logical channel configured with additionalPriority; and - if the smallest remaining value of the running PDCP discardTimers among the data available for transmission in this logical channel, evaluated at the time of the first symbol of the next PUCCH transmission for the SR, is below or at priorityAdjustmentThreshold configured for the logical channel: 2> consider additionalPriority as the priority of the PUCCH transmission.

It means if the MAC entity is configured with lch-basedPrioritization and if an SR is triggered by an LCH configured with additionalPriority, it needs to check if there is data available for transmission with remaining time smaller than or equal to priorityAdjustmentThreshold, which may increase the number of operation the UE needs to perform prior to a PUCCH/PUSCH transmission. Besides, whether SR can be transmitted depends on PHY prioritization as well. MAC does not decide the priority of the PUCCH but only the priority of the SR. It depends on simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCHgroups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup as well as PHY priority which is already very complex. Thus, changing SR priority dynamically depending on data status should rather be avoided.

Proposal 2: LCH priority adjustment should not impact SR priority determination.

## 2.3	Impact of congestion (i.e., PSI-based SDU discard) on priority adjustment

Current running MAC CR covers allocation of resources and priority adjustment as follows:

The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> for a logical channel configured with priorityAdjustmentThreshold, if the smallest remaining value of the running PDCP discardTimers among its data available for this transmission, evaluated at the time of the first symbol of this transmissoin, is below its priorityAdjustmentThreshold: 3> apply additionalPriority of this logical channel; 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

When PSI-based SDU discard is activated, low importance data uses discardTimerForLowImportance instead of discardTimer. It is not urgent to send the low importance data even if the discardTimerForLowImportance becomes below threshold, thus it should not impact priority adjustment if no other data with remaining time below threshold. Note the current running CR already refers to discardTimer only, we see no change needed.

Proposal 3: confirm that remaining time of low priority data is not considered for LCP priority adjustment since it does not use discardTimer.

# 3	Conclusion

Remaining issues on LCP are discussed in this contribution with the following proposals proposed:

Proposal 1: no enhancement needed for Bj handling for LCH priority adjustment.

Proposal 2: LCH priority adjustment should not impact SR priority determination.

Proposal 3: confirm that remaining time of low priority data is not considered for LCP priority adjustment since it does not use discardTimer.