3GPP TSG-RAN WG2 Meeting #130	R2-2504414

St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.1.2.2

Source:	Nokia, T-Mobile USA Inc., Ericsson, BT plc, Verizon, Deutsche Telekom AG

Title:	On Enhancements for NW Involvement in UE-side DC for BM

WID/SID:	NR_AIML_Air-core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses some enhancements to NW involvement in the configuration for UE-side data collection for beam management. The agreements thus far require each UE to send an additional message, a UEAssistanceInformation, when it wants to indicate a preference for UE-side data collection, and for the NW to send at least an RRCReconfiguration and for the UE to send at least an RRCReconfigurationComplete when it wants to configure the UE with a measurement configuration. This paper proposes a few ways to limit the impact of requests for measurement configurations for UE-side data collection.

# 2	State of UE-side DC Until RAN2#130

## 2.1	Agreements

The following agreements related to UE-side data collection were reached during RAN2#129.

- The network can provide or release the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

And the following additional agreements were reached during RAN2#129bis.

- The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

  - An indication on start/stop of data collection

  - Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request

## 2.2	Implementation

The agreements thus far provide the following mechanisms for NW control over UE-side DC.

- The NW is in control of which data collection options to provide to the UE.

- The NW can enable or disable the ability for the UE to request a data collection configuration preference.

- The NW can ignore a request for data collection.

- The NW can configure or de-configure reference signals for data collection at any time.

Figure 1 below illustrates the call flow of the current agreements. After the gNB provides the data collection options to the UE, the UE must send a UEAssistanceInformation to request the RS configuration, then the gNB must send an RRCReconfiguration, requiring an RRCReconfigurationComplete in response from the UE. These messages are required whether or not the gNB is already transmitting those RS for other UEs with the same preference.

Figure 1: UE-side Data Collection Configuration as of RAN2#129bis

# 3	Optimizations to UE-side Data Collection

## 3.1	Enhanced NW involvement

Per the existing agreements, UEs are provided data collection configuration candidates whose content has yet to be agreed. UEs provide their data collection configuration preference(s) to the NW using UEAssistanceInformation in response, and then the NW configures the UE with a measurement configuration. An association between the data collection configuration and an eventual inference configuration, and the way that the UE provides its data collection configuration preference(s) and how the UE receives its data collection configuration are suboptimal.

In the next three sections, we propose the following to address an open issue and optimize the reporting of UE-side data collection preference(s) and configuring the UE based on its preference(s):

- Provide UE-side data collection configuration options as CSI-ReportConfig, referred to by CSI-ReportConfigId

- The UE can provide its data collection configuration preferences in RRCReconfigurationComplete

- The gNB can activate or deactivate a data collection configuration option using a MAC-CE

### 3.2	(RRC-3) UE data collection request [Full configurations]

In a list of open issues being discussed as an email discussion prior to this meeting, RAN2#130, here, the following was proposed as an RRC open issue in AI/ML PHY feature.

Issue description: For UE-side data collection, the details of UAI signaling for the UE start/stop request and especially the following aspects are not yet clarified:

•	how to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW

•	where/what the NW provides as candidate configurations

•	what the content of otherConfig for enabling UE data collection requests in UAI should be (e.g. just a flag, the list of candidate UE data collection configurations, etc.).

In RAN1#120, the following was agreed.

For UE-sided model, for configuring the resource for data collection purpose, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

- One CSI-ResourceConfigId is configured for Set A.

- One CSI-ResourceConfigId is configured for Set B.

- Note: UE performs measurement on all resources

- One or two associated IDs can be configured in CSI-ReportConfig

- When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

- Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: whether/how to support 'aperiodic' CSI RS

In addition to the RAN1 agreement, it could be beneficial to provide the data collection configuration options as CSI-ReportConfigs because the same structure is used to configure UEs for inference. The association between a configuration for data collection and for inference would provide the UE the information it needs to determine applicability of a model resulting from the data collection based on the data collection configuration. Therefore, we can already propose to agree to follow RAN1’s agreement, satisfying the first two bullets.

Observation 1: Using CSI-ReportConfig for data collection configuration candidates also provides the UE-side with information sufficient to determine applicability of a model resulting from the data collection based on the data collection configuration.

Observation 2: RAN1 agreed CSI-ReportConfig can be used to configure resources for UE-side data collection.

Proposal 1: Candidate UE-side data collection configuration candidates are provided by the gNB as CSI-ReportConfigs under CSI-MeasConfig and are referred to by CSI-ReportConfigIds. FFS if these are measurement data collection configurations or also inference configurations.

Proposal 2: In otherConfig, UE-side data collection configuration candidates are provided as a list of pairs of CSI-ReportConfigId and a serving cell index, i.e., PCell or SCell index.

Proposal 3: Preferred UE-side measurement configurations are indicated to the gNB as CSI-ReportConfigIds.

Proposal 4: UE-side measurement configurations are applied without resulting in CSI measurement reports.

### 3.3	Provide data collection preference in RRCReconfigurationComplete

The NW provides UE-side measurement configuration candidates to the UE in an RRCReconfiguration message which must be acknowledged by the UE with an RRCReconfigurationComplete message. For UEs whose data collection needs do not change as frequently as it transitions between states, the UE should be able to provide a measurement configuration preference as soon as it receives the options. By doing so, the message volume related to providing UE-side data collection preferences is reduced.

Observation 3: Message volume related to providing measurement configuration preferences can be reduced by enabling the UE to provide its preference(s) in RRCReconfigurationComplete after receiving data collection configuration candidates.

It may be that an individual UE does not need to measure immediately after receiving the data collection configuration options. However, it is also likely that more than one UE has the same data collection configuration preference as others, and that the gNB will decide to activate a data collection configuration candidate based on the aggregation of requests from many UEs.

Therefore, a UE having provided its measurement configuration preference prior to needing to make measurements does not result in wasted resources since some UEs with the same preference may be collecting measurements.

Observation 4: A UE providing its measurement configuration preference prior to needing to make measurements does not result in wasted resources since some UEs with the same preference may be collecting measurements on the same preferred measurement configuration.

Proposal 5: In addition to being able to provide its measurement configuration preference in UEAssistanceInformation, the UE can provide data collection preference in RRCReconfigurationComplete.

### 3.4	Enable the use of a MAC-CE to enable or disable reference signals for UE-side data collection

To support UE selection with minimal overhead, the gNB could use a MAC-CE to inform UEs on an individual basis that certain reference signals for UE-side data collection are transmitting. If the previous proposals are agreed, then the only additional overhead still present is the transmission of an RRCReconfiguration and RRCReconfigurationComplete to configure and acknowledge the configuration of reference signals for UE-side data collection.

Observation 5: UE-side measurement configuration overhead could be further reduced by introducing a MAC CE to enable and disable UE-side L1 CSI-RS measurement in a UE-side data collection configuration.

Proposal 6: The gNB can enable and disable UE-side L1 CSI-RS measurements in a UE-side data collection configuration with a MAC-CE. FFS whether a new MAC CE is needed or whether a legacy MAC CE can be reused.

## 3.5 Summary of Sections 3.2, 3.3, and 3.4

The previous sections can be summarized with the call flow of Figure 2 below.

Figure 2: Summary Call Flow of Sections 3.2, 3.3, and 3.4

# 4	Conclusion

This document has made the following observations:

Observation 1: Using CSI-ReportConfig for data collection configuration candidates also provides the UE-side with information sufficient to determine applicability of a model resulting from the data collection based on the data collection configuration.

Observation 2: RAN1 agreed CSI-ReportConfig can be used to configure resources for UE-side data collection.

Observation 3: Message volume related to providing measurement configuration preferences can be reduced by enabling the UE to provide its preference(s) in RRCReconfigurationComplete after receiving data collection configuration candidates.

Observation 4: A UE providing its measurement configuration preference prior to needing to make measurements does not result in wasted resources since some UEs with the same preference may be collecting measurements on the same preferred measurement configuration.

Observation : UE-side measurement configuration overhead could be further reduced by introducing a MAC CE to enable and disable UE-side L1 CSI-RS measurement in a UE-side data collection configuration.

And proposed the following:

Proposal 1: Candidate UE-side data collection configuration candidates are provided by the gNB as CSI-ReportConfigs under CSI-MeasConfig and are referred to by CSI-ReportConfigIds. FFS if these are measurement data collection configurations or also inference configurations.

Proposal 2: In otherConfig, UE-side data collection configuration candidates are provided as a list of pairs of CSI-ReportConfigId and a serving cell index, i.e., PCell or SCell index.

Proposal 3: Preferred UE-side measurement configurations are indicated to the gNB as CSI-ReportConfigIds.

Proposal 4: UE-side measurement configurations are applied without resulting in CSI measurement reports.

Proposal 5: In addition to being able to provide its measurement configuration preference in UEAssistanceInformation, the UE can provide data collection preference in RRCReconfigurationComplete.

Proposal 6: The gNB can enable and disable UE-side L1 CSI-RS measurements in a UE-side data collection configuration with a MAC-CE. FFS whether a new MAC CE is needed or whether a legacy MAC CE can be reused.