3GPP TSG-RAN WG2 Meeting #130	R2-2504417

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.5.2

Source:	Panasonic

Title:	On-demand SSB SCell operation

Document for:	Discussion/Decision

Introduction

In this contribution, we share our views on configuration of OD-SSB and L3 measurement.

Discussion

  - OD-SSB configuration

Followings were agreed in RAN2.

Agreements on OD-SSB Scell in RAN2#127 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. 2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. Agreements on OD-SSB Scell in RAN2#129b 1. For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). 2. A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case. 3. For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. 4. OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB. 5. RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

Referring to RAN1 progress, multiple candidate values need to be configured by RRC.

Agreement in RAN1#119 For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer Agreement in RAN1#120 - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Agreement in RAN1#120b For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

- RRC configuration of OD-SSB

For OD-SSB configuration, sCellConfigCommon and sCellConfigDedicated can support multiple candidate values of OD-SSB parameters. In RRC to initial configuration, states (activation/ deactivation) are indicated.  UE can be indicated any parameter that RAN1 concludes to support multiple candidate values.

- Activation/Deactivation by RRC

OD-SSB is used for L1/L3 measurements. We consider OD-SSB triggering timing is up to network implementation irrespective of UE situation. Even after the SCell is configured on-demand SSB may also be further triggered and indicated by network, even if the AO-SSB in Case 2 is already available. Therefore, independent signaling from Scell activation is also useful. For RRC, RAN2 agreed that no need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. Therefore, we propose SCell modification indicates activation/deactivation of OD-SSB.

- Activation/Deactivation by MAC

For MAC CE, RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. RAN1 agreed “the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].” Whether MAC CE can deactivate the OD-SSB transmission with configured od-ssb-nrofBurst is bracket. For UE side, there is an impact on measurement pipeline implementation for L1 CSI report, especially AP-CSI report. However, Network side can handle the OD-SSB activation and deactivation properly to avoid potential issues. Deactivation by MAC CE gives network better control on energy saving [1]. Therefore, we propose a single MAC CE format for both explicit and implicit OD-SSB deactivations. Implicit OD-SSB can be deactivated by MAC CE.

Proposal 1: sCellConfigCommon and sCellConfigDedicated support multiple candidate values of OD-SSB parameters.

Proposal 2: Scell modification indicates activation/deactivation of OD-SSB.

Proposal 3: Single MAC CE format for both explicit and implicit OD-SSB deactivations. Implicit OD-SSB can be deactivated by MAC CE.

  - L3 RRM

In RAN2#129b meeting, following was agreed for SSB adaptation.

5. For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. 6. Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

In RAN2#129b, following was working assumption for OD-SSB.

6. Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

RAN1 agreed follows.

Agreement RAN1#120 Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

As same as SSB adaptation, we propose to keep SMTC based L3 RRM framework and to introduce additional SMTC. For the same frequency case, a SMTC window could cover both always on SSB and OD-SSB. However, longer/dense SMTC window would restrict the network scheduling flexibility and would lead to more UE power consumption for measurement. Therefore, the configuration of two SMTCs would be reasonable. It can support different periodicities for always on SSB and OD-SSB. In the current specification, up to 4 SMTCs can be configured. As similar as smtc2, smtc2-LP, smtc3list and smtc4list, additional smtc can be configured for OD-SSB. It could be additional UE capability. When UE has the capability to support additional SMTC for OD-SSB, UE measures both always on SSB and OD-SSB. If UE doesn’t have this capability, SMTC is configured for one of among 1) always on SSB, 2) OD-SSB or 3) one SMTC cover both SSBs with longer/dense window.

For different frequency case, different SMTCs are necessary to measure different SSBs as working assumption. When UE doesn’t have the capability to support additional SMTCs, one of always on SSB or OD-SSB should be measured at one time. Even if additional SMTC is supported, RAN1 assumes UE is not required to measure both AO-SSB and OD-SSB. When one of SMTC is active in a time, SMTC patterns can be configured for not to be overlapped in time domain.

Proposal 4: For same frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability.

Proposal 5: Allow multiple OD-SSBs with different frequencies for a given Scell. For different frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability. When one of SMTC is active in a time, SMTC patterns are configured as not to be overlapped in time domain.

Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: sCellConfigCommon and sCellConfigDedicated support multiple candidate values of OD-SSB parameters.

Proposal 2: Scell modification indicates activation/deactivation of OD-SSB.

Proposal 3: Single MAC CE format for both explicit and implicit OD-SSB deactivations. Implicit OD-SSB can be deactivated by MAC CE.

Proposal 4: For same frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability.

Proposal 5: Allow multiple OD-SSBs with different frequencies for a given Scell. For different frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability. When one of SMTC is active in a time, SMTC patterns are configured as not to be overlapped in time domain.

Reference

[1] R1-2504235 “Discussion on on-demand SSB SCell operation”, Panasonic