3GPP TSG-RAN WG2 #130 	R2-2504426

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Discussion on DL messages for Ambient IoT UEs

Document for:	Discussion/Decision

# 1	Introduction

In RAN#107, a new WI has been approved for Ambient IoT devices [1]. The following RAN2 related objectives are capture in the WID:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

In this contribution we continue to discuss the open issues on downlink messages for A-IoT devices based on the scope highlighted above considering the agreements so far.

# 2	Discussion

## 2.1 Paging procedure in multiple reader scenarios (Issue 1-1 in [3])

Multiple readers may be deployed to serve an area, and their coverage can overlap. A reader may start an inventory procedure which is interleaved by another inventory procedure started by another reader assuming that there is no coordination between readers in time and/or frequency domains. RAN2 has agreed that an A-IoT device should participate in a single procedure, i.e., “The device is expected to only perform one procedure at a time”.

In RAN2#129bis, device behaviour was discussed in case an A-IoT device receives a service request (from another reader) while it has been performing a procedure associated with a service request that was received earlier. One solution is that the device abandons performing the ongoing procedure associated with the service request received earlier and starts performing the procedure associated with the new service request. Assuming that it is not possible for the device to know whether the new service request is provided by the same/different reader, this behaviour can be specified regardless of whether the new service request is provided by same/different reader and/or marked with same/different transaction ID.

Another solution is that the device ignores any other service request received if it is performing a procedure associated with a service request received earlier.

The first solution, i.e., Option B, may lead to waste of radio and energy resources since A-IoT device would switch procedures before completion assuming that the readers are not coordinated. It is also possible that overall network performance is degraded as successful completion may be prolonged when procedures from multiple readers are interleaved. Note that it is not possible for a reader to know whether a device has abandoned the procedure so it would continue to perform the procedure until it assumes that there has been a failure.

If the device abandons the ongoing procedure, radio and energy resources may be wasted since A-IoT device would switch procedures before completion assuming that readers are not coordinated.

Overall performance may be degraded as successful completion may be prolonged if ongoing procedure is abandoned when procedures from multiple readers are interleaved.

In the second solution, i.e., Option A, the device proceeds with the ongoing procedure that it has started to perform. This solution is more efficient from radio and energy resources and overall network performance standpoint when compared to the first solution. However, for this solution, it should be clear for the device whether and when an ongoing procedure is completed so that the device can determine whether to respond to another paging request.

It should be clear for the device whether and when an ongoing procedure is completed.

In [7], we discuss the details of option A and the cons for Option B and based on that discussion we propose to adopt Option A:

Device ignores any other paging service request received if it is performing a procedure associated with a paging service request received earlier, i.e., Option A.

## 2.2 Access occasion trigger message

In RAN2#129bis, it was agreed that access occasion trigger message indicates the start of a set of Msg1 resources that are configured in the paging message. RAN2 assumes that those messages do not include slot number/count down number.

It may be possible for a device to know whether an R2D message is associated with the ongoing procedure and ignore if it is not associated or expected, e.g., if the device has transmitted Msg1, it will ignore if it receives Msg4. However, there are cases where it may not be possible for an A-IoT device to distinguish between R2D messages from different readers. Access occasion trigger message is one such case and considering the need to distinguish those messages grom different readers, RAN2 should discuss how this can be achieved.

In multi-reader scenario, assuming that it is not possible for the device to know from which reader an R2D message is received, a device may use unintended time/frequency resources configured by a reader which is not associated with its ongoing procedure. This would not only mean a collision and therefore a failure for multiple devices, but also advancing the access occasion, i.e., the slot number, mistakenly. This can be addressed by introducing an identifier in the message. Some possible solutions are transaction ID, reader ID, and random ID.

Reader ID would introduce complexity as it requires coordination between readers. Transaction ID would imply that this ID would be per reader, i.e., generated by the reader based on the correlation ID or provided by the network with coordination. This means that RAN2 would limit what one can benefit from subsequent paging, since the device may end up responding to multiple paging requests from different readers for the same service request from the CN.

A similar approach used for Msg1, can be utilized here, i.e., a reader generated random number per paging round can be provided in the access occasion trigger message to distinguish the source reader. Each reader can generate this random number independently, i.e., coordination between readers is not required, if the random number has large enough size. Suppose that a device can receive from M readers and the random number has N bits, the table below presents the probability that at least two readers have the same random number for some values of M and N. Based on the evaluation below, 5-8 bits (depending on availability in the message) should be enough to ensure that such probability is less than 5%, especially when indoor scenarios are considered.

Table 1. Probability that at least two readers have the same random number

There may also be a need for end indication. The reader can utilize the end indication when it needs to terminate an ongoing procedure. For example, a reader may want to terminate the procedure earlier, e.g., since it turns out that there is no need for allocating a large size of number of slots. If Option A is agreed as discussed above, without such end indication it would not be possible for the reader to terminate the procedure. The end indication can be provided as part of the access occasion trigger message.

The reader can utilize the end indication when it needs to terminate an ongoing procedure.

Based on the discussion above, we propose the following:

Access occasion trigger message includes the following:

- - “end” indication that marks the last access occasion in a paging round

- - a reader generated random number per paging round. FFS size, i.e., 5-8 bits.

## 2.3 How to generate the transaction ID (Issue 1-2 in [3])

In RAN2#127, it was agreed to support a mechanism for the reader to trigger multiple/subsequent A-IoT paging messages that are associated with the same service request from the CN. In RAN2#127bis, it was agreed that A-IoT paging message can include information to avoid duplicate response from the device to a reader. In RAN2#129, it was agreed that the “transaction ID” can be generated by reader based on CN correlation ID.  But it is FFS how reader will generate “transaction ID” and the size of transaction ID”.

In this section we discuss how reader should generate transaction ID based on CN correlation ID. We think that there should be one to one mapping between transaction ID and correlation ID, otherwise one may observe the following:

- Same CN correlation ID maps to different transaction IDs, i.e., subsequent A-IoT paging messages associated with the same service request will have different transaction IDs. This may lead to that the device send duplicate responses to subsequent paging unnecessarily.

- Different CN correlation IDs map to the same transaction ID. In this case the device will regard the later received A-IoT paging message as subsequent paging message and ignore if it has successfully responded earlier.

RAN2 assumes there is one to one mapping between CN correlation ID and transaction ID.

We do not think there is a need for an additional reader ID or a need for using reader specific aspects when generating the transaction ID since this would require coordination in the network which would increase complexity. We prefer to use the CN correlation ID as the transaction ID assuming that SA2/CT4 would end up designing a ID with a “short” size. If it turns out that this is not the case, i.e., CN correlation ID has a size long enough not acceptable from RAN2 standpoint, one can consider reducing the ID in size by masking or introducing a function that maps the ID to a transaction ID following the principle that there is one to one mapping between CN correlation ID and the transaction ID as discussed above, i.e., a CN correlation ID should always be mapped to the same transaction ID regardless of reader. It should be possible to map the CN correlation ID to an ID shorter in size assuming that for indoor scenarios service areas are rather limited, i.e., no need to differentiate many readers.

Send an LS to SA2/RAN3 (cc: CT4) to inform those WGs about the assumptions in RAN2 and ask for the size of transaction ID.

## 2.4 Paging message content for CFRA (Issue 1-5 in [3])

There was a discussion on whether transaction ID can be optional for CFRA [4]. In our view, determining whether or not to respond can be based on the paging identifier (in the upper layer message) rather than the transaction ID. RAN2 agreed the following in the last meeting: “Introduce an explicit 1-bit indication to indicate whether it is CFRA or CBRA per paging message.” Therefore, one can assume that same message type is used for CBRA and CFRA, since RAN2 has agreed to introduce a flag which explicitly indicates whether the message is CFRA or CBRA. Considering that the paging message always includes the paging identifier for CFRA transaction ID is not needed.

For CFRA, transaction ID is not included to the paging message.

## 2.5 Paging identifier and visibility in the MAC layer (Issue 1-6 in [3])

In RAN2#129, the following agreements have been made:

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/” filtering criteria”, while the exact format of latter is supposed to be designed by SA2.”

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer”

RAN2#129bis further made the following agreements:

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to take this into account for their design.

Regarding the design of the paging ID length field, RAN2 should further discuss based on the reply from SA2 and other relevant WGs.

For visibility in MAC, we think that RAN2 should clarify what is intended by “visible” here. If the discussion is on whether it should be possible for the reader to, for example, mask/regenerate the identifier provided by the CN, we prefer that this should not be possible. In that sense, we do not support MAC layer sub-group paging solutions. We assume that temporary ID is intended for paging a single device, hence there is no need for sub-grouping. Note that it is not clear yet whether group paging via masking permanent ID would be possible since paging identifier may be encrypted.

Besides, some companies argued that the paging identifier needs to be visible to reader’s MAC layer at least in CFRA for the reader to associate AS ID with upcoming command request [5]. To make this work CN needs to explicitly include the paging identifier in NGAP signalling together with the command request. This is not a good design practice as paging identifier is primarily used between the CN and the device, not between the CN and the reader. A NGAP device ID is more suitable for such association. Note that RAN3#127bis already agreed to introduce NGAP device ID and RAN2 can use this ID to associate the AS ID with the upcoming command request at reader.

RAN3 agreed to introduce NGAP device ID which can be used to associate AS ID with upcoming command request at reader.

The paging identifier should be transparent to the reader, i.e., it should not be possible for the reader to, e.g., mask/regenerate the identifier provided by the CN

## 2.6 Forward Compatibility

In principle, a device should not respond to a R2D message type, if it is introduced in a future release with no backwards compatibility. This can be achieved by reserving some R2D message types so that those message types can be used by future releases, if/when needed.

R2D message types are reserved for future compatibility.

## 2.7	SA2 LS on service type

SA2 has been discussing whether “Service Type” (i.e., Inventory or Command) information in “Assistance Information” provided by AIOTF to NG-RAN is needed and has sent an LS to RAN2 and RAN3 to ask whether there is any concern if SA2 agrees to remove the “Service Type” from “Assistance Information” [6]. It is important for the reader to know in prior whether a command request will follow since it may terminate the inventory procedure after receiving Msg3 successfully.

In the meantime, RAN3 agreed to introduce a follow-on command indication in Inventory Request Transfer IE in case command request follows inventory. This means that the reader would be able to know if a command request is to follow after inventory.

The reader needs to know whether there is a command request that follows the inventory.

RAN3 has agreed to introduce a follow-on command indication in Inventory Request Transfer IE.

RAN2 informs SA2 that the reader needs to know whether there is a command that follows the inventory and assumes that this can be achieved by follow-on command indication introduced in Inventory Request Transfer IE agreed in RAN3.

# 3	Conclusion

In this contribution we discussed downlink/paging messages for A-IoT devices. Based on the discussion in the previous sections we made the following observations:

Observation 1	If the device abandons the ongoing procedure, radio and energy resources may be wasted since A-IoT device would switch procedures before completion assuming that readers are not coordinated.

Observation 2	Overall performance may be degraded as successful completion may be prolonged if ongoing procedure is abandoned when procedures from multiple readers are interleaved.

Observation 3	It should be clear for the device whether and when an ongoing procedure is completed.

Observation 4	The reader can utilize the end indication when it needs to terminate an ongoing procedure.

Observation 5	RAN3 agreed to introduce NGAP device ID which can be used to associate AS ID with upcoming command request at reader.

Observation 6	The reader needs to know whether there is a command request that follows the inventory.

Observation 7	RAN3 has agreed to introduce a follow-on command indication in Inventory Request Transfer IE.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Device ignores any other paging service request received if it is performing a procedure associated with a paging service request received earlier, i.e., Option A.

Proposal 2	Access occasion trigger message includes the following:

- “end” indication that marks the last access occasion in a paging round

- a reader generated random number per paging round. FFS size, i.e., 5-8 bits.

Proposal 3	RAN2 assumes there is one to one mapping between CN correlation ID and transaction ID.

Proposal 4	Send an LS to SA2/RAN3 (cc: CT4) to inform those WGs about the assumptions in RAN2 and ask for the size of transaction ID.

Proposal 5	For CFRA, transaction ID is not included to the paging message.

Proposal 6	The paging identifier should be transparent to the reader, i.e., it should not be possible for the reader to, e.g., mask/regenerate the identifier provided by the CN

Proposal 7	R2D message types are reserved for future compatibility.

Proposal 8	RAN2 informs SA2 that the reader needs to know whether there is a command that follows the inventory and assumes that this can be achieved by follow-on command indication introduced in Inventory Request Transfer IE agreed in RAN3.

# 4      References

RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #107, Incheon, South Korea, March, 2025.

TR 22.840, V0.4.0, “Study on Ambient power-enabled Internet of Things (Release 19)”

[POST129bis][017][AIoT] 38.391 Running CR, Huawei, RAN2#129bis, Wuhan, China, 7th – 11th April, 2025.

R2-2501828, “Discussion on A-IoT paging”, HONOR, RAN2#129bis, Wuhan, China, 7th – 11th April, 2025

R2-2502819, “Discussion on Ambient IoT paging message design”, ASUSTeK, RAN2#129bis, Wuhan, China, 7th – 11th April, 2025

S2-2504294, “LS on the removal of service type information”, Interdigital, SA2#168, Gothenburg, Sweden, 7 - 11 April, 2025.

R2-2504112, “Details of option A for overall A-IoT MAC procedure”, ZTE Corporation, Sanechips, Continental Automotive, Tejas Networks, Nokia, Ofinno, Samsung, Fujitsu, Kyocera, Sony, ETRI, III, Ericsson, RAN2#130, St. Julian’s Malta, 19th – 23rd May, 2025