3GPP TSG-RAN WG3 Meeting #130	R2-2504442

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.3

Source:	CMCC

Title:	Discussion on RAN4 LS on UE assistance information

Document for:	Discussion

# 1 Introduction

In the incoming LS [1] from RAN4, the following information about UE assistance information for RRM measurement gaps/restrictions enhancements are included:

1.	Overall Description:

RAN4 thanks RAN1 for the LS on UE assistance information (R4-2411007/R1-2405736).

Regarding UE assistance information related to measurement occasions, the following was agreed on RAN4#114 (R4-2502700):

Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.

And the following was agreed in RAN4 #114bis (R4-2504911):

- The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

2	Actions:

To RAN2

ACTION:  RAN4 respectfully asks RAN2 to implement the signaling of the above mentioned UAI reporting for XR.

In this contribution, we intend to discuss the details of UAI as agreed by RAN4, including when to trigger UAI, application of prohibit timer and the signalling design on UAI.

# 2 Discussion

Similar with the existing definition of UAI, a UE capable of providing gap skipping or gap cancellation assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so. One trigger for UE to initiate UAI procedure could be upon having a preference or recommendation on the ratio of canceled gap occasions for enabling TX/RX for XR. Moreover, considering UE implementation and the variation of channel quality, UE may update its preference on the ratio of canceled gap occasions. Therefore, the other trigger for UE to initiate UAI procedure could be upon change of its preference or recommendation on the ratio of canceled gap occasions.

Proposal 1: The following triggers for UE to initiate UAI procedure could be considered:

- upon having a preference or recommendation on the ratio of canceled gap occasions for enabling TX/RX for XR;

- upon change of its preference or recommendation on the ratio of canceled gap occasions.

Moreover, we think the prohibit timer is needed for UE providing gap cancellation assistance information to NW. And the maximum value of the prohibit timer could be further discussed.

Proposal 2: RAN2 to introduce the prohibit timer for UE providing gap cancellation assistance information to NW. The maximum value of the prohibit timer could be further discussed.

Therefore, the following statements could be used for UE initiating UAI procedure related to XR:

A UE capable of providing gap cancellation assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon having a preference or recommendation on the ratio of canceled gap occasions for enabling TX/RX for XR and upon change of its preference or recommendation on the ratio of canceled gap occasions.

Upon initiating the procedure, the UE shall:

1> if configured to provide gap cancellation assistance information:

2> if the UE has a preference or recommendation (FFS) on the ratio of canceled gap occasions for enabling TX/RX for XR and the UE did not transmit a UEAssistanceInformation message with gapcancellation-Preference (FFS) since it was configuration to provide its preference on the ratio of canceled gap occasions for enabling TX/RX for XR and prohibit timer (FFS) is not running; or

2> if the current gapcancellation-Preference (FFS) information on the ratio of canceled gap occasions for enabling TX/RX for XR is different from the one indicated in the last transmission of the UEAssistanceInformation message including gapcancellation-Preference (FFS) and prohibit timer (FFS) is not running:

3> start the timer with the timer value set to the gapcancellation-PreferenceProhibitTimer (FFS);

3> initiate transmission of the UEAssistanceInformation message to provide the current gapcancellation-Preference (FFS);

Proposal 3: RAN2 to introduce the above statements for UE initiating UAI procedure related to XR.

Furthermore, UE could be configured via the OtherConfig message with the prohibit timer related configuration to provide gap cancellation assistance information.

Proposal 4: The OtherConfig message with the prohibit timer related configuration could be used to configure UE to provide gap cancellation assistance information.

In the LS [1], RAN4 provides the value of the ratio (R) of gap occasions that is recommended for cancellation during a time period of 1s as follow:

R ∈ {0, 20, 40, 60}%

A new IE (e.g., gapcancellation-Preference) should be introduced in UEAssistanceInformation message for the gap cancellation assistance information, for example:

gapcancellation-Preference (FFS)  ::= SEQUENCE {

canceledpercentGap (FFS)                      ENUMERATED {0, p20, p40, p60}

}

gapcancellation-Preference

Indicates the UE preferred the ratio of canceled gap occasions for enabling TX/RX for XR. Value p20 corresponds to 0.2, value p40 corresponds to  0.4, and so on.

Proposal 5: RAN2 to introduce a new IE for the gap cancellation assistance information in UEAssistanceInformation message.

Therefore, upon initiating the procedure, the UE should provide the value of the ratio in UEAssistanceInformation message, for example:

The UE shall set the contents of the UEAssistanceInformation message as follow:

1> if transmission of the UEAssistanceInformation message is initiated to provide gapcancellation-Preference (FFS):

2> include gapcancellation-Preference (FFS) in the UEAssistanceInformation message;

2> if the UE has a preference on the ratio of canceled gap occasions for enabling TX/RX for XR:

3> include canceledpercentGap (FFS) to the value of the ratio of canceled gap occasions which the UE desired;

2> else (if the UE has no preference on the ratio of canceled gap occasions for enabling TX/RX for XR):

3> do not include canceledpercentGap (FFS) in the gapcancellation-Preference (FFS) IE;

Proposal 6: RAN2 to introduce the above statements for UEs actions related to transmission of UEAssistanceInformation message related to XR.

Proposal 7: RAN2 to introduce a new optional UE capability signalling, indicating the support of providing gap cancellation assistance information for enabling TX/RX for XR.

# 3 Conclusion

In this paper, we discuss on the details of UAI as agreed by RAN4 to provide gap cancellation assistance information for enabling TX/RX for XR, we have the following proposals:

Proposal 1: The following triggers for UE to initiate UAI procedure could be considered:

- upon having a preference or recommendation on the ratio of canceled gap occasions for enabling TX/RX for XR;

- upon change of its preference or recommendation on the ratio of canceled gap occasions.

Proposal 2: RAN2 to introduce the prohibit timer for UE providing gap cancellation assistance information to NW. The maximum value of the prohibit timer could be further discussed.

Proposal 3: RAN2 to introduce the statements for UE initiating UAI procedure related to XR.

A UE capable of providing gap cancellation assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon having a preference or recommendation on the ratio of canceled gap occasions for enabling TX/RX for XR and upon change of its preference or recommendation on the ratio of canceled gap occasions.

Upon initiating the procedure, the UE shall:

1> if configured to provide gap cancellation assistance information:

2> if the UE has a preference or recommendation (FFS) on the ratio of canceled gap occasions for enabling TX/RX for XR and the UE did not transmit a UEAssistanceInformation message with gapcancellation-Preference (FFS) since it was configuration to provide its preference on the ratio of canceled gap occasions for enabling TX/RX for XR and prohibit timer (FFS) is not running; or

2> if the current gapcancellation-Preference (FFS) information on the ratio of canceled gap occasions for enabling TX/RX for XR is different from the one indicated in the last transmission of the UEAssistanceInformation message including gapcancellation-Preference (FFS) and prohibit timer (FFS) is not running:

3> start the timer with the timer value set to the gapcancellation-PreferenceProhibitTimer (FFS);

3> initiate transmission of the UEAssistanceInformation message to provide the current gapcancellation-Preference (FFS);

Proposal 4: The OtherConfig message with the prohibit timer related configuration could be used to configure UE to provide gap cancellation assistance information.

Proposal 5: RAN2 to introduce a new IE for the gap cancellation assistance information in UEAssistanceInformation message.

Proposal 6: RAN2 to introduce the above statements for UEs actions related to transmission of UEAssistanceInformation message related to XR.

The UE shall set the contents of the UEAssistanceInformation message as follow:

1> if transmission of the UEAssistanceInformation message is initiated to provide gapcancellation-Preference (FFS):

2> include gapcancellation-Preference (FFS) in the UEAssistanceInformation message;

2> if the UE has a preference on the ratio of canceled gap occasions for enabling TX/RX for XR:

3> include canceledpercentGap (FFS) to the value of the ratio of canceled gap occasions which the UE desired;

2> else (if the UE has no preference on the ratio of canceled gap occasions for enabling TX/RX for XR):

3> do not include canceledpercentGap (FFS) in the gapcancellation-Preference (FFS) IE;

Proposal 7: RAN2 to introduce a new optional UE capability signalling, indicating the support of providing gap cancellation assistance information for enabling TX/RX for XR.

# Reference

- R2-2503328/R4-2504972, Reply LS on UE assistance information, Apr, 2025.