3GPP RAN WG2 Meeting #130                                                                                     R2-2504462

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.2.2

Source:	HONOR

Title:	LCM for UE-side models for beam management

Document for:	Discussion and Decision

- Introduction

In RAN2#129-bis meeting, RAN2 made the following agreements on UE-side model for BM case, including the signalling details for Option B, cause of non-applicability report, inference configuration handling upon non-applicability and UE-side data collection configuration.

Agreements on option B

- RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list).

Potential aspects to consider if RAN2 revisit:

- To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig.

- to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior.

Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability. FFS how to define this simple cause related to model availability and how we capture it in the spec.

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced.

Agreements on data collection configuration

- The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

- An indication on start/stop of data collection.

- Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In this contribution, we will discuss the remaining issues on LCM framework for UE-side model for beam management, including the signaling details for Option B and applicability reporting.

- Discussion

  - The signalling details for Option B

2.1.1 (Open issue RRC-7) Applicability reporting for option B in RRCReconfigurationComplete

RAN2#129 agreements [1] have disclosed that at least for Option A, the initial applicability report and subsequent applicability report upon applicability changes are piggybacked by RRCReconfigurationComplete and UAI separately. Undoubtedly, both initial and subsequent reports indicate the applicability and/or inapplicability of CSI-ReportConfig(s) for Option A. However, it remains unclear whether the reporting content/granularity of initial and subsequent reports are consistent for Option B.

RAN2#129 Agreement • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

Benefit from the contribution from several companies [2] for last meeting, the primary signaling processes of Option A and Option B have been progressively clarified. Before discussing the signaling details of Option B, let’s firstly review how Option A works. Figure 1 serves as a procedure example for Option A.

Figure 1

Procedures description for Option A:

Step 3: UE receives RRCReconfiguration message including one set or multiple sets of full inference configuration(s) (e.g., one or more CSI-ReportConfig) and UEAssistanceInformation (UAI) report configuration for subsequent applicability report of CSI-ReportConfig (e.g., via OtherConfig).

Step 4: For initial reporting, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability, e.g. including one or more CSI-ReportConfigId for applicability and/or one or more CSI-ReportConfigId for inapplicability.

NOTE: If the initial reporting indicates that all the CSI-ReportConfig provided in Step 3 are unavailable, Steps 3 and 4 may be repeated.

Step 5: For subsequent reporting triggered by applicability changes, UE sends UAI message including applicability and/or inapplicability, e.g., including one or more CSI-ReportConfigId for applicability and/or one or more CSI-ReportConfigId for inapplicability.

It is clear that the reporting contents of both initial and subsequent reports for Option A pertain to the applicability/inapplicability of full inference configurations (i.e., CSI-ReportConfig). For Option B, considering the co-existence of full inference configurations and inference related parameters, it remains unclear that whether the initial and subsequent reports have consistent reporting content. Therefore, we provide three possibilities of Option B’s procedures.

- Alt-1: Initial and subsequent reports have consistent reporting content, i.e., the applicability/inapplicability of inference related parameters.

Figure 2

Procedures description for Option B:

Step 3: UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters and UAI report configuration (for subsequent applicability reporting of inference related parameters) via OtherConfig.

Step 4: For initial reporting, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability for each set of inference related parameters.

Step 5: UE receives RRCReconfiguration message including one set or multiple sets of full inference configuration(s), e.g., one or more CSI-ReportConfig.

NOTE: In this case, NW does not require UE to actively report when the availability of full inference configuration(s) (i.e., CSI-ReportConfig) changes.

Step 6: UE sends RRCReconfigurationComplete message, including optional applicability and/or inapplicability report, e.g., including one or more CSI-ReportConfigId for applicability and/or one or more CSI-ReportConfigId for inapplicability.

Step 7: For subsequent reporting upon change: UE sends UAI message including applicability and/or inapplicability of inference related parameters provided in Step 3.

- Alt-2: Initial and subsequent reports have different reporting contents. That is, the initial report includes the applicability/inapplicability of inference related parameters, while the subsequent report includes the applicability /inapplicability of both inference related parameters and full inference configurations.

Figure 3

Procedures description for Option B:

Step 3: UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters and UAI report configuration (for subsequent applicability reporting of inference related parameters) via OtherConfig.

Step 4: For initial reporting, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability for each set of inference related parameters.

Step 5: UE receives RRCReconfiguration message including one set or multiple sets of full inference configuration(s) (e.g., one or more CSI-ReportConfig) and UAI report configuration for subsequent applicability report of CSI-ReportConfig (e.g., via OtherConfig).

NOTE: In this case, the Step 5 for Option B is the same as the Step 3 for Option A.

Step 6: UE sends RRCReconfigurationComplete message, including optional applicability and/or inapplicability report, e.g., including one or more CSI-ReportConfigId for applicability and/or one or more CSI-ReportConfigId for inapplicability.

Step 7-1: For subsequent reporting upon change: UE sends UAI message including applicability and/or inapplicability of inference related parameters provided in Step 3.

Step 7-2: For subsequent reporting upon change: UE sends UAI message including applicability and/or inapplicability of full inference configuration(s) provided in Step 5.

NOTE: In this case, the UE will report the availability changes for both the inference related parameters provided in Step 3 and the full inference configurations provided in Step 5.

- Alt-3: Initial and subsequent reports have different reporting contents. That is, the initial report includes the applicability/inapplicability of inference related parameters, while the subsequent report includes the applicability /inapplicability of full inference configurations.

Figure 4

Procedures description for Option B:

Step 3: UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig.

NOTE: In this case, UAI report configuration for subsequent applicability report is not included in Step 3.

Step 4: For initial reporting, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability for each set of inference related parameters.

Step 5: UE receives RRCReconfiguration message including one set or multiple sets of full inference configuration(s) (e.g., one or more CSI-ReportConfig) and UAI report configuration for subsequent applicability report of CSI-ReportConfig (e.g., via OtherConfig).

Step 6: UE sends RRCReconfigurationComplete message, including optional applicability and/or inapplicability report, e.g., including one or more CSI-ReportConfigId for applicability and/or one or more CSI-ReportConfigId for inapplicability.

Step 7: For subsequent reporting upon change: UE sends UAI message including applicability and/or inapplicability of full inference configuration(s) provided in Step 5.

NOTE: In this case, the Steps 5-7 for Option B are the same as the Step 3-5 for Option A.

We think three alternatives can work, but we prefer Alt-3 due to the following reasons:

- Alt-1: Before UE reports that one set of inference related parameters become unavailable, one or more CSI-reportConfig(s) currently in use and configured based on these inference related parameters may already be unavailable. In this case, the NW is unable to detect this issue in time and reconfigure or activate other CSI-reportConfig(s) for UE.

- Alt-2: UE needs to monitor the applicability of both inference related parameters and full inference configurations, potentially resulting in more frequent UAI messages for applicability updates.

- Alt-3: Since NW has configured the specific full inference configuration (i.e., CSI-ReportConfig), it’s more likely that NW is more concerned about the availability changes of these specific configurations. Moreover, for Alt-3, the procedure of Option A can be entirely embedded into that of Option B after checking for the applicability by using rough parameter settings of Set A and B.

Proposal 1: (Open issue RRC-7) Considering the potential differences between initial and subsequent reports, RAN2 to further discuss the three possible procedures of Option B. It’s suggested that initial report for the applicability/inapplicability of inference related parameters and subsequent report for that of full inference configuration (Alt 3 of Option B) can be considered as a starting point.

2.1.2 (Open issue RRC-8) Consistency of Option A and Option B

Based on the above analysis, we list the signaling details for Option A and Option B’s three alternatives in the table below.

For Option B, if UE is configured to report the applicability updates of CSI-ReportConfig (i.e., Option B’s Alt-2 and Alt-3), NW will send the same RRCReconfiguration message as Step 3 of Option A. Thus, UE may be confused whether to report applicability/inapplicability again in Step 6 for Option B. To avoid redundancy reports, a proposed solution is to add an indicator within RRCReconfiguration to assist UE to distinguish Option A or B the NW applies. However, we think it’s also feasible to continue the following procedures without this indicator. That is, UE takes the same behavior whatever Option A or Option B.

Our view is based on the following reasons. First, UE will inevitably send RRCReconfigurationComplete after receiving RRCReconfiguration. Thus, UE reporting applicability again will not cost significant signaling overhead. Second, considering that UE has provided some inference parameter suggestions (via applicability/non-applicability report) to gNB for reference, NW may configure fewer sets of full inference configuration(s) compared to Step 3 of Option A. Therefore, even if UE reports applicability/non-applicability again, the report content would be small (e.g., a small list of CSI-ReportConfigId for applicability/inapplicability). Moreover, though RAN1 has not decided which IEs are included in the set of inference related parameters, in our view, inference related parameters may only involve part of configuration information of Set A and B (e.g., subset of IEs in CSI-ReportConfig) or just a rough configuration reference. Thus, NW may still configure unsuitable full inference configurations even the applicability of inference related parameter is provided. Thus, it’s necessary for UE to report the applicability of CSI-ReportConfig from a functional perspective.

Proposal 2: (Open issue RRC-8) For Option B, after receiving full inference configurations via CSI-ReportConfig in RRCReconfiguration message, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability report of CSI-ReportConfig.

| Alternative Procedures | NW -> UE | UE -> NW | UAI report content |
| --- | --- | --- | --- |
| Option A | RRCReconfiguration including (1) full inference configuration(s), e.g., one or more CSI-ReportConfig (2) UAI report configuration (for subsequent applicability report of CSI-ReportConfig) | RRCReconfigurationComplete, including mandatory applicability and/or inapplicability report of CSI-ReportConfig | applicability / inapplicability of CSI-ReportConfig |
| Option B: Alt-1 | RRCReconfiguration including (1) inference related parameters via OtherConfig; (2) UAI report configuration (for subsequent applicability reporting of inference related parameters) | RRCReconfigurationComplete including applicability and/or inapplicability for inference related parameters | applicability / inapplicability of inference related parameters |
|  | RRCReconfiguration including (1) full inference configuration(s) via CSI-ReportConfig | RRCReconfigurationComplete, including optional applicability and/or inapplicability report of CSI-ReportConfig |  |
| Option B: Alt-2 | RRCReconfiguration including (1) inference related parameters, (2) UAI report configuration (for subsequent applicability report of inference related parameters) | RRCReconfigurationComplete including applicability and/or inapplicability for inference related parameters | applicability / inapplicability of inference related parameters and CSI-ReportConfig |
|  | RRCReconfiguration including (1) full inference configuration(s), (2) UAI report configuration (for subsequent applicability report of CSI-ReportConfig) | RRCReconfigurationComplete, including optional applicability and/or inapplicability report of CSI-ReportConfig |  |
| Option B: Alt-3 | RRCReconfiguration including - inference related parameter(s) | RRCReconfigurationComplete, including applicability and/or inapplicability for inference related parameters | applicability / inapplicability of CSI-ReportConfig |
|  | RRCReconfiguration including (1) full inference configuration(s), (2) UAI report configuration (for subsequent applicability report of CSI-ReportConfig) | RRCReconfigurationComplete, including optional applicability and/or inapplicability report of CSI-ReportConfig |  |

  - Applicability reporting

2.2.1 (Open issue RRC-1) Cause of inapplicability

It was agreed in RAN2#129-bis meeting that UE may indicate a simple cause value of inapplicability. From RAN2#127 agreement, UE may decide its applicability based on NW-side additional condition (e.g., mapping relationship of Set A and Set B, the order of model input and output, etc), UE-side additional condition (e.g., power, buffer, etc) and model availability. Consequently, the reasons for different full inference configurations and/or inference related parameters becoming unavailable may also vary.

In our opinion, if an inapplicability reporting indicates several inference configurations become inapplicable, they can share a common cause of inapplicability (e.g., low power or UE moving to a new cell), or UE can indicate each inapplicable inference configuration with a different cause.

RAN2#129-bis Agreement on applicability reporting

Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability. FFS how to define this simple cause related to model availability and how we capture it in the spec.

Proposal 3: (Open issue RRC-1) When an inapplicability reporting indicates several inference configurations become inapplicable, they can share a common cause of inapplicability (e.g., low power or UE moving to a new cell), or UE can specify different causes for each configuration.

  - Other Open issues

Functionality identification

Recalling that the applicability/inapplicability reporting in Step 4 of Option B is for each set of inference related parameters. Thus, an index or ID is needed to identify inference related parameters provided in Step 3. Moreover, we noticed that RAN1#119 agreement has disclosed that for Option B, associated ID can be optionally indicated by NW to UE along with inference related parameters. This raises a question: whether associated ID can be used to indicate each set of inference related parameters, if configured in Step 3.

In our view, the associated ID cannot effectively distinguish between different sets of inference related parameters. Associated ID corresponds to the physical conditions or configurations on the NW side (e.g., transmission power, down tilt angles, beam codebook, etc.) under which UE measures, collects data, and trains the model. For example, UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID [3]. Thus, multiple sets of inference related parameters may share the same associated ID while containing different Set A/B related information and/or report content related information. Additionally, RAN1#120 agreed that two associated IDs can be configured separately for Set A and Set B. Therefore, the associated ID is not suitable for differentiating between various sets of inference-related parameters.

Proposal 4: For applicability reporting under Option B, an index or ID is introduced to distinguish each set of inference related parameters provided in Step 3.

RAN1#119 Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction RAN1#120 Agreement For UE-sided model, in CSI-ReportConfig for inference - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

- Conclusions

In this contribution, we discussed several remaining issues on LCM procedure of UE-sided model for AI/ML based beam management and made the following proposals:

Proposal 1: (Open issue RRC-7) Considering the potential differences between initial and subsequent reports, RAN2 to further discuss the three possible procedures of Option B. It’s suggested that initial report for the applicability/inapplicability of inference related parameters and subsequent report for that of full inference configuration (Alt 3 of Option B) can be considered as a starting point.

Proposal 2: (Open issue RRC-8) For Option B, after receiving full inference configurations via CSI-ReportConfig in RRCReconfiguration message, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability report of CSI-ReportConfig.

Proposal 3: (Open issue RRC-1) When an inapplicability reporting indicates several inference configurations become inapplicable, they can share a common cause of inapplicability (e.g., low power or UE moving to a new cell), or UE can specify different causes for each configuration.

Proposal 4: For applicability reporting under Option B, an index or ID is introduced to distinguish each set of inference related parameters provided in Step 3.

- References

- RAN2#129, Chair Notes.

- R2-2501783, Discussion on Applicable Functionality Reporting Option B for BM.

- R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model.