3GPP RAN WG2 Meeting #130	R2-2504464

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.3

Source:	HONOR

Title:	Discussion on NW side data collection

Document for:	Discussion and Decision

1. Introduction

In Rel-19 AI/ML for NR air interface WID, the following objectives were achieved:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models

In RAN2#129-bis meeting, the following agreements were made:

- Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details. - The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc) - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit

In this contribution, we discuss the remaining issues on NW side data collection.

2. Discussion

2.1 (issue RRC-18) NW control on retaining logged data at HO

In RAN2#129-bis, it was agreed, the measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g., BM, Mobility, etc). It means that multiple functionalities/use cases related measurement configurations are configured for data collection. When a HO occurs, for one certain functionality/ use case/ measurement configuration, the NW may have completed the data collection or the related model has completed the model training. Therefore, to enable more granular management, it should be clearly indicated whether to release or retain un-retrieved data based on each functionality, use case, or measurement configuration.

Proposal 1: (issue RRC-18) It is suggested to indicate release or retain un-retrieved data per functionality/ use case/ measurement configuration.

If the UE is indicated to retain un-retrieved data in HO command, while accessing to the target gNB, the UE needs to indicate the availability of collected data. Such indication can be included in RRCReconfigurationComplete message.

Proposal 2: (issue RRC-18) While accessing to the target gNB, the UE indicates the availability of collected data in RRCReconfigurationComplete message.

2.2 (issue RRC-21) Time related content of logged data

The issue whether to report a timestamp was raised, and no consistent conclusion has been reached. In general, it is necessary to introduce a timestamp to indicate when the data is logged or measured, especially for temporal prediction. This can be regarded as the necessary information for model training. For instance, the data logged or measured at time t1 and the data logged or measured at time t2 can be regarded as model input and output respectively for model training.

Proposal 3: (issue RRC-21) A timestamp should be reported for logged data.

If Proposal 3 is agreed, how to report timestamp in one report should be taken into account. On this issue, the understandability of the timestamp and the signaling load need to be considered. We think that at least the absolute timestamp of the first sample needs to be provided. If the data logging interval is configured, the relative timestamp of the subsequent samples can be deduced by the configured interval. If the data logging interval is not configured, the relative timestamp of the subsequent samples can be derived from the periodicity of configured CSI-RS resource.

Proposal 4: (issue RRC-21) The absolute timestamp of the first sample should be indicated and the relative timestamp of the subsequent samples can be deduced based on the configured data logging interval or the configured CSI-RS resource periodicity.

2.3 (issue RRC-25) Dynamic activation/deactivation of data collection configurations for logging

For periodic data logging, data logging will start after receiving the measurement configuration of AI/ML data collection. For event-triggered data logging, data logging will start if the configured condition is satisfied. In the previous meeting, it was agreed that the measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g., BM, Mobility, etc). If NW expects to deactivate data logging for a specific set of resources or a particular use case, the corresponding measurement configuration of AI/ML data collection can be modified or released. Therefore, we think it is unnecessary to introduce L1/L2 for dynamic activation/deactivation.

Proposal 5: (issue RRC-25) Dynamic activation/deactivation for data logging via L1/L2 signalling is not supported.

2.4 (issue RRC-27) How to set logging periodicity

Similar to MDT, the data logging interval can be configured in the configuration of data collection for both periodic data logging and event triggered data logging. If the data logging interval is not configured, the UE will perform data logging by default in the periodicity of CSI-RS resource.

Proposal 6: (issue RRC-27) The data logging interval can be configured for both periodic data logging and event triggered data logging.

2.5 (other issue) The content of logged data

In the configuration of data collection, the associated ID may be included. If the associated ID is provided, the UE will perform data collection corresponding to the associated ID. And the associated ID should be reported together with logged data for filtering data by NW. For instance, the NW can categorize the collected data based on the associated ID and handle the corresponding model training.

Proposal 7: The associated ID configured in training data configuration can be reported together with the logged data.

In addition to associated ID, the UE side additional condition can also be used for filtering data. Therefore, the UE side additional condition needs to be reported. The issue regarding how to report this information should be considered. In our opinion, the UE side additional condition can be reported separately. For instance, the UE side additional condition can be sent via UAI message. Subsequently, UE can report the corresponding data. Alternatively, the UE side additional condition can be reported together with the logged data. However, this will bring about a relatively large signaling overhead. From this perspective, the way of reporting UE side additional condition separately is preferred.

Proposal 8: For logged data report, the UE side additional condition can be reported separately.

3. Conclusions

In this contribution, we discussed the issues related to NW side data collection. Based on the discussion, the following proposals are concluded:

Proposal 1: (issue RRC-18) It is suggested to indicate release or retain un-retrieved data per functionality/ use case/ measurement configuration.

Proposal 2: (issue RRC-18) While accessing to the target gNB, the UE indicates the availability of collected data in RRCReconfigurationComplete message.

Proposal 3: (issue RRC-21) A timestamp should be reported for logged data.

Proposal 4: (issue RRC-21) The absolute timestamp of the first sample should be indicated and the relative timestamp of the subsequent samples can be deduced based on the configured data logging interval or the configured CSI-RS resource periodicity.

Proposal 5: (issue RRC-25) Dynamic activation/deactivation for data logging via L1/L2 signalling is not supported.

Proposal 6: (issue RRC-27) The data logging interval can be configured for both periodic data logging and event triggered data logging.

Proposal 7: The associated ID configured in training data configuration can be reported together with the logged data.

Proposal 8: For logged data report, the UE side additional condition can be reported separately.

4. References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface