3GPP RAN WG2 Meeting #130	R2-2504469

# St Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.4.4

Source:	HONOR

Title:	Discussion on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

1. Introduction

In RAN2#129bis meeting, the following agreements on procedures for LP-WUS in RRC_CONNECTED were made [1]:

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback. Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously. If configured, the UE can signal a preferred time offset via UAI signalling. Ask RAN1 for further information regarding their conclusions. FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. For NR-DC, the LP-WUS in MCG and SCG can be configured independently. Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC

In this contribution, we will discuss the fallback mechanism of LP-WUS monitoring.

2. Discussion

2.1. (MAC-2) Fallback of LP-WUS monitoring

One open issue regarding Option 1-2 is as follows.

Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Since LP-WUR detection performance is not as good as MR, it may miss to detect or falsely detect LP-WUS In some scenarios. In this case, gNB cannot wake up UE to monitor PDCCH. Although gNB can determine whether UE correctly receives LP-WUS by HARQ feedback, UE is not aware that whether it is missed detection or just because NW does not send anything. So, fallback mechanism may be considered to avoid misalignment between UE and gNB. For example, for periodic data transmission, gNB may configure a parameter N. UE should fall back to the legacy CDRX monitoring mechanism if it does not receive any LP-WUS targeted to itself within N LP-WUS cycles.

Proposal 1: (MAC-2) For Option 1-2, UE should fall back to the legacy CDRX monitoring mechanism if it does not receive any LP-WUS targeted to itself within N LP-WUS cycles.

3. Conclusions

In this contribution, we discussed fallback mechanism of LP-WUS monitoring. The proposal is summarised as below.

Proposal 1: (MAC-2) For Option 1-2, UE should fall back to the legacy CDRX monitoring mechanism if it does not receive any LP-WUS targeted to itself within N LP-WUS cycles.

4. References

[1] Report of 3GPP TSG RAN WG2 meeting #129bis