3GPP TSG RAN WG2 #130				R2-2504490

St. Julians, Malta, May. 19th – 23rd, 2025

Agenda Item:	8.2.3

Source:		ASUSTeK

Title:				Discussion on Ambient IoT random access

Document for: Discussion and Decision

- Introduction

In RAN2#129bis meeting [1], the following agreements on A-IoT random access were made.

Agreements on msg1

1. In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

2. RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Agreements on NACK

1	For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

2	For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

3	For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

4	FFS on end of procedure

In this contribution, we would discuss the open issues in the A-IoT random access procedure.

- Discussion

2.1 	Msg1 resource selection

From previous RAN2 meetings, it was agreed that the A-IoT paging message can include a number of Msg1 resources, and a new R2D trigger message is used to determine Msg1 resources by indicating the start of a set of Msg1 resources that were configured in the paging message. However, the exact method for the device to select Msg1 resource has not been specified.

On the other hand, RAN1 has agreed that TDMA and FDMA of D2R transmissions for Msg1 are supported. For TDMA, X=1 and X=2 are supported for Msg1 transmission. For each set of Msg1 resources, there could be 1 or 2 time domain resources. From RAN2 perspective, whether the time domain resources information (e.g., X) is included in the paging message or the R2D trigger message needs discussion. We propose to include the time domain resources information in the R2D trigger message, which could bring flexibility in resource scheduling for the reader by assigning different numbers of time resources in each access round. For example, when the reader detects serious Msg1 collision in the beginning access rounds after sending the paging message, the reader could set X=2 to accommodate more devices in the following access rounds.

Proposal 1:  The information of time domain resources is included in the R2D trigger message.

If Proposal 1 is agreed, a preferred way for Msg1 resource selection is that the paging message includes the total number of access rounds (e.g., N), and the R2D trigger message includes the number of time domain resources within one access round (e.g., X). The device would randomly select a number (e.g., n) based on N to determine which access round to access (1 ≤ n ≤ N). And after receiving the R2D trigger message indicating the n-th access round, the device would randomly select another number (e.g., x) based on X to determine which time domain resource to access (1 ≤ x ≤ X).

Proposal 2:  For Msg1 resource selection, the device randomly selects an access round from the total number of access rounds included in the paging message, and randomly selects a time domain resource from the number of time domain resource within the access round included in the R2D trigger message for the access round.

2.2  NACK indication

In TR 38.769 [2], the support of re-access is captured.

It is supported for the A-IoT device to re-access in another opportunity controlled/provided by the reader (i.e., to retry the random access above), in case of D2R data transmission failure and contention resolution failure of contention-based random access. The A-IoT device is not expected to autonomously re-access. The re-access is always controlled by reader. It is supported for reader to use the optional explicit R2D failure/success feedback indication to determine the re-access of device: - This indication can be used at least to determine the re-access for addressing the transmisison failure of the first D2R message, which contains the device ID and/or any other upper layer data (i.e. "Msg3"); - This indication can be also used for the following D2R data (as described in clause 6.3.5), to determine the re-access for addressing the transmisison failure. The R2D message is used by the reader to provide access occasion(s), which can be used for re-access purpose. It needs to be further discussed if additional information is needed in this R2D message to differentiate the re-access purpose. - A-IoT paging message is one of the options for this R2D message (e.g., see the "subsequent A-IoT paging message" in Figure 6.3.4-1). - Another option can be some R2D messages between A-IoT paging (e.g., see the other "R2D message" in Figure 6.3.4-1).

In addition, it is agreed in RAN2#129 meeting that NACK based mechanism is supported for Msg3 to determine re-access. However, the design of NACK indication is still undecided, i.e., in which message should the NACK indication be included.

Option 1: introducing a new message to include NACK indication.

Option 2: reusing paging message to include NACK indication.

Option 3: reusing R2D trigger message to include NACK indication.

It appears to be straightforward to design a separate message for NACK, and the device could realize transmission failure upon receiving this message. However, always transmitting a separate message would cause signalling overhead and power consumption to the devices.

On the other hand, Option 2 doesn’t seem to be so reasonable considering the usage of paging message. Note that the determination of re-access is whether the device receives NACK indication before a subsequent R2D message; also, re-access is triggered by subsequent paging message. The device should be aware of failure indication before performing re-access, which is not the case for containing NACK in a paging message.

For Option 3, it seems to be a feasible solution comparing with Option 2. There would be several R2D trigger messages within a paging round. If the device receives a R2D trigger message including NACK indication for itself before the subsequent paging, the device considers the random access failed and determines to re-access in the next paging round. Nonetheless, this approach requires to include an ID field(s) and a bit for NACK indication within the R2D trigger message.

Based on the above analysis, we prefer Option 3 for NACK indication.

Proposal 3: The R2D trigger message could include NACK indication.

- Conclusion

Proposal 1:  The information of time domain resources is included in the R2D trigger message.

Proposal 2:  For Msg1 resource selection, the device randomly selects an access round from the total number of access rounds included in the paging message, and randomly selects a time domain resource from the number of time domain resource within the access round included in the R2D trigger message for the access round.

Proposal 3:  The R2D trigger message could include NACK indication.

- Reference

[1] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China

[2] TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”