3GPP TSG RAN WG2 #130				R2-2504500

Malta, MT, May 19 – 23, 2025

Agenda Item:	8.12.3 Others (NR_MIMO_Ph5-Core)

Source:		ASUSTeK

Title:				Discussion on MAC impacts for UEI Beam Reporting Mode B

Document for: Discussion and Decision

- Introduction

In this document, we discuss some impacts for UEI Beam Reporting Mode B in MAC spec.

- Discussion

How to Prevent MAC PDU Generation on a CG Type 1

According to the current MAC spec, when RRC configures a CG Type 1, the UE performs the following steps to generate a MAC PDU based on the CG Type 1 for transmission:

Step 1:  Initialise the CG Type 1 to reoccur with a periodicity (in clause 5.8.2)

Step 2:  Set the HARQ information and deliver the CG Type 1 to HARQ entity (in clause 5.4.1)

Step 3:  Attempt to obtain a MAC PDU (in clause 5.4.2.1)

Step 4:  Obtain the MAC PDU (in clause 5.4.3)

Step 5:  Instruct the physical layer to generate a transmission (in clause 5.4.2.2)

Since RAN2 has confirmed that a CG Type 1 PUSCH carrying the Mode-B beam report does not carry a MAC PDU (i.e., UL-SCH), the goal in this case is not to generate a MAC PDU for such CG Type 1 transmissions. To achieve this, five possible options can be considered, each corresponding to stopping the procedure at one of the five steps. Logically, it is preferable to stop as early in the process as possible; therefore, we begin analysis from Step 1.

In Step 1, the CG Type 1 is initialized to ensure it reoccurs according to the configured periodicity and other timing parameters. This step is essential and cannot be omitted, as the physical layer relies on this timing information to determine when the CG can be used for UEI beam reporting.

Observation:  Initializing CG Type 1 with periodicity is necessary and serves as a common prerequisite for the physical layer to determine the timing for UEI beam reporting.

In Step 2, for each configured uplink grant, the UE sets the HARQ information and delivers the CG Type 1 to the HARQ entity. However, HARQ operation is not required when transmitting the UEI beam report via CG Type 1. Therefore, Step 2 is the most appropriate step to stop in this context. Additionally, the corresponding impact on the specification is minimal, as shown in text proposal below.

Proposal:  Add a restriction to clause 5.4.1 to reflect the RAN2 understanding that CG Type 1 PUSCH carrying the Mode-B beam report does not carry a MAC PDU (i.e., UL-SCH).

### Text Proposal

### 5.4.1 UL Grant reception … For each Serving Cell and each configured uplink grant not associated with configuredPUSCHResourceOfModeB, if configured and activated and available for use as specified in clause 5.8.2, the MAC entity shall: …

### The current procedure from CG Type 1 grant to MAC PDU generation

### 5.8 Transmission and reception without dynamic scheduling ### 5.8.2 Uplink … Upon configuration of a configured grant Type 1 for a BWP of a Serving Cell by upper layers, the MAC entity shall: 1> store the uplink grant provided by upper layers as a configured uplink grant for the indicated BWP of the Serving Cell; 1> if cg-SDT-PeriodicityExt is configured: 2> … 1> else: 2> initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset, timeReferenceSFN, and S (derived from SLIV or provided by startSymbol as specified in TS 38.214 [7]), and to reoccur with periodicity. … If cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured, after an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

## 5.4 UL-SCH data transfer ### 5.4.1 UL Grant reception … For each Serving Cell and each configured uplink grant, if configured and activated and available for use as specified in clause 5.8.2, the MAC entity shall: … 2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration; … 4> consider the NDI bit for the corresponding HARQ process to have been toggled; 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

### 5.4.2 HARQ operation #### 5.4.2.1 HARQ Entity ## … For each uplink grant, the HARQ entity shall: 1> identify the HARQ process associated with this grant, and for each identified HARQ process: 2> if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or … 4> obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any; 3> if a MAC PDU to transmit has been obtained: … 5> deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process; 5> instruct the identified HARQ process to trigger a new transmission; …

### 5.4.3 Multiplexing and assembly … ##### 5.4.3.1.3 Allocation of resources … The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and … 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and … 3> not generate a MAC PDU for the HARQ entity. ### …

#### 5.4.2.2 HARQ process … To generate a transmission for a TB, the HARQ process shall: … 3> instruct the physical layer to generate a transmission according to the stored uplink grant.

- Conclusion

In this document, we have the following proposals for UEI Beam Reporting Mode B.

Observation:  Initializing CG Type 1 with periodicity is necessary and serves as a common prerequisite for the physical layer to determine the timing for UEI beam reporting.

Proposal:  Add a restriction to clause 5.4.1 to reflect the RAN2 understanding that CG Type 1 PUSCH carrying the Mode-B beam report does not carry a MAC PDU (i.e., UL-SCH).

### Text Proposal

### 5.4.1 UL Grant reception … For each Serving Cell and each configured uplink grant not associated with configuredPUSCHResourceOfModeB, if configured and activated and available for use as specified in clause 5.8.2, the MAC entity shall: …

Reference

Annex