3GPP TSG-RAN2 Meeting # 130	R2-2504502

# Malta, MT,  19 – 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5321 | rev | 2 | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on sl-PRS-ResourcePoolID for SL-PRS CG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK, Ericsson, Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2 #123bis (2023-10), it was agreed that CG confirmation MAC CE is needed when the DCI for CG type 2 activation/deactivation command is successfully received. Decide on the issue of whether to reuse the legacy Sidelink Configured Grant Confirmation MAC CE when the CG configurations are provided by RAN1. In RAN1 #115 (2023-11), it was agreed that The total number of SL configured grants (including both Type1 and Type2) at a UE across all resource pools is not larger than 8. In R2-2401365 (RRC Positioning RIL List) (2024-02), the following comments (a.b.c.) were made and directly incorporated into the Rapporteur CR. (R2-2401318 / R2-2401632). - V815 - Distinguish CG Type 1 configurations using a unified parent IE. - H606 - sl-ConfigIndexCG of CG for SL-PRS on dedicated pool should not be duplicated with sl-ConfigIndexCG of CG for SL data. Reason: The CG for dediated resrouce pool for SL-PRS should share the same indexing space as the legacy. That is the reason why we can reuse the legacy CG confirmation MAC CE. - H607 – Similar clarification as H606. Reason: The resource pool index should indicate a dedicated resource pool. For shared pool, can use the legacy configuration. - For H607, since SL-PRS resource pools and SL resource pools are configured separately (i.e. sl-PRS-PoolToAddModList-r18 and sl-PoolToAddModList-r16), and no common “confirmation” (as stated in H606) or procedure applied to both types of resource pools, the network restriction on setting sl-PRS-ResourcePoolID and sl-ResourcePoolID in CG type 1 configuration is unnecessary and should be removed. - The SL-ResourcePoolID-r16 uses 4 bits, while the SL-PRS-ResourcePoolID-r18 uses 3 bits. Therefore, it should be clarified that the maximum resource pool ID is 8. - Since there is no activation command indicating resource pool ID for SL-PRS CG type 1, the network should always configure sl-PRS-ResourcePoolID when SL-PRS CG type 1 is configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the network restriction on configuring sl-PRS-ResourcePoolID. - Clarify that the field value is configured with maximum value 8 (maxNrofSL-PRS-TxPool-r18). - Clarify that the network always configures sl-PRS-ResourcePoolID when rrc-ConfiguredSidelinkGrantDedicated-SL-PRS-RP is included. Inter-Operability analysis: Impacted Architecture NR SA Impacted functionality: SL positioning Inter-operability: If the UE is implemented according to this CR while the network is not, or If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecesary network restriction for setting sl-PRS-ResourcePoolID and sl-ResourcePoolID in CG type 1 configuration exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2502815, R2-2502990 |  |  |  |  |  |  |  |  |

6.3.5	Sidelink information elements

…

#### –	SL-ConfiguredGrantConfigDedicated-SL-PRS-RP

The IE SL-ConfiguredGrantConfigDedicated-SL-PRS-RP specifies the configured grant configuration information for NR sidelink positioning in a dedicated SL-PRS resource pool.

SL-ConfiguredGrantConfigDedicated-SL-PRS-RP information element

-- ASN1START

-- TAG-SL-CONFIGUREDGRANTCONFIGDEDICATEDSL-PRS-RP-START

SL-ConfiguredGrantConfigDedicatedSL-PRS-RP-r18 ::=     SEQUENCE {

sl-PRS-ConfigIndexCG-r18                               SL-ConfigIndexCG-r16,

sl-PRS-PeriodCG-r18                                    SL-PeriodCG-r16                                         OPTIONAL, -- Need M

sl-PRS-ResourcePoolID-r18                              SL-ResourcePoolID-r16                                   OPTIONAL, -- Need M

rrc-ConfiguredSidelinkGrantDedicated-SL-PRS-RP-r18     SEQUENCE {

sl-TimeOffsetCG-Type1-r18                              INTEGER (0..7999)                                   OPTIONAL, -- Need R

sl-TimeReferenceSFN-Type1-r18                          ENUMERATED {sfn512}                                 OPTIONAL, -- Need S

sl-TimeResourceCG-Type1-r18                            INTEGER (0..496)                                    OPTIONAL, -- Need M

sl-PRS-ResourceIndicationFirstType1-r18                INTEGER(0..11)                                      OPTIONAL, -- Need M

sl-PRS-ResourceIndicationFutureType1-r18               INTEGER(0..143)                                     OPTIONAL  -- Need M

}

}

-- TAG-SL-CONFIGUREDGRANTCONFIGDEDICATEDSL-PRS-RP-STOP

-- ASN1STOP

SL-ConfiguredGrantConfigDedicated-SL-PRS-RP field descriptions

sl-PRS-ConfigIndexCG This field indicates the ID to identify sidelink configured grant. The field value should not be duplicated with sl-ConfigIndexCG in IE SL-ConfiguredGrantConfig.

sl-PRS-PeriodCG This field indicates the period of SL PRS configured grant in a dedicated resources in ms for either CG type 1 or CG type 2.

sl-PRS-ResourceIndicationFirstType1 Indicates SL-PRS Resource ID for the first SL-PRS transmission.

sl-PRS-ResourceIndicationFutureType1 Indicates SL-PRS resource IDs for future SL PRS transmissions. An index giving valid combinations of up to two SL PRS resource IDs (jointly encoded) .

sl-PRS-ResourcePoolID Indicates the resource pool in which the configured sidelink grant Type 1 is applied. This field value is configured with maximum value 8 (maxNrofSL-PRS-TxPool-r18). The network always configures this field when rrc-ConfiguredSidelinkGrantDedicated-SL-PRS-RP is included.

sl-TimeOffsetCG-Type1 This field indicates the slot offset with respect to logical slot defined by sl-TimeReferenceSFN-Type1-Dedicated-SL-PRS-RP, as specified in TS 38.321 [3].

sl-TimeReferenceSFN-Type1 Indicates SFN used for determination of the offset of a resource in time domain. If it is present, the UE uses the 1st logical slot of associated resource pool after the starting time of the closest SFN with the indicated number preceding the reception of the sidelink configured grant configuration type 1 as reference logical slot, see TS 38.321 [3], clause 5.8.3. If it is not present, the reference SFN is 0.

sl-TimeResourceCG-Type1 This field indicates the time resource location of sidelink configured grant type 1. An index giving valid combinations of up to two slot positions (jointly encoded) as time resource indicator value (TRIV), as defined in TS 38.212 [17].