3GPP TSG-RAN WG2 #130	R2-2504518

St.Julians, Malta, 19th – 23rd Apr. 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on DSR enhancements for XR

Document for:        	Discussion and decision

Agenda Item:         	8.7.4.2		DSR enhancements

- Introduction

In RAN #107, revised WID on Rel-19 XR enhancements was approved [1], and the following objective was specified regarding scheduling enhancements.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

And the following agreements related to DSR enhancements were reached in RAN2#129bis meeting.

Agreements on DSR enhancements - To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

In this contribution, we provide the discussion on DSR enhancements for XR.

- Discussion

In RAN2#129bis meeting, RAN2 discussed data volume calculation for Rel-19 DSR. However, it is still FFS which delay-reporting PDCP data volume shall consider control PDUs and data to be retransmitted. The following options have been considered.

- Option 1: Data volume associated with the smallest reported dsr-ReportingThreshold.

- Option 2: Data volume associated with the smallest configured dsr-ReportingThreshold (i.e., i=1).

For option 1, the UE needs to determine which dsr-ReportingThreshold the control PDU and the data to be retransmitted are associated with, based on the smallest remaining time among other data buffered for the LCG. If more than one LCH belongs to the LCG, the UE may need to consider the remaining time among multiple PDCP entities, which could increase the UE complexity. In addition, option 1 still has the issue that if the LCG only has the control PDU and/or the data to be retransmitted, it is not clear in which portion those data should be reported. There may be an opinion that the case where the LCG only has such data cannot occur because the DSR is triggered based on the remaining time falling below the triggering threshold. However, it is possible that only such data is buffered for LCHs other than the LCH that triggered the DSR. Option 2 is a simple solution because the control PDU and the data to be retransmitted can always be associated with the first dsr-ReportingThreshold, regardless of the remaining time of other data buffered for the LCG. Compared to option 1, the overhead may be increased due to the additional field for those data, but its size is marginal. From the above perspective, we prefer option 2. This is an issue in both PDCP and RLC entities, but it is not justified to apply different principles to PDCP and RLC. To avoid unnecessary complexity, the same solution should be applied.

Proposal 1:	(PDCP-1, RLC-5) Both PDCP and RLC entities consider control PDUs and data to be retransmitted as delay-reporting data volume associated with the smallest configured reporting threshold.

If option 2 is agreed, RAN2 needs to discuss the corresponding remaining time field setting in the case where only control PDUs and data to be retransmitted are associated with the smallest configured reporting threshold for an LCG. In this case, it may not be clear what value should be set for the corresponding remaining time field for the LCG because such data is not associated with the discard timer. In our opinion, the field can be set to a fixed value (e.g., zero).

Proposal 2:	If only control PDUs and data to be retransmitted are associated with the smallest configured reporting threshold for an LCG, the corresponding remaining time field for the LCG can be set to zero.

- Conclusion

In this contribution, we provided the following proposals.

Proposal 1:	(PDCP-1, RLC-5) Both PDCP and RLC entities consider control PDUs and data to be retransmitted as delay-reporting data volume associated with the smallest configured reporting threshold.

Proposal 2:	If only control PDUs and data to be retransmitted are associated with the smallest configured reporting threshold for an LCG, the corresponding remaining time field for the LCG can be set to zero.

- References

[1]	RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3