3GPP TSG-RAN WG2 Meeting #130        											                          R2-2504528

Malta, Malta, May 19th – 23th, 2025                                                          Revision of R2-2502771

Agenda item:	8.9.3

Source:	MediaTek Inc.

Title:	Discussion on CB-Msg3-EDT procedure

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1] and one of the objectives is to enhance EDT procedure:

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we continue discussing the CB-Msg3 on details of DSA mechanism, Msg4 monitor window, RNTI, details of Msg4 and fallback.

# Discussion

## MAC-2: CB-RNTI calculation

The CB-RNTI has been agreed to derived on the transmit resource for the transmission window.

RAN2#129bis Agreement - For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). - Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

In legacy random access procedure, the RA-RNTI is calculated as follows:

36.321 5.1.4 For BL UEs and UEs in enhanced coverage, RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1+t_id + 10*f_id + 60*(SFN_id mod (Wmax/10)) where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6), SFN_id is the index of the first radio frame of the specified PRACH, and Wmax is 400, maximum possible RAR window size in subframes for BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where is defined in clause 5.7.1 of TS 36.211 [7]. For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0.

The resource of the transmission window could be the start time in SFN and window length. The CB-RNTI can be the index of monitoring window during the H-SFN. The index of the monitoring window can be obtained by dividing the starting SFN by the window length and then rounding down. To make the calculation easier, the actual window length can be replaced by the minimum window length. For NB-IoT, the carrier id should also be taken into account.

One possible issue is that RA-RNTI is also generated by the SFN. To avoid same value of generated CB-RNTI and RA-RNTI, the CB-RNTI can start to count from the maximum value or RA-RNTI, which is 2400 for eMTC and 4096 for NB-IoT according to 36.321 7.1 RNTI value.

The formulation of the CB-RNTI of eMTC would be CB-RNTI = 2401+ floor(SFN_id / Wmin) where the SFN_id is the start SFN of the transmission window and the Wmin is the minimum value of transmission window length.

For formulation of the CB-RNTI of NB-IoT would be CB-RNTI = 4097+ floor(SFN_id / Wmin) + N*carrier_id where the SFN_id is the start SFN of the transmission window, the Wmin is the minimum value of transmission window length, the carrier_id is the index of UL carrier and the N can be the maximum value of floor(SFN_id / Wmin). The carrier_id of the anchor carrier is 0.

Proposal 1a: (MAC-2) For eMTC, CB-RNTI = 2401+ floor(SFN_id / Wmin) where the SFN_id is the start SFN of the transmission window and the Wmin is the minimum value of transmission window length.

Proposal 1b: (MAC-2) For NB-IoT, CB-RNTI = 4097+ floor(SFN_id / Wmin) + N*carrier_id where the SFN_id is the start SFN of the transmission window, the Wmin is the minimum value of transmission window length, the carrier_id is the index of UL carrier and the N can be the maximum value of floor(SFN_id / Wmin). The carrier_id of the anchor carrier is 0.

## MAC-7: Whether the HARQ operation is applicable to transmit CB-Msg3.

Since the HARQ entity is connected to the UL-SCH in the 36.321 Figure 4.2.1-1 MAC structure overview UE side, the HARQ operation is anyway used to transmit the CB-Msg3. The HARQ process and HARQ buffer are at least to use to transmit the CB-Msg3.

The HARQ process 0 is used to transmit the Msg3 in the legacy RA procedure. For CB-Msg3 transmission, even for DSA transmission, we can follow the legacy. For the HARQ feedback, the HARQ NACK is used to re-transmit the Msg3 in the legacy RA. But RAN2 has already introduced retransmission of CB-Msg3 in the same CE level, there is no need to introduce another way of retransmission.

Proposal 2a: (MAC-7) HARQ process 0 is used to transmit CB-Msg3.

Proposal 2b: (MAC-7) HARQ feedback is not used for CB-Msg3 transmission.

## MAC-9: Whether NW/UE processing time is needed when determine the Msg4 monitoring starts.

RAN2#129bis Agreement - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).  - FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring.

Normally, the processing time is specified for the time of decoding message and UL/DL switching. RAN2 has agreed that the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT, and FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT. Since the UE-eNB RTT is added, there is no need to introduce any processing time for UE UL/DL switching. But the time of network decoding message is still needed, which is normally 3ms.

Proposal 3: (MAC-9) A 3ms processing time is added when the Msg4 monitoring starts.

## MAC-10: FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT.

RAN2 has agreed the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT. Defining another way of starting the monitoring window not only takes time but also is not essential. To keep the solution simple, the current solution is enough.

Proposal 4: (MAC-10) RAN2 does not specify another way of starting Msg4 monitoring window.

## MAC-11: Whether a CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in CP solution.

The RRCEarlyDataComplete-NB only has one IE that is NAS message. In CP solution CB-Msg3-EDT, when there is no DL response message for the UE, the RRCEarlyDataComplete-NB is empty. Therefore, this RRC message is not needed.

For eMTC, the RRCEarlyDataComplete has many IEs, but they are all optional. It is also possible that RRCEarlyDataComplete is empty and thus not needed.

When the L3 signaling is not needed, the contention resolution identity in CB-Msg4 will be enough to finish the CB-Msg3-EDT procedure.

Proposal 5: (MAC-11) A CB-Msg4 with only contention resolution identity is allowed as the complete response to the CB-Msg3 in CP solution.

## MAC-12:  FFS how the multiplexing is organized for CB-MSG4.

To multiplexing multiple UEs in CB-Msg4, an identity to differentiate UEs is needed. In this case, the contention resolution ID is sufficient to identify a UE. The other information related to this UE can be assembled in the MAC PDU after the corresponding contention resolution ID.

Proposal 6: (MAC-12) Multiple contention resolution IDs could be included in CB-MSG4, the information related to the UE can be assembled in the MAC PDU after the corresponding contention resolution ID.

## MAC-14:  FFS for the detail of HARQ operation on CB-Msg4.

We had agreement that HARQ feedback is adopted to acknowledge Msg4. Since one CB-Msg4 can target multiple UEs simultaneously, there could be multiple HARQ feedback for one CB-Msg4. How the PUSCH resources are provided to support multiple HARQ feedback is unknown. Whether the ACK or NACK should be used also needs further discussion.

RAN2#129bis agreement: - HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

The HARQ resource in DCI is 4 bits for NB-IoT and 2 bits for eMTC. This information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE.

Proposal 7: (MAC-14) The HARQ resource information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE.

Since the CB-Msg3 the contention-based, it is normal that the corresponding response is not received, there is no point to send NACK to the network. If the UE does not receive the corresponding response in the CB-Msg4, UE will continuous monitoring the PDCCH in the monitoring window. After the monitoring window is passed, UE can re-attempt CB-Msg3 transmission as RAN2 has agreed.

Proposal 8: (MAC-14) UE will send HARQ ACK for CB-Msg4 if the contention resolution ID matches. No HARQ NACK is sent.

Additionally, there is a R18 feature namely the HARQ feedback disabling. The HARQ feedback can be skipped according the network configuration. The idea can also be applied here. If the network does not want to the HARQ feedback, due to for example lack of radio resource, then the HARQ resource will not be provided and UE will not send the HARQ feedback.

Proposal 9: (MAC-14) The HARQ feedback for CB-Msg4 can be disabled if the HARQ feedback resource is not provided.

For NB-IoT, there is another issue. The SCS of the HARQ feedback (i.e., 3.75kHz or 15kHz) is still unknow. In the legacy, this is same as the PUSCH transmission. We can follow the legacy.

Proposal 10: (MAC-14) For NB-IoT, the SubCarrierSpacing of the HARQ feedback for CB-Msg4 is same as the CB-Msg3.

## MAC-13:  FFS new MAC PDU format for CB-Msg4

RAN2 has agreed to introduce Contention Resolution Identity, C-RNTI, Backoff parameter, TA information in the CB-Msg4. The number of reserved LCIDs for future expansion is only 2. Therefore, it is proposed to introduce a new MAC PDU format for CB-Msg4 so that no new LCID is required.

The proposal shows in the figure 1 is to introduce 3 types of new sub header and a new type of MAC payload.

Proposal 11: (MAC-13) The MAC PDU for CB-Msg4 is consist of sub-header(s) follow by MAC payload and optional padding if needed. RAN2 takes the following figure as the baseline for the MAC PDU format of CB-Msg4.

Figure 1 MAC PDU format of CB-Msg4

Similar to legacy RA, for the backoff indication, we can define a new type of MAC sub-header. The UEs which cannot match any contention resolution ID from the CB-Msg4 will backoff a certain time before initiating another attempt of CB-Msg3 transmission. As in the legacy, the BI field is 4 bits. The same table for backoff parameters in 36.321 7.2 can be used.

Proposal 12a: (MAC-13) Introduce a new CB BI MAC sub-header in CB-MSg4 for backoff parameter. It has 1bit E for subhead/payload indication, 2 bits T for subhead type, 4 bits BI for backoff indication and 1 bit R for reservation.

Figure 2 CB BI MAC sub-header

To identify the CB-Msg3 Response in the MAC PDU payload, a new type of MAC sub-header is needed. The HARQ ACK resource, the TAC and the C-RNTI should be provided together with the contention resolution ID. The HARQ ACK resource can be optional. If the network does not provide the HARQ ACK resource, the HARQ ACK can be skipped. The TAC and C-RNTI are only needed when the network wants UE to monitor PDCCH after contention resolution for early contention resolution or when the UE will go to connected mode. Thus, they are also optional. In the corresponding MAC sub-header, there should be bits to indicate the presence of HARQ ACK resource, TAC and C-RNTI.

Proposal 12b: (MAC-13) Introduce a new CB-Msg3 Response (CBR) MAC sub-header in CB-MSg4. It has 1bit E for subhead/payload indication, 2 bits T for subhead type, 1bit T2 for HARQ ACK resource present, 1 bit T3 for TAC present, 1 bit T4 for C-RNTI present and 2bit R for reservation.

Figure 3 CBR MAC sub-header

Similar to the legacy, to identify the MAC SDU for CCCH RRC signaling or DTCH user data in the MAC PDU payload, a new type of MAC sub-header is needed. The 5-bit LCID field is used to indicate the logical channel ID of the MAC SDU. The L field is used to indicate the length of the MAC SDU. The L field can be 7 bits or 15 bits as in the legacy. A F field is needed to indicate which type of the L field is used. A sub-header without L field is also needed for some special cases.

Proposal 12c: (MAC-13) Introduce a new CB Data MAC sub-header in CB-MSg4. It has 1 bit E for subhead/payload indication, 2 bits T for subhead type, 5 bits LCID, 7 bits or 15 bits L for MAC SDU length, 1 bit F for 15 bits L indication.

Figure 4 CB Data MAC sub-header

The 48-bit contention resolution ID is used to identify the UE. It of course should be included in the CB-Msg3 response. The HARQ ACK resource, the TAC and C-RNTI should be carried together with contention resolution ID, so that the UE can know this information is meant for it.

The HARQ ACK resource in DCI is 2 bits for eMTC and is 4 bits for NB-IoT. To keep the discussion short and simple, we better not change the length and definition of HARQ ACK resource bits in DCI, but just reuse the bits in the MAC PDU.

Proposal 13a: (MAC-13) The 2-bit HARQ ACK resource for eMTC and 4-bit HARQ ACK resource for NB-IoT is used in the CB-Msg3 response.

The current TAC can be 11 bits and 6 bits. The 11-bit TAC is an absolute value for PRACH TA calibration in RAR and the 6-bit TAC is used to TA adjustment for PUSCH in connected mode. In CB-Msg3-EDT, the CB-Msg3 is transmitted via PUSCH. The cyclic prefix of PUSCH is relatively smaller than the PRACH, the 6-bit TAC is enough to adjust the TA. Additionally, according to the TS 36.213, the 6-bit TAC is used where NTA,new = NTA,old + (TA 31)16. The (TA – 31) can a negative value, where the TA is the 6-bit TAC. It is quite useful when the UL timing needs to be delayed. In other hands, the 11-bit TAC can only advance the UL timing. Therefore, the 6-bits TAC is more suitable to be adopted in the MAC PDU.

Proposal 13b: (MAC-13) The 6-bit TAC is used in the in the CB-Msg3 response.

To summarize the CB-Msg3 response, there will be 48 bits contention resolution ID, 2 bits HARQ ACK resource offset for eMTC, 4 bits HARQ-ACK resource for NB-IoT, 6 bits TAC, 16 bits C-RNTI.

Proposal 13c: (MAC-13) New CB-Msg3 Response (CBR). It has 48 bits contention resolution ID, 2 bits HARQ ACK resource offset for eMTC, 4 bits HARQ-ACK resource for NB-IoT, 6 bits TAC, 16 bits C-RNTI.

Figure 5 CBR for NB-IoT

Figure 6 CBR for eMTC

## MAC-15:  What should be the UE behavior (e.g. the can initiate the legacy 4-step RA) when the CB-Msg3 procedure fails.

In the last RAN2 meeting, all companies agreed that UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails. The unresolved issue is which upper layer will be indicated to. The candidates are RRC layer or NAS layer. Normally, the MAC layer does not direct connect to the NAS layer, we shouldn’t break this tradition and introduce complexity. Therefore, the MAC layer should indicate the CB-Msg3 failure to the RRC layer. The RRC layer can decide whether to initiate the legacy 4-step RA or indicate to the NAS layer the failure of connection establishment.

Proposal 14a: (MAC-15) The MAC layer should indicate the CB-Msg3 failure to the RRC layer.

Proposal 14b: (MAC-15) The RRC layer can decide whether to initiate the legacy 4-step RA or indicate to the NAS layer the failure of connection establishment.

## MAC-17: Whether to allow multiple TBSs as in EDT.

The multiple TBSs in legacy EDT is to provide a list of TBSs rather than a single TBS. UE can choose a smaller TBS so that it is no need to pad the maximum TBS. The PUSCH transmission can be shorter. It is also captured in the RAN1 spec, so that RAN1 discussion is required.

In current stage, we think it is better not to introduce complicated optimization but use the reset of time wisely to complete the necessities.

Proposal 15: (MAC-17) RAN2 postpones the discussion on multiple TBSs in CB-Msg3-EDT.

## MAC-18: How to model the CB-Msg3 response window (i.e. MSG4 monitoring window) ? Should it be a timer as in legacy RA response window, and what should be the value range.

During the MAC running CR offline discussion, companies mentioned that the CB-Msg3 response window should be modelled as a timer. We agree that a timer would be more flexible. It can be started, stopped, expired and restarted. This is very similar to how MAC SPEC handle the timer mac-ContentionResolutionTimer.

In the meantime, due to the UE multiplexing in the CB-Msg4, the length of the CB-Msg3 response window should be much longer than the length of contention resolution window.

Proposal 16: (MAC-18) A CB-Msg3 contention resolution timer is used to model the CB-Msg3 response window (similar to legacy mac-ContentionResolutionTimer). The network should be able to configure the length of the CB-Msg3 response window longer than the length of legacy contention resolution window.

# Conclusion

Proposal 1a: (MAC-2) For eMTC, CB-RNTI = 2401+ floor(SFN_id / Wmin) where the SFN_id is the start SFN of the transmission window and the Wmin is the minimum value of transmission window length.

Proposal 1b: (MAC-2) For NB-IoT, CB-RNTI = 4097+ floor(SFN_id / Wmin) + N*carrier_id where the SFN_id is the start SFN of the transmission window, the Wmin is the minimum value of transmission window length, the carrier_id is the index of UL carrier and the N can be the maximum value of floor(SFN_id / Wmin). The carrier_id of the anchor carrier is 0.

Proposal 2a: (MAC-7) HARQ process 0 is used to transmit CB-Msg3.

Proposal 2b: (MAC-7) HARQ feedback is not used for CB-Msg3 transmission.

Proposal 3: (MAC-9) A 3ms processing time is added when the Msg4 monitoring starts.

Proposal 4: (MAC-10) RAN2 does not specify another way of starting Msg4 monitoring window.

Proposal 5: (MAC-11) A CB-Msg4 with only contention resolution identity is allowed as the complete response to the CB-Msg3 in CP solution.

Proposal 6: (MAC-12) Multiple contention resolution IDs could be included in CB-MSG4, the information related to the UE can be assembled in the MAC PDU after the corresponding contention resolution ID.

Proposal 7: (MAC-14) The HARQ resource information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE.

Proposal 8: (MAC-14) UE will send HARQ ACK for CB-Msg4 if the contention resolution ID matches. No HARQ NACK is sent.

Proposal 9: (MAC-14) The HARQ feedback for CB-Msg4 can be disabled if the HARQ feedback resource is not provided.

Proposal 10: (MAC-14) For NB-IoT, the SubCarrierSpacing of the HARQ feedback for CB-Msg4 is same as the CB-Msg3.

Proposal 11: (MAC-13) The MAC PDU for CB-Msg4 is consist of sub-header(s) follow by MAC payload and optional padding if needed. RAN2 takes the following figure as the baseline for the MAC PDU format of CB-Msg4.

Proposal 12a: (MAC-13) Introduce a new CB BI MAC sub-header in CB-MSg4 for backoff parameter. It has 1bit E for subhead/payload indication, 2 bits T for subhead type, 4 bits BI for backoff indication and 1 bit R for reservation.

Proposal 12b: (MAC-13) Introduce a new CB-Msg3 Response (CBR) MAC sub-header in CB-MSg4. It has 1bit E for subhead/payload indication, 2 bits T for subhead type, 1bit T2 for HARQ ACK resource present, 1 bit T3 for TAC present, 1 bit T4 for C-RNTI present and 2bit R for reservation.

Proposal 12c: (MAC-13) Introduce a new CB Data MAC sub-header in CB-MSg4. It has 1 bit E for subhead/payload indication, 2 bits T for subhead type, 5 bits LCID, 7 bits or 15 bits L for MAC SDU length, 1 bit F for 15 bits L indication.

Proposal 13a: (MAC-13) The 2-bit HARQ ACK resource for eMTC and 4-bit HARQ ACK resource for NB-IoT is used in the CB-Msg3 response.

Proposal 13b: (MAC-13) The 6-bit TAC is used in the in the CB-Msg3 response.

Proposal 13c: (MAC-13) New CB-Msg3 Response (CBR). It has 48 bits contention resolution ID, 2 bits HARQ ACK resource offset for eMTC, 4 bits HARQ-ACK resource for NB-IoT, 6 bits TAC, 16 bits C-RNTI.

CBR for NB-IoT

CBR for eMTC

Proposal 14a: (MAC-15) The MAC layer should indicate the CB-Msg3 failure to the RRC layer.

Proposal 14b: (MAC-15) The RRC layer can decide whether to initiate the legacy 4-step RA or indicate to the NAS layer the failure of connection establishment.

Proposal 15: (MAC-17) RAN2 postpones the discussion on multiple TBSs in CB-Msg3-EDT.

Proposal 16: (MAC-18) A CB-Msg3 contention resolution timer is used to model the CB-Msg3 response window (similar to legacy mac-ContentionResolutionTimer). The network should be able to configure the length of the CB-Msg3 response window longer than the length of legacy contention resolution window.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)