3GPP TSG-RAN WG2 Meeting #130	R2-2504529

St. Julian’s, Malta, 19th - 23rd May, 2025

Agenda Item		:	8.8.4

Source		:	LG Electronics Inc.

Title		:	Discussion on support for broadcast service in NTN

Document for		:	Discussion and Decision

1	Introduction

In last meetings, RAN2 agreed following statement:

RAN2#128 Agreements: - A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). - RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. RAN2#129 Agreements: - In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list. - It shall be possible to signal multiple service area IDs to one MBS service (we insert a list of service area IDs in MCCH). RAN2#129bis Agreements: - UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. FFS whether and how the association between intended service area and frequencies is acquired.

In this contribution, we provide our views on support for broadcast service in NTN.

2	Discussion

2.1	MBS service continuity in NTN

RAN2 agreed that a UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de-prioritize the frequency. It remains to be discussed whether and how the association between intended service area and frequencies is acquired. Due to this issue is scenario dependant, a company suggests that the following scenarios are considered as a baseline.

- Multiple frequencies, served by the same satellite, illuminating the same geographical area. Each frequency provides a different MBS service. These services are not being broadcast in the serving cell.

- Multiple frequencies, served by the same or different satellites, but not covering the same geographical area (they are neighbour cells). Each frequency provides a different MBS service. These services are not being broadcast in the serving cell.

In both scenarios, IDLE/INACTIVE UE receives a MBS service from the serving cell, and there are no neighbour cells/frequencies providing the same MBS service. If the UE figure out that there are no neighbour cells/frequencies providing the same MBS service through mbs-NeighbourCellList, the UE needs to request unicast reception of the MBS service before moving to a neighbour cell/frequency not providing the MBS service using point-to-multipoint (PTM) transmission. This is a legacy mechanism, then there is no need to introduce additional enhancement for MBS service continuity in above scenarios.

RAN2 agreed that a signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s), and the geographic area information for the intended service areas can be semi-static and not cause frequent updates. Some companies believe that the introduction of ISA causes necessary changes to legacy mechanisms for MBS service continuity in NTN. In the overlapping ISAs between serving cell and neighbour cell(s), if the legacy mechanism is reused for the list of neighbour cells providing the same MBS service, it could allow the UE to prioritize the frequency not providing current MBS service. However, if the list of neighbour cells providing the same MBS service is signalled taking into account the associated ISA, the UE could prioritize appropriate frequency. The signalling of the list of neighbour cells, considering the associated ISA, is the NW implementation. That is, it does not needed to any changes of legacy mechanisms for MBS service continuity in NTN.

Observation 1	When a UE receives a MBS service associated with ISA(s) from a serving cell and receives a list of neighbouring cells that provide the same MBS service, it interprets that the neighbouring cells also provide this MBS service for the corresponding ISA(s) and the other neighbouring cells do not provide this MBS service for the corresponding ISA(s). In this example case, there is no strong technical motivation to introduce additional signaling of ISA information that the neighbouring cell provides the same MBS service as the serving cell.

Proposal 1	There is no need to introduce additional enhancements related to MBS service continuity.

2.2	MII procedure in NTN

RAN2 did not conclude whether the enhancement is needed or not to support MII procedure initiation upon entering/leaving the intended service area. One company thinks that the UE in RRC_CONNECTED does not need to initiate the MII procedure upon leaving the intended service area. However, it is worth that the UE indicates absence of interest in any of MBS broadcast services provided in serving cell. The network may release the MBS broadcast service reception capability of the UE by using this information. As a result, an MBS-capable UE in RRC_CONNECTED may need to initiate the MII procedure initiation upon entering or leaving the intended service area.

On the other hand, most of the companies commented in RAN2#127bis thinks that the MII procedure may be initiated by RRC_CONNECTED UE in any time. However, in our understanding, it is a misunderstanding of the procedural text. According to the TS 38.331 section 5.9.4.2, several cases are specified when UE may initiate the MII procedure. Although UE behaviour is specified as “may”, it does not mean that the MBS-capable UE in RRC_CONNECTED could initiate the MII procedure whenever it wants. Therefore, modification of specification is needed to support the MII procedure initiation upon entering or leaving the intended service area.

Observation 2	When UE in RRC_CONNECTED is not interested in any of MBS broadcast services provided in serving cell, it is redundant to maintain the MBS broadcast service reception capability.

Proposal 2	An MBS-capable UE in RRC_CONNECTED may initiate the MII procedure upon entering or leaving the intended service area.

It can be easily implemented in the current spec by adding a simple clarification as a note. There is a condition initiating the MII procedure upon entering or leaving the broadcast service area. In our understanding, the intended service area is the broadcast service area.

Proposal 3	Add a note to section 5.9.4.2 of TS 38.331 as the intended service area can be considered as the broadcast service area.

3. Conclusion

In this contribution, following statements are proposed:

Observation 1	When a UE receives a MBS service associated with ISA(s) from a serving cell and receives a list of neighbouring cells that provide the same MBS service, it interprets that the neighbouring cells also provide this MBS service for the corresponding ISA(s) and the other neighbouring cells do not provide this MBS service for the corresponding ISA(s). In this example case, there is no strong technical motivation to introduce additional signaling of ISA information that the neighbouring cell provides the same MBS service as the serving cell.

Observation 2	When UE in RRC_CONNECTED is not interested in any of MBS broadcast services provided in serving cell, it is redundant to maintain the MBS broadcast service reception capability.

Proposal 1	There is no need to introduce additional enhancements related to MBS service continuity.

Proposal 2	An MBS-capable UE in RRC_CONNECTED may initiate the MII procedure upon entering or leaving the intended service area.

Proposal 3	Add a note to section 5.9.4.2 of TS 38.331 as the intended service area can be considered as the broadcast service area.

4. Reference

[1] RAN2#129bis Chairman’s note, Wuhan, China, April 7 – 11, 2025.