3GPP TSG RAN WG2 Meeting #130	 R2-2504532

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.2

Source: 	ITL

Title: 	Discussion on A-IoT paging

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to A-IoT paging are as follows:

Agreements

- FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

- RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Agreements on CFRA

- Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to take this into account for their design.

This contribution presents our position regarding the ongoing discussion in RAN2 on device behavior when a new service request is received while an existing procedure is still ongoing.

Discussion

RAN2 has reached consensus that parallel service execution by Ambient IoT (AIoT) devices is not supported in Rel-19. However, the device behavior when receiving a new request during an ongoing procedure remains under discussion, with two main options proposed:

- Option A: Ignore all new service requests and R2D messages not associated with the ongoing procedure

- Option B: Abort the current procedure and respond to the latest request

RAN2#129 Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.  FFS device behaviour if multiple requests are received in parallel (if needed).

Some companies have proposed leaving this decision to implementation (i.e., not specifying behavior in the standard).

We support Option A and believes that the expected device behavior should be clearly specified in the Rel-19 specification for the following reasons:

- Avoiding interoperability risks: Leaving behavior undefined creates significant risks of inconsistent implementations across vendors. In a multi-vendor environment, undefined behavior could result in unpredictable system-level behavior and degradation of performance.

- Simplicity and feasibility: Option A allows for a simpler device architecture, aligning with the low complexity and energy constraints of typical AIoT devices. Unlike Option B, Option A does not require inter-reader coordination, which is currently outside the scope of Rel-19.

- Consistency with RAN2 agreement: The interpretation that 'device supports only one procedure at a time' implies that the device should ignore any subsequent requests until the current one concludes. This aligns naturally with Option A and avoids ambiguity.

- Implementation Burden of Option B: Option B increases complexity by requiring the device to detect and decide whether to abort current procedures and handle potential synchronization issues. This goes against the principle of lightweight design for AIoT devices.

Proposal 1: Introduce Option A as the normative behavior in Rel-19 A-IoT.

Deadlock problem in Option A

Option A, which restricts AIoT devices to a single procedure at a time, aligns with RAN2's Rel-19 agreement to not support parallel services. However, this approach poses the risk of device deadlock in scenarios such as:

- Reader restarts or power failure during a procedure, causing the device to indefinitely wait for a procedure to complete.

- Device fails to receive a NACK or release message from the reader.

- Reader initiates a new service intentionally, but the device continues to wait for the previous procedure to conclude.

In such cases, the device's strict adherence to ignoring new messages could lead to service disruption, energy waste, and decreased reliability

We support introducing a timer-based mechanism in AIoT devices to autonomously determine when a procedure can be considered expired. This timer would start upon initiation of a service request and expire after a predefined period of inactivity, upon which the device would consider the procedure terminated and ready to accept new requests.

Main key advantages can be achieved as follows:

- Maintains Option A’s single-procedure simplicity while preventing indefinite blocking.

- Requires minimal additional resources (One timer per device).

- Complies with energy constraints of low-complexity devices if the timer is implemented with lightweight logic.

To ensure robustness of Option A without introducing heavy signaling overhead or ambiguity, We recommend RAN2 adopt a timer-based release mechanism. This addition provides a deterministic escape from potential deadlock conditions and upholds system reliability in Rel-19.

Proposal 2: Introduce a timer-based release mechanism to enable the device to autonomously determine the end of an ongoing procedure

R2D message associated with the ongoing procedure

According to RAN2 agreements, Rel-19 does not support simultaneous procedures or parallel services in A-IoT devices, nor does it mandate support for multi-reader environments. However, devices should still determine whether incoming R2D messages are part of the currently ongoing procedure. This is critical especially under Option A, where the device is expected to ignore all unrelated messages while a procedure is in progress.

Without a proper identifier, the device would lack a reliable method to determine message relevance, potentially leading to incorrect message processing or ignoring necessary procedure-continuing messages.

We think that a Transaction ID be included in all R2D messages that are associated with a specific procedure. The Transaction ID will allow the device to determine whether a received message is relevant to the ongoing procedure, even in the absence of multi-reader support.

- No change to the Rel-19 position on multi-reader support (i.e., it remains unsupported).

- The Transaction ID is only used to relate messages within a single procedure.

- Devices can safely ignore any message with a non-matching Transaction ID during an ongoing procedure.

To preserve the simplicity of Rel-19 while ensuring procedural integrity and reliability we prefer to introduce the adoption of Transaction ID in procedure-related R2D messages. This inclusion allows the device to distinguish relevant messages without requiring support for multi-reader operations, consistent with the Rel-19 scope. While future releases such as Rel-20 may consider supporting multi-reader scenarios and thus require both Transaction ID and Reader ID for reliable message association, we think that Transaction ID alone is sufficient and appropriate for the Rel-19 context.

Proposal 3: Include Transaction ID in R2D messages to enable message relevance detection under Option A

Conclusion

In this contribution, we discussed the A-IoT paging. According to discussion in section 2, we have the following proposals:

Proposal 1: Introduce Option A as the normative behavior in Rel-19 A-IoT.

Proposal 2: Introduce a timer-based release mechanism to enable the device to autonomously determine the end of an ongoing procedure

Proposal 3: Include Transaction ID in R2D messages to enable message relevance detection under Option A

Reference

RAN2#129bis meeting report