3GPP TSG-RAN WG2 Meeting #130	R2-2504537

St.Julians, Malta,  May 19th – 23rd , 2025

Source:				ETRI

Title:					Discussion on LCP enhancements for XR

Agenda Item:		8.7.4.1	LCP enhancements

Document for:		Discussion and Decision

1. Introduction

During the study of LCP enhancements by the RAN2#129bis meeting, agreements on LCP enhancements have been made as the followings:

Agreements on LCP enhancements

1.	Working assumption (to be revisited next meeting): No Bj enhancement is introduced.

2.	It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications

3.	We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

4.	It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP.

5.	For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

6.	Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR.

7.	All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

We discussed Bj enhancements for handling delay-critical data, but no consensus was reached. Therefore, a working assumption will serve as a baseline to continue the discussion on LCP enhancements. In this contribution, we provide discussions on the LCP enhancements for XR service.

2. Discussion

Negative Bj and fairness

Discussions on LCP enhancement have been ongoing. In the last meeting, there was a debate on whether to allow negative Bj when allocating resources to delay-critical data. The core issue was a disagreement on how to prioritize the transmission of delay-critical data while ensuring fairness for data transmitted between LCHs when selecting multiple data units for resource allocation from an Uplink (UL) grant.

Delay-critical data is given priority, and in the resource allocation process, resources are allocated to delay-critical data first. During resource allocation, the UE selects priority based on LCH priority and schedules the data in the LCH. In this process, to ensure fairness in resource utilization among LCHs, the allocation of resources is limited by the bucket level, Bj. Discussions continued regarding the Bj > 0 condition for selecting the initially transmitted data. Consequently, there was opposition to relaxing the Bj  > 0 condition to allow allocating resources to delay-critical data when Bj < 0. The legacy Bj operation provides fairness among data of LCHs transmitted in scheduling. This determines the amount of data transmitted in the 1st round between LCHs at a specific time. This limitation on transmitted data can provide fairness between LCHs, but it may prevent the transmission of delay-critical data residing in the buffer. Delay-critical data is time-sensitive; if not transmitted within its required deadline, it is discarded. The scheduling algorithm can be described as a procedure that changes the order of data transmitted within the allocated resources. It occurs where delay-critical data is not transmitted due to the limitation imposed by the legacy Bj condition, leading to the loss of this time-sensitive data. This ultimately degrades radio interface performance and fails to guarantee the Quality of Service (QoS) for flows with stringent latency requirements.

Observation 1) In the 1st round of resource allocation in the LCP procedure, the legacy Bj procedure restricts the priority to delay-critical data.

Observing the scheduling operation over a long period, scheduling determines the order of data transmission. In this ordering methodology, delay-critical data is transmitted first, which can cause delays for other, non-delay-critical data. This has the positive effect of reducing transmission time for delay-critical data but increases latency for other data. From a long-term perspective, transmitting delay-critical data first reduces its discard rate, potentially increasing the overall volume of successfully transmitted data. However, a review of fairness is needed from the perspective of LCHs to which the delayed, non-delay-critical data belongs. Fairness can be interpreted based on the amount of delay-critical data that has been assigned additional priority. If the amount of delay-critical data is small compared to the total transmitted data, fairness can be ensured after the delay-critical data has been transmitted. If transmission of delay-critical data is allowed when Bj < 0, resources are allocated to it. The LCH that transmitted this delay-critical data may then maintain a Bj < 0 state, potentially preventing its non-delay-critical data from being transmitted. Subsequently, as its priority might fall back to a lower level, resources are allocated with this lower priority, and the Bj < 0 condition restrictively determines the transmission of non-delay-critical data. Over time, when the condition changes to Bj > 0, non-delay-critical data can then be transmitted. This can be seen as a process where fairness is eventually provided for the resources that were initially prioritized for delay-critical transmission.

Observation 2) When the proportion of delay-critical data is low, fairness is generally provided when observed over a long period.

In the UE resource allocation process, if delay-critical data is granted adjusted priority, allowing negative Bj enables this delay-critical data to be transmitted first. Data can be declared delay-critical if its remaining time until its discard deadline is less than a certain threshold, and then additional priority is granted. When configuring an LCH, it is determined whether to establish the LCH by considering radio resources and other LCHs sharing those resources. In this process, configuration is performed to maintain a low proportion of delay-critical data within an LCH. Under the additional priority condition, allowing negative Bj ensures that delay-critical data is prioritized for transmission. After delay-critical data is transmitted, non-delay-critical data typically reverts to its legacy priority, and negative Bj might not be allowed for it under normal LCP rules.

Observation 3) Using negative Bj when allocating resources to LCH priority adjusted data  results in prioritizing the transmission of this delay-critical data, for which LCH priority has been temporarily boosted.

Proposal 1) A negative Bj value is allowed in the 1st round resource allocation for LCH priority adjusted data within the LCP procedure.

Proposal 2) The UE may be configured to allow a specific LCH to utilize a negative Bj value for its delay-critical data.

If the proportion of delay-critical data is high relative to the total amount of data to be transmitted, allowing negative Bj can make it difficult to ensure fairness. In scheduling scenarios where most data is delay-critical, the process of allocating resources among LCHs with a priority becomes an implementation issue. If multiple LCHs with priority-adjusted data have the same priority level, the UE can choose among them. It is reasonable to prioritize data with the shortest remaining time. If new delay-critical data arrives before the negative Bj state is recovered (i.e., before Bj becomes positive again), non-delay-critical data may not be transmitted for extended periods. To address such fairness issues, methods like limiting the magnitude of negative Bj or imposing restrictions on how "additional Bj" (effectively, the depth of the negative Bj allowed) is utilized may need consideration.

Observation 4) Fairness issues arise when the proportion of delay-critical data is high; therefore, it is necessary to aim for configurations that maintain a low proportion of delay-critical data within LCHs.

Intra-UE Prioritization

LCH priority is utilized in the process of allocating resources from a UL grant to the data of different LCHs. PUSCH resources, which carry a UL burst, may overlap resource assignments from different grants. At the time of transmission, the UE selects which data to transmit based on applicable priorities for the specific granted resources. The process of allocating data from a UL grant involves allocating resources by considering Bj in conjunction with both the legacy priority and any additional (adjusted) priority assigned to an LCH. A Scheduling Request (SR) transmitted on PUCCH resources, which requests a UL grant, can collide with other transmissions. In case of a collision, the UE selects what to transmit based on defined priority rules.

An SR triggered from a delay-critical data could potentially be associated with either the legacy priority of that LCH or an additional priority if the LCH's priority has been adjusted due to the delay-critical data. An SR originating from an LCH without any delay-critical data uses its legacy priority. The question arises: what priority should an SR originating from an LCH with delay-critical data use? The SR serves to request UL resource allocation from the gNB/base station, and the SR's priority determination often uses the priority of the LCH(s) that triggered it. If an LCH is operating with an additional (higher) priority due to its delay-critical data, it seems appropriate to apply this additional priority to the SR itself, as it represents the urgency of the data awaiting transmission.

Proposal 3) LCH adjusted priority is applied for SR priority determination.

3. Conclusion

In the discussion we made the following observations:

Observation 1) In the 1st round of resource allocation in the LCP procedure, the legacy Bj procedure restricts the priority to delay-critical data.

Observation 2) When the proportion of delay-critical data is low, fairness is generally provided when observed over a long period.

Observation 3) Using negative Bj when allocating resources to LCH priority adjusted data  results in prioritizing the transmission of this delay-critical data, for which LCH priority has been temporarily boosted.

Observation 4) Fairness issues arise when the proportion of delay-critical data is high; therefore, it is necessary to aim for configurations that maintain a low proportion of delay-critical data within LCHs.

We propose the followings:

Proposal 1) A negative Bj value is allowed in the 1st round resource allocation for LCH priority adjusted data within the LCP procedure.

Proposal 2) The UE may be configured to allow a specific LCH to utilize a negative Bj value for its delay-critical data.

Proposal 3) LCH adjusted priority is applied for SR priority determination.

4. References

R2-2408907, “Discussion on additional LCP handling,” ETRI.

R2-2410844, “Further Discussion on additional LCP handling,” ETRI