3GPP TSG-RAN WG2 Meeting #130	 R2-2504540

St Julian’s, Malta, May 19th -23rd , 2025

Source:	KDDI Corporation

Title:	Discussion on On-demand SSB

Tdoc Type:	Discussion

Agenda Item:	8.5.2

Document for:  Discussion

Release:       Rel-19

1	Introduction

In RAN2#129b, the following discussion points were left for this meeting. In this contribution, we will provide our considerations on the related points.

Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. Send LS to inform RAN2 working assumption to RAN4, and to check if ok to them. Comeback in CB session (Apple, LS in R2-2503015) FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell. [Ericsson]: Clarifies RAN1 decided both A-SSB and OD-SSB are located in the same BWP, that’s why we changed our position. [Nokia]: The requirement that one MO per carrier has been there from Rel-15. Afraid to change this principle based on Ericsson proposal. For Apple proposal, if multiple OD-SSBs are configured for a given SCell, how to handle it? [Apple]: Do not consider multiple OD-SSB for a given SCell. [OPPO]: Agree with Apple proposal. For multiple OD-SSB configurations, we can handle it by MO index in the corresponding configuration. [ZTE]: Multiple OD-SSB per SCell is possible, but not sure whether it is possible with the different frequencies. [Huawei]: Agree with Apple proposal. However, it will be good to remove “RedCap” since the detailed signalling can be different. [Ericsson]: Should RAN4 decide it? [Nokia]: Understand it is RAN2 job. Ok with the proposal, but wants to add “FFS whether/where multiple OD-SSBs are on the different frequencies for a given SCell.” [Apple, LG]: For Nokia’s FFS proposal, it is RAN1 scope. We do not need to create this explicit FFS in RAN2. If RAN1 introduces it, we can discuss it otherwise not needed. [Ericsson]: If we go Apple proposal, it may bring some issues when OD-SSB is activated, e.g. how to combine measurements, how to apply filtering from different MO, etc. Good to check with RAN4.

2	Discussion

Multiple OD-SSBs

As quoted above, in the last RAN2 meeting, the topic of configuring multiple OD-SSBs with different frequencies for a given SCell was discussed but no consensus was reached. Since AO-SSB and OD-SSB are already allowed to be configured on different frequencies, it is technically possible to configure multiple OD-SSBs with different frequencies.

Regarding the measurement-related concerns raised by some companies, we share Oppo’s view that these issues can be addressed using the MO index. Therefore, from a technical perspective, there appears to be no fundamental issue with allowing multiple OD-SSBs on different frequencies.

However, we have concerns about the practical benefits of configuring a large number of OD-SSBs and the scenarios in which the network would need to configure so many OD-SSBs with different frequencies. We would like to hear other companies’ views on this matter and are open to further discussion.

Proposal 1: RAN2 to discuss whether multiple OD-SSBs with different frequencies can be configured for a given SCell.

L3 RRM measurements and report for OD-SSB

In the last meeting, it was agreed that when the AO-SSB and OD-SSB have different center frequencies, a new servingCellMO should be introduced in ServingCellConfig to indicate the MO of the OD-SSB. However, for the case where the AO-SSB and OD-SSB share the same frequency, it has not yet been discussed whether to use a different servingCellMO or to enhance the SMTC in the same sevingCellMO.

Regarding this issue, we share the same view as mentioned in [1]—that legacy SMTC L3 measurement is configured per frequency rather than per cell. Therefore, when the AO-SSB and OD-SSB have the same frequency, it would be difficult for the network to configure a single SMTC window that effectively covers both AO-SSB and OD-SSB. Applying the legacy SMTC approach to OD-SSB may result in unnecessary power consumption, as the UE could continue measuring even when no OD-SSB is transmitted or when the number of SSBs is significantly reduced.

To address this inefficiency, we propose that when the AO-SSB and OD-SSB share the same frequency, they may use the same servingCellMO. However, the SMTC for OD-SSB should be associated with the adaptive OD-SSB periodicity.

Proposal 2: When the AO-SSB and OD-SSB have the same frequency, they may share the same servingCellMO, but the OD-SSB SMTC should be linked to the adaptive OD-SSB periodicity.

In the legacy measurement configuration, the measurement and reporting configurations are linked via a measId, and only one type of SMTC can be configured per frequency. This approach is inefficient for OD-SSB, as the network would need to explicitly configure multiple OD-SSB periodicities in each OD-SSB configuration, according to the RAN1 agreement. Therefore, it adds unnecessary complexity and signaling overhead to the measurement configuration.

We propose that the OD-SSB SMTC be dynamically activated only when the OD-SSB is active, and that the UE applies the OD-SSB-specific SMTC only during periods when the OD-SSB is activated.

Proposal 3: The UE applies the OD-SSB specific SMTC when the OD-SSB is activated.

There are two L3 measurement reporting mechanisms defined: periodic reporting and event-triggered reporting. Periodic reporting allows the UE to send measurement results at regular intervals, while event-triggered reporting enables reporting only when specific conditions are met. However, for OD-SSB scenarios, we believe these mechanisms may be insufficient, as the UE must either wait for the next periodic interval or for predefined event conditions to be satisfied. This delay contradicts the purpose of OD-SSB, which is to enable both the UE and the network to quickly assess the current cell quality immediately after the OD-SSB is transmitted. Therefore, we consider a MAC CE-based measurement reporting mechanism to be beneficial. The same MAC CE used to trigger the OD-SSB could also be extended to support this reporting function, enabling low-latency feedba

Proposal 4: Introducing a new MAC CE-triggered measurement reporting mechanism is beneficial for OD-SSB

3	Conclusion

We make the following observations and proposals:

Proposal 1: RAN2 to discuss whether multiple OD-SSBs with different frequencies can be configured for a given SCell.

Proposal 2: When the AO-SSB and OD-SSB have the same frequency, they may share the same servingCellMO, but the OD-SSB SMTC should be linked to the adaptive OD-SSB periodicity.

Proposal 3: The UE applies the OD-SSB specific SMTC when the OD-SSB is activated.

Proposal 4: Introducing a new MAC CE-triggered measurement reporting mechanism is beneficial for OD-SSB

4	Reference

[1] R2-2500245 Discussion on on-demand SSB for Scell Apple