3GPP TSG-RAN WG2 Meeting #130                                 R2-2504543

St.Julians, Malta, May 19th- 23th, 2025

Agenda item:		8.2.3

Source:		Samsung

Title:			Discussion on A-IoT paging

Document for:	Discussion & Decision

- 1 Introduction

In RAN2#129bis, the AIoT paging was extensively discussed with the following agreements:

Agreements

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Agreements on CFRA

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to tak this into account for their design.

In this contribution, we will share our further views on the Paging design.

- 2 Discussion

In AIoT paging message, the resource configuration should be included for Msg1 transmission of devices. Depending on the RA types, the resource configuration may be different.

- CFRA case

In Rel19, the CFRA only supports one device case so that the AIoT paging message should indicate the dedicated resource for Msg1 transmission of one AIoT device. Two options can be considered, as shown in the following figure:

  - Option 1: immediately follow AIoT Paging

In this option, the dedicated resource always immediately follows the AIoT paging message, i.e., once the device receives the AIoT paging message for such device, it can transmit the Msg1 immediately. Thus, the AIoT paging message needn’t contain the number of access occasions. In this sense, the R2D trigger message is not needed since AIoT paging message indicate the starting point of the accessing occasion for Msg1 transmission.

  - Option 2: dedicated access occasion

In this option, the AIoT paging message can indicate the dedicated access occasion, e.g., the number of access occasions before transmitting Msg1 by the device. For example, as shown in Fig.1, AIoT paging message includes number of 6, which indicates that the device 1 should wait for 6 access occasions before transmitting Msg1. This option can provide more flexible resource allocation. Moreover, if CFRA is supported for multiple devices in the future, such option can include multiple access occasions used by different devices.

Fig. 1 Dedicated resource indication for CFRA

In Rel-19, the CFRA only supports one device, which means that Option 1 is enough, and Option 2 can be considered in later release. So, we propose

Proposal 1: RAN2 is kindly asked to agree the Option 1: resource immediately following AIoT paging message, which indicates that AIoT paging message can omit the information on the number of access occasions.

- CBRA

In this case, the AIoT paging message should indicate a resource range so that each device can randomly select one of resource for Msg1 transmission. In RFID design, Q value is provided to the device so that the device can derive the range via 2^Q. Such design can be applied for AIoT, and the length of the Q value can be set a small value, e.g., 4.

Proposal 2: RAN2 is kindly asked to agree that the resource configuration in AIoT paging message for CBRA can include Q value, which is used to calculate the number of access occasions by 2^Q (same as RFID).

Proposal 3: RAN2 is kindly asked to agree 4 bits for Q value.

Besides, to support the FDMA, the FDMA related scheduling information may need to be additionally included in the paging message. If it is included in Paging message, it means that each access occasion has the same FDMA resource allocation. On the other hand, if the reader wants to dynamically change the FDMA resource allocation, it can also indicate the FDMA resource in R2D triggering message. Since FDMA details need RAN1’s further input, whether and how to add in paging message needs further discussion.

Proposal 4: RAN2 is kindly asked to discuss whether and how to include FDMA related resource allocation information in Paging message.

- 3 Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 is kindly asked to agree the Option 1: resource immediately following AIoT paging message, which indicates that AIoT paging message can omit the information on the number of access occasions.

Proposal 2: RAN2 is kindly asked to agree that the resource configuration in AIoT paging message for CBRA can include Q value, which is used to calculate the number of access occasions by 2^Q (same as RFID).

Proposal 3: RAN2 is kindly asked to agree 4 bits for Q value.

Proposal 4: RAN2 is kindly asked to discuss whether and how to include FDMA related resource allocation information in Paging message.