3GPP TSG-RAN WG2 Meeting #130	R2-2504548

St.Julians, Malta, May 19th- 23th, 2025

Agenda item:	7.0.2.19

Source:	Samsung

Title:	Further discussion on the prerequisite of Rel-17/18 SL relay capability

WID/SID:	NR_SL_relay_enh-Core

Document for:	Discussion and Decision

Introduction

This contribution tries to address the following discussion left from the last meeting.

R2-2503082 Summary of offline discussion on the prerequisite of Rel-17/18 SL relay capability Samsung discussion Rel-18 NR_SL_relay_enh-Core Postponed (coordination by interested companies encouraged towards next meeting)

Discussion

- Rel-17 part

Whether the following is agreeable?

  - (Rel17) 31-1 (BC for relay discovery): prerequisite of 31-3 (Basic L2 relay UE)/4(Basic L2 remote UE)/8 (Basic L3 relay UE)/9 (Basic L3 remote UE)

  - (Rel17) 31-4(Basic L2 remote UE): prerequisite of 31-5(HO to idle/inactive relay UE)

During the discussion in last meeting, the above items seem to be OK for majority companies (3 yes vs. 1 not sure). Technically, without relay discovery, the relay UE cannot serve remote UE, and the remote UE cannot connect to the relay UE. Moreover, without basic L2 remote UE operation, the handover to the relay UE cannot perform.

Proposal 1: RAN2 is kindly asked to agree the following:

  - (Rel17) 31-1 (BC for relay discovery): prerequisite of 31-3 (Basic L2 relay UE)/4(Basic L2 remote UE)/8 (Basic L3 relay UE)/9 (Basic L3 remote UE)

  - (Rel17) 31-4(Basic L2 remote UE): prerequisite of 31-5(HO to idle/inactive relay UE)

- Rel-18 part

During the meeting, the followings are agreeable among companies:

  - 31-1/31-4: prerequisite of 51-4/51-5

  - 51-5: prerequisite of 51-8/9/15/16/19

  - 51-9: prerequisite of 51-13/14

  - 51-1: prerequisite of 51-2/3/11/12

  - 51-5(remote UE)/31-3(relay UE): prerequisite of 51-18

So, we can agree the above items.

Proposal 2: RAN2 is kindly asked to agree the following:

  - 31-1/31-4: prerequisite of 51-4/51-5

  - 51-5: prerequisite of 51-8/9/15/16/19

  - 51-9: prerequisite of 51-13/14

  - 51-1: prerequisite of 51-2/3/11/12

  - 51-5(remote UE)/31-3(relay UE): prerequisite of 51-18

Issue 1: Whether 31-5 (i.e., remote UE performs handover to idle/inactive relay UE) is the prerequisite of 51-4 (L2 remote UE path switch)?

Majority companies can accept that 31-5 is not the prerequisite of 51-4. So, we can go this way. However, some clarifications may be needed w.r.t. the capability for the handover to idle/inactive relay UE for Rel-18. According to the comments, two possible understandings:

  - Understanding 1: the support of 51-4 can indicate that the I2I handover to relay UE in any state can be supported.

  - Understanding 2: 31-5 and 51-4 are used jointly to indicate the support of the I2I handover to the relay UE in inactive/idle state.

It seems that Understanding 1 is simpler than Understanding 2 since the clarification is not needed.

Proposal 3: RAN2 is kindly agree that 31-5 is not the prerequisite of 51-4.

Issue 2: Whether 31-6 (i.e., simultaneous transmission/reception of PC5 data (relay discovery) and Uu uplink/downlink) is the prerequisite of 51-4/51-5?

Majority companies think the path switch/multipath operation can be performed even if there is not simultaneous transmission and reception. The reason is that the gNB can configure the remote UE to work over PC5 and Uu separately (e.g., TDM way). Technically, the gNB can have such implementation. In this sense, 31-6 is not a necessary capability for 51-4/51-5.

Proposal 4: RAN2 is kindly asked to agree that 31-6 is not the prerequisite of 51-4.

The following table summarize the discussion in last meeting, and the above proposals.

| Features | Index | Feature group | Pre-requisite (Agreeable vs. FFS) | Proposal |
| --- | --- | --- | --- | --- |
| 31. NR_SL_Relay-Core | 31-1 | Band combination list supporting transmission and reception of relay discovery |  |  |
|  | 31-2 | Band combination list supporting transmission and reception of non-relay discovery |  |  |
|  | 31-3 | Basic NR L2 sidelink relay UE operation | 31-1 (Yes: 3; not sure: 1) | Yes |
|  | 31-4 | Basic NR L2 sidelink remote UE operation | 31-1 (Yes: 3; not sure: 1) | Yes |
|  | 31-5 | Remote UE performs handover to idle/inactive relay UE | 31-4 (Yes: 3; not sure: 1) | Yes |
|  | 31-6 | UE supports simultaneous transmission/reception of PC5 data (Relay discovery) and Uu uplink/downlink respectively |  |  |
|  | 31-7 | UE supports simultaneous transmission/reception of PC5 data (non-Relay discovery) and Uu uplink/downlink respectively |  |  |
|  | 31-8 | Support L3 sidelink relay UE operation | 31-1 (Yes: 3; not sure: 1) | Yes |
|  | 31-9 | Support L3 sidelink remote UE operation | 31-1 (Yes: 3; not sure: 1) | Yes |
| 51. NR_SL_relay_enh-Core | 51-1 | Band combination list supporting transmission and reception of U2U relay discovery |  |  |
|  | 51-2 | Basic NR U2U L2 sidelink relay UE operation | 51-1 | Yes |
|  | 51-3 | Basic NR U2U L2 sidelink remote UE operation | 51-1 | Yes |
|  | 51-4 | NR U2N L2 sidelink remote UE path switch operation | 31-1/31-4 31-5 (Yes: 3, No: 5) 31-6 (Yes: 1, No: 4) | Yes: 31-1/31-4 No: 31-5/31-6 |
|  | 51-5 | NR L2 multipath remote UE operation using PC5 connection | 31-1/31-4 31-6 (Yes: 1, No: 4) | Yes: 31-1/31-4 No: 31-6 |
|  | 51-6 | NR L2 multipath relay UE operation using non-3GPP connection |  |  |
|  | 51-7 | NR L2 multipath remote UE operation using non-3GPP connection |  |  |
|  | 51-8 | Indirect path addition/change to idle or inactive Relay UE | 51-5 | Yes |
|  | 51-9 | PDCP duplication with more than one Uu RLC | 51-5 | Yes |
|  | 51-10 | UE supports simultaneous transmission/reception of PC5 data (U2U relay discovery) and Uu uplink/downlink respectively |  |  |
|  | 51-11 | L3 sidelink U2U relay UE operation | 51-1 | Yes |
|  | 51-12 | L3 sidelink U2U remote UE operation | 51-1 | Yes |
|  | 51-13 | CA-based PDCP duplication over DRB | 51-9 | Yes |
|  | 51-14 | CA-based PDCP duplication over SRB1/2 | 51-9 | Yes |
|  | 51-15 | PDCP duplication over split DRB in L2 multi-path relay | 51-5 | Yes |
|  | 51-16 | PDCP duplication over split SRB1/2 in L2 multi-path relay | 51-5 | Yes |
|  | 51-17 | recovery from direct path RLF via split SRB1 using either PC5 connection or non-3GPP connection |  |  |
|  | 51-18 | L2 PC5-RRC trigger | 51-5(remote UE)/31-3(relay UE) | Yes |
|  | 51-19 | UL transmission via both direct path and indirect path for split DRB | 51-5 | Yes |

- Specification change

The following tables summarize the Rel-17/18 features with/without prerequisite. It can be observed that, most of features define the prerequisite in TR38.822.

|  | With prerequisite | Without prerequisite |
| --- | --- | --- |
| Rel-17 | LTE_NR_DC_enh2 NR_IIOT_URLLC_enh NR_SmallData_INACTIVE NR_NTN_solutions NR_RedCap NR_ENDC_SON_MDT_enh NR_QoE NR_SL_enh NR_ext_to_71GHz NR_UDC TEI17 and Others | NR_MBS LTE_NR_MUSIM NR_IAB_enh NR_SL_Relay NR_UE_pow_sav_enh NR_pos_enh NR_RAN_PRN_enh |
| Rel-18 | NR_pos_enh2 Netw_Energy_NR NR_Mob_enh2 NR_XR_enh NR_NTN_enh NR_UAV NR_IDC_enh NR_MBS_enh NR_QoE_enh NR_MT_SDT NR_redcap_enh NR_ATG NR_DualTxRx_MUSIM TEI18 and Others | NR_netcon_repeater NR_SL_relay_enh NR_SL_enh2 NR_mobile_IAB NR_ENDC_SON_MDT_enh2 |

For some features without prerequisite in TR38.822, the TS38.306 also defines the prerequisite. For example

Rel-17 IAB

TR38.822

TS38.306

Technically, adding prerequisite to TS38.306 would be the correct thing. However, in last meeting, how to make the specification is controversial. So, the following options are list below:

  - Option 1: no change to specification, and capture the agreements in the chairman note.

  - Option 2: make changes to specification (TS38.306) from Rel-17

  - Option 3: make changes to specification (TS38.306) from Rel-18.

It is better to make a choice between option 2 and option 3. However, if we cannot have consensus, option 1 may be the way to go, which means that we will lose all prerequisite for SL relay.

Proposal 5: RAN2 is kindly asked to make a choice between Option 2 and Option 3:

  - Option 2: make changes to specification (TS38.306) from Rel-17

  - Option 3: make changes to specification (TS38.306) from Rel-18.

Conclusion

In this contribution, the prerequisite for SL relay is discussed, and the following proposals are given:

Proposal 1: RAN2 is kindly asked to agree the following:

  - (Rel17) 31-1 (BC for relay discovery): prerequisite of 31-3 (Basic L2 relay UE)/4(Basic L2 remote UE)/8 (Basic L3 relay UE)/9 (Basic L3 remote UE)

  - (Rel17) 31-4(Basic L2 remote UE): prerequisite of 31-5(HO to idle/inactive relay UE)

Proposal 2: RAN2 is kindly asked to agree the following:

  - 31-1/31-4: prerequisite of 51-4/51-5

  - 51-5: prerequisite of 51-8/9/15/16/19

  - 51-9: prerequisite of 51-13/14

  - 51-1: prerequisite of 51-2/3/11/12

  - 51-5(remote UE)/31-3(relay UE): prerequisite of 51-18

Proposal 3: RAN2 is kindly agree that 31-5 is not the prerequisite of 51-4.

Proposal 4: RAN2 is kindly asked to agree that 31-6 is not the prerequisite of 51-4.

Proposal 5: RAN2 is kindly asked to make a choice between Option 2 and Option 3:

  - Option 2: make changes to specification (TS38.306) from Rel-17

  - Option 3: make changes to specification (TS38.306) from Rel-18.