3GPP TSG RAN WG2 #130       	                            R2-2504522

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.2

Source:	KDDI Corporation

Title:	Discussion on measurement report and configuration for UE sided model

SID:	FS_NR_AIML_Mob – Release 19

Document for: 	Discussion and Decision

- Introduction

In RAN2 #129bis, discussion on specification impact for AI/ML for mobility is started, where the discussion focused on LCM[1]. A baseline of the LCM for AI/ML for mobility is that of AI/ML for air interface, and the following discussion and agreement on the functionality is provided in #129bis.

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2. Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM) - Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity - The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. Agreements 1. For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details 2. For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details 3. Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP.

In this contribution, we will mainly discuss specification impact for prediction-based event report and configuration of measurement period.

- Discussion

2.1 Measurement report for prediction result

This section focuses on future state prediction (temporal domain case A) in UE-side model. To facilitate early handover or its preparation by utilizing prediction result, a reporting system for the prediction is necessary. There is a way to utilize current measurement report system by treating the prediction result as actual measurement, therefore the timestamp of the prediction object is calculated by implementation. In our view, such approach will complicate the system, so we think new specification for the report of prediction result (and its configuration) will be straightforward way. We think it is difficult to indicate the timing information of the predicted event in current measurement report format. BM-Case2 of the AI/ML for beam management can be reference.

RAN2 should consider possibility of new measurement report for prediction-based measurement.

RAN2 should consider how to indicate time of the prediction object in a report. BM-Case2 of the AI/ML for beam management can be reference.

Another approach is to define a new measurement event for the predicted event. There is a possibility that defining new event is easier than modifying the report framework.

RAN2 should consider the impact of predicted-based new event for measurement.

In the inter-frequency prediction, our concern is that whether it is possible to configure not to measure the neighbour cell in current mobility spec. When we simply “do not configure measurement object (MeasObjectNR) for the target frequency cell because it will be predicted,” ReportConfig related to that target cell will not be generated. Thus the prediction result cannot be reported.

In inter-frequency prediction, RAN2 should check that whether the current spec can support measurement skip configuration for neighbour cell prediction.

2.2 Configuration of measurement timing and period

For purpose of measurement reduction in time domain (temporal domain case B), a configuration for measurement skip is required. In our understanding, the observation timings are assigned by measurement period configuration of SSB, measurement gap repetition period for MeasurementGap and etc. While those configurations only support simple periodic pattern.

| Figure 1 Example 1 of intra-frequency temporal domain case B (from text proposal of TR 38.744) | Figure 2 Example 2 of intra-frequency temporal domain case B (from text proposal of TR 38.744) |
| --- | --- |

Figure 2 can be realized by simply adjusting the measurement period. We should consider that (1) whether the case of Figure 1 or more complex skipping pattern is necessary or not, and (2) whether it can be realized by current measurement period configuration.

In temporal domain case B, RAN2 should consider the necessity of the complex measurement reduction pattern and its configuration.

From the viewpoint of NW-side, there is possibility of reference signal reduction which may be related to On-demand SSB of NES. It should be discussed in Agenda Item 8.3.3.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1 RAN2 should consider possibility of new measurement report for prediction-based measurement.

Proposal 2 RAN2 should consider how to indicate time of the prediction object in a report. BM-Case2 of the AI/ML for beam management can be reference.

Proposal 3 RAN2 should consider the impact of predicted-based new event for measurement.

Proposal 4 In inter-frequency prediction, RAN2 should check that whether the current spec can support measurement skip configuration for neighbour cell prediction.

Proposal 5 In temporal domain case B, RAN2 should consider the necessity of the complex measurement reduction pattern and its configuration.

- References

- Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China