3GPP TSG-RAN WG2 Meeting #130	R2-2504558

St.Julians, Malta,  May 19th – 23rd , 2025

Source:                Qualcomm Incorporated

Title:	MRO enhancements for mobility

Agenda Item:	8.10.2

Document for:	Discussion and Decision

# Introduction & Background

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## 2.1 LTM

### MHR enhancements for LTM

A UE supporting LTM might undergo both LTM and normal L3 HO scenarios depending on the gNB implementation. A gNB might do a back-to-back L3 HO and LTM due to L3 execution conditions being met in gNB-CU and LTM execution condition met in gNB-DU.

Therefore, it useful for the gNB to know whether the UE’s visited cells were due to LTM or L3 HO. Without such knowledge, the gNB might unnecessarily be trying to optimize ping-pong handovers even though they happened due to different reasons (i.e., L3 HO and LTM). It is beneficial to optimize the MHR to include a HO-type (i.e., LTM or L3 HO) similar to how it has been agreed by RAN2 for RLF Report.

- Enhance MHR to include a “HO-type” (i.e., LTM or L3 HO).

### RACH information in RLF Report

In the previous meetings, some companies proposed to discuss RACH-less LTM to RACH-based LTM “fall back” scenarios. There could be two cases that the RACH-less LTM is not successful:

Case 1: The UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure.

Case 2: The RACH-less LTM is initiated (because of UE having a valid TA either received from LTM CSC or via UE-based measurement) but the RACH-less LTM cell switch execution is not successful e.g., because UE couldn’t determine that the network has successfully received its first UL data. In such a case, T304 would expire.

Upon T304 expiry, UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM recovery after LTM execution failure, then the UE attempts RACH-based LTM recovery once, otherwise re-establishment is performed. Irrespective of whether the LTM recovery was configured or not and if the LTM recovery was successful or not, it is beneficial if the UE indicates to gNB if RACH-less LTM was attempted or not before RACH-based LTM failure so that gNB will know whether TA configuration was not proper or UL transmission was not good.

- UE indicates to gNB in RLF Report if RACH-less LTM was attempted or not before RACH-based LTM failure.

Similarly, in successful case, it is also possible that RACH-less LTM fails and then UE fall backs to RACH-based LTM and succeeds. In this case also, it is beneficial to let gNB know if RACH-less LTM was attempted or not before successful RACH-based LTM recovery.

- UE indicates to gNB in SHR if RACH-less LTM was attempted or not before successful RACH-based LTM recovery.

## 2.2 CHO with candidate SCGs

During running CR discussion, there is email discussion on the following issue.

Editor’s Note: FFS whether it is needed to avoid duplication of information in case of two reports being generated CHO with candidate SCGs, any redundancy (e.g., measurements) are recorded in the reports for PCell (i.e., in SHR, SPR).

It is understand that it should be corner case that both MCG and SCG encounter failure during CHO, then the SHR and SPR are generated and reported as the same time should not be frequent. The signalling overhead could be acceptable. And based on the information in the reports for SHR and SPR, network should be able to associate the two reports.

- There is no need to avoid duplication of information in case of two reports generate CHO with candidate SCGs, i.e.in SHR and SPR.

# Conclusion

- Enhance MHR to include a “HO-type” (i.e., LTM or L3 HO).

- UE indicates to gNB in RLF Report if RACH-less LTM is attempted or not before RACH-based LTM failure.

- UE indicates to gNB in SHR if RACH-less LTM is attempted or not before RACH-based LTM successful RACH-based LTM recovery.

- There is no need to avoid duplication of information in case of two reports generate CHO with candidate SCGs, i.e.in SHR and SPR.

# Reference