3GPP TSG-RAN WG2 Meeting #130		R2-2504568

Malta, May 19 – 23, 2025

Agenda Item:	8.10.2

Source: 	LG Electronics

Title:  	Remaining MRO issues for MOB

Document for:	Discussion

Introduction

In this contribution, we focus on additional remaining issues for LTM MRO and CHO with associated CPAC.

Discussion

Duplicated CHO+CPAC logging in RLF report

At RAN2#129bis, RAN2 identified that there could be scenarios where the UE logs duplicated CHO and associated CPAC information. This issue was left as FFS [1]:

Agreements 8. FFS whether it is needed to avoid duplication of information in case of two reports being generated CHO with candidate SCGs, any redundancy (e.g., measurements) are recorded in the reports for PCell (i.e., in SHR, SPR).

We identify an additional scenario where the UE may log duplicated CHO and associated CPAC information in both the RLF report and SCG failure information.

Specifically, this problem arises when MCG RLF and SCG RLF occur in close succession while the UE is evaluating CHO and CPAC execution conditions. In this case, the same evaluation results may be logged in both the SCG failure information and the RLF report. Since these logs are reported at different times, the network may interpret them as two separate evaluation samples. This misinterpretation can lead to incorrect mobility optimizations, where the network treats the same evaluation outcome as distinct events. As a result, this could lead to improper CHO+CPAC execution condition tuning for similar UEs.

We present an example failure scenario like below:

- SCG RLF occurs → UE logs CHO and CPAC evaluation results in SCG failure information

- UE sends the information to the source MN

- Source MN receives this as the first log sample

- Later, MCG RLF occurs → UE logs the same CHO and CPAC evaluation results in the RLF report

- After RRC re-establishment, a new PCell (possibly under a new MN) receives the RLF report and forwards it to the source MN

- The source MN receives this data again, treating it as a second log sample. Due to the time gap between the two log deliveries, the network may not recognize them as duplicates, leading to erroneous parameter updates.

To avoid this duplication issue, we propose the following two options:

Option 1: Log Deduplication Check before logging

The UE checks whether the same CHO and CPAC evaluation results have already been reported.

If so, the UE excludes them from the RLF report if they were previously logged in the SCG failure information.

Option 2: One-time Logging Behavior

The UE discards CHO and CPAC evaluation results immediately after they are logged in either the RLF report or SCG failure information.

This ensures that once logged, the results are not reused in any subsequent report.

Based on the above, we propose to discuss to avoid duplication of information in case of two reports being generated CHO with candidate SCGs for RLF-report and SCG failure information.

Proposal 1: RAN2 discuss to avoid duplication of CHO and CPAC evaluation results when two separate reports (RLF report and SCG failure information) are generated based on the the following options:

- Option 1: The UE checks whether the CHO and CPAC evaluation results have already been reported, and excludes them if so.

- Option 2: The UE removes CHO and CPAC evaluation results after logging them once in any report.

LTM Access type logging in RLF report

At the RAN2#128, RAN2 made the following agreement [2]:

Agreements - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues. - Reuse the existing ra-InformationCommon for the RA-based LTM failure. - If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

During the last meeting, there was a discussion about the highlighted part, and we still believe that there are issues regarding this part, so we want to revisit the discussion. We still believe that in some cases, based on implicit access information, the network will not be able to accurately determine which access type the UE used for the entire LTM cell switch attempt.

A typical example is when the UE attempts a RACH-less LTM cell switch based on a dynamic grant (i.e., DG-based RACH-less LTM). In this case, the UE receives a RACH-less LTM CSC (cell switch command) from the source cell and monitors the PDCCH in the target cell for the UL grant. However, for certain reasons from the network side, the target cell may command the UE to use RACH for LTM. Then, if RLF is detected in this case, the UE may include both RACH-less and RACH information as the access information. Without explicit access type information, the network cannot determine which access type the UE attempted until RLF unless the network receives this information explicitly.

Scenario Example:

- The UE receives an LTM cell switch command to perform DG-based RACH-less LTM from the source cell. The UE starts T304.

- The UE applies the RRC Reconfiguration related to LTM.

- The UE performs PDCCH monitoring to obtain the first UL grant for sending the RRC Reconfiguration complete to the target cell (since it is DG-based).

- During PDCCH monitoring on the target cell, the target cell commands the UE to perform RACH.

- The UE attempts to send the RRC Reconfiguration complete message based on RACH information in the target cell.

- The UE repeatedly fails RACH and declares HOF due to T304 expiry.

- The UE performs RLF-logging due to HOF, logging RACH information even though it was initially commanded to perform RACH-less LTM.

By above reason, we think the implicit access type information will leave a fundamental lack of trust in the access type information reported by the UE seems not be enough. Thus, we think that the UE should log explicit access type information to provide clear information preventing any potential confusion from the network side.

Proposal 2: In the RLF report for MCG LTM, RAN2 should revisit and decide whether the UE should explicitly log the access type.

In the above example of DG-based RACH-less LTM, we confirmed that the UE may face a situation where RACH-less LTM CSC is commanded but RACH-based LTM should be performed. We think RAN2 need to consider more about this case where the UE actually tried different access type from the access type commanded by the network in LTM CSC.

In the above example, it is unclear whether the UE should log the RACH actually performed as the access type instead of the RACH-less commanded by the network. If the UE logs the access type as RACH, the network may not know that the LTM CSC has been indicated to RACH-less, the network may misunderstand that the cause of HOF/RLF is due to improper early synchronization before LTM. Otherwise, if the UE logs the access type as RACH-less, the network doesn’t know that the UE had actually tried RACH based LTM until HOF occurs and the network may misunderstand that the UE failed RACH-less procedure during LTM.

Since the above inaccurate logging information cannot help the network’s MRO, we think, for this case, the UE should be able to report both the access type actually performed by the UE and the access type commanded by the network. The access type commanded by the network can only need to be optionally logged if the information is different from the access type actually performed by the UE.

Proposal 3: In the RLF report for MCG LTM, the UE logs not only the access type but also the commanded access type which was indicated LTM CSC if the access type is different from the commanded access type.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1: RAN2 discuss to avoid duplication of CHO and CPAC evaluation results when two separate reports (RLF report and SCG failure information) are generated based on the the following options:

- Option 1: The UE checks whether the CHO and CPAC evaluation results have already been reported, and excludes them if so.

- Option 2: The UE removes CHO and CPAC evaluation results after logging them once in any report.

Proposal 2:	In the RLF report for MCG LTM, RAN2 should revisit and decide whether the UE should explicitly log the access type.

Proposal 3:	In the RLF report for MCG LTM, the UE logs not only the access type but also the commanded access type which was indicated LTM CSC if the access type is different from the commanded access type.

References

- R2-2502986 Report from session on maintenance and SON/MDT Session chair (Ericsson)

- R2-2410916 Report from session on maintenance, CovEnh, SON/MDT and eRedCap Session chair (Ericsson)